3GPP TSG-RAN WG3 #114bis-e	R3-221146

17th - 26th January 2022

E-Meeting

Agenda Item:	30.2

Source:	Ericsson - Moderator

Title:	Summary of Offline Discussion – MultiSIM

Document for:	Approval

# Introduction

This is the SOD document for the following come back:

CB: # 17_MultiSIM

- Check the LS from other groups and focus on WID scope

- Identify RAN3 related issues and discuss the corresponding solutions

- Capture agreements and open issues

- Provide stage2/3 BL CRs if agreeable, split work

(E/// - moderator)

Summary of offline disc R3-221009

Deadline by 19th, Wednesday 11h UTC ( before the online session)

# For the Chairman’s Notes

Agreements (from Second round):

- It is agreed that the procedural text state that the “Paging Cause support indication “IE indicates “UE support”. No change need in the BL CR.

- It is agreed that the “Reject Paging” cause value in S1AP is not needed. BL CR should remove it.

- It is agreed that the over E1AP is not introduced. It can be further discussed based on contribution. BL CR should remove it.

- It is agreed that the “MUSIM Gap” over F1AP is not introduced. It can be further discussed based on RAN2 progress. BL CR should remove it.

- The topics in the “Remaining issues to be discussed at RAN3#115-e” are marked as FFS in the BL CRs.

- It is agreed that the BL draft CR on 38.300 is needed (on Radisys).

The content is to be further discussed at the next meeting. Radisys to submit an input paper to RAN3#115.

Proposed to be discussed online.

If no agreement, the topics should be moved to “Remaining issues to b discussed at RAN3#115-e”.

- “Paging Cause support indication” IE: naming:

  - Using “Paging Cause Indication for Voice Service Supported”, which is the SA2 term;

- and Coding: One code point with extension mark is agreeable. Further:

  - Using “supported, …” which is align with SA2.

Remaining issues to be discussed at RAN3#115-e:

- Paging Cause IE Coding.  Choose from the below two Options:

  - Option 1: “voice, …”

  - Option 2: A new IE structure as in below: This IE indicates the paging cause for paging a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | O |  | ENUMERATED (voice, …) | Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If this field is not present but PagingRecord is present, it implies that the corresponding paging entry is for a service other than IMS voice. |

- New Cause value over NGAP “Release due to MUSIM”, discuss if needed / what is the usage.

# First round agreements:

Agreements:

- It is agreed to name the Baseline CRs as “Introduction of MultiSIM support over NG or Xn or etc.”.

- Align the IE names in all the specification.

- Paging Cause is included in NGAP/XnAP/F1AP/E1AP/S1AP. The Coding is to be discussed further.

- Paging Cause support indication is included in NGAP. Both the name and the coding are to be discussed further.

- Paging Cause support indication is not needed in XnAP.

- The below Baseline CR should capture the agreement at this meeting:

R3-220219 is revised in R3-221127 (CR rev. no. 1)

R3-220364 is revised in R3-221128 (CR rev. no. 1)

R3-220618 is revised in R3-221129 (CR rev. no. 1)

R3-220677 is revised in R3-221130 (CR rev. no. 1)

R3-221132  (CR number: 0677)

# Second round discussions

## “Paging Cause” IE Coding:

- “Paging Cause IE” coding:

  - one code point “voice” or

  - extension mark “voice, …”

  - two code points “voice, non-voice”

  - With extension mark “voice, non-voice, …”

  - “Paging Record” proposal as in R3-220220:

#### 9.3.3.xx Paging Record This IE indicates the paging cause for paging a UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Paging Cause; O; ENUMERATED (voice, …); Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If this field is not present but PagingRecord is present, it implies that the corresponding paging entry is for a service other than IMS voice..

- A1: Please indicate your choice and reasoning.

| - Company | - Choice | - Comments |
| --- | --- | --- |
| - Ericsson | - b | - We only need one value. But to make it extendable for the future use. - The RAN2 coding does not need any input from AMF. |
| - Radisys | - b | - Needs to be extendible. - The term Paging Record used for Paging cause is confusing with the Paging Records in RRC Paging message. Moreover when one IE can provide the cause, we do not see a need for multilevel IE. |
| - Huawei | - e | Option e best matches RAN2’s agreement. If we have a look at how RAN2 encode this, we can find we need to make sure our coding satisfies two requirements: The first is, we can represent whether Paging cause is supported. And the second is, when the paging cause is supported, whether the paging is originated from voice or not. So only option c and e satisfies these two requirements. Moreover, if we look at RAN2’s running CR, it does not explicitly shows whether the paging comes from a non-voice, but only implicitly shows this which follows the same logic as option e. |
| - Samsung | - b | - We have the similar view as Ericsson. |
| - ZTE | - e | - The detail reason has been provided previously (see “Moderator’s summary”). - We can keep it FFS and discuss it in the next meeting. |
| - CATT | - b | - Share with E/// |

- Moderator Summary 1:

- Among the 6 companies answered, 4 choose b, 2 choose e. RAN3 works on consensus, thus this topic is to be further discussed at the next RAN3 meeting.

## “Paging Cause support indication” IE: naming and Coding:

Naming aspect:

It is commented that we align with the SA2 naming, which is “Paging Cause Indication for Voice Service Supported”.

It is also commented to use a general name for future extension.

- Paging Cause Indication for Voice Service Supported, this is the SA2 term;

- Paging Cause Indication Supported; generalized SA2 term

- Paging Cause support indication

- Paging Cause Indication Support

- A2: Please indicate your preferred choice and also indicate if you are fine with other choices

| - Company | - Choice | - Comments |
| --- | --- | --- |
| - Ericsson | - a) - Fine with other choices, | - It is perhaps easy to use SA2 term for people to search specifications. |
| - Radisys | - D or C | - We prefer option d or C, as indicated in our paper. It is generic and can be extendible. Later we may have options like IMS voice, or CSFB voice or data or anything. Paging Cause Indication Support; ENUMERATED (voice, … ). |
| - Huawei | - A | - Slight prefer A, no strong view |
| - Samsung | - Slightly a.) | - It’s aligned with SA2. Other one is also ok. |
| - ZTE | - A | - Aligned with SA2’s CR. |
| - CATT | - a) | - We may use a) now. no strong view |

- Moderator Summary 2:

- Propose to have an online discussion to see if we can reach agreement on option a).

- Coding aspect:

- one code point “supported”

- one code point with extension “supported, …”

- one code point “voice, …”

- two code points “supported, not-supported”

- two code points “supported, not-supported, …”

- A3: Please indicate your preferred choice and also indicate if you are fine with other choices

| - Company | - Choice | - Comments |
| --- | --- | --- |
| - Ericsson | - b) - Fine with others | - This should be very static, one code point is enough. |
| - Radisys | - c | - One code point with extension is enough |
| - Huawei | - b | - Same view as Ericsson |
| - Samsung | - b | - It’s aligned with description in SA2 CR. And we think single code point with extension is enough. |
| - ZTE | - b |  |
| - CATT | - b |  |

- Moderator Summary 3:

- Propose to have an online discussion to see if we can reach agreement on option b).

- Others: Should in the procedural text state that this IE indicates

- “UE support” (i.e. current CR wording)

- “AMF and UE support”

- A4: Please indicate your preferred choice and also indicate if you are fine with other choices

| - Company | - Choice | - Comments |
| --- | --- | --- |
| - Ericsson | - b) - Fine with others | - Such IE is usually stated indicating “AMF and UE support”… |
| - Radisys | - a - Fine with others | - We need to think of the Downlink path only. Hence UE support is sufficient |
| - Huawei | - a | - By checking the BL CR of SA2, we slightly prefer not add AMF. Because the motivation of adding this IE is to indicate that UE supports the Paging cause indication for voice service feature (of course AMF should also support the feature, this is by default) |
| - Samsung | - Either is fine | - No strong view. |
| - ZTE | - a | - No strong view. |
| - CATT | - a | - a) is ok. It is in UE context. |

- Moderator Summary 4:

- It is agreed that the procedural text state that the “Paging Cause support indication “IE indicates “UE support” (i.e. current CR wording),

## “Paging Cause support indication” over E1AP needed?

- A5: Please indicate your view and reasoning.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - No | - See no strong need. It complicates gNB-CU-UP. |
| - Radisys | - Yes | - We think this is needed for Inactive paging. If UE does not support Paging Cause, then CU-UP does not have analyse the DL packet to provide the Paging cause in DL data Notification. - But we suggest Paging cause indication support is sent to Cu-UP only when UE is sent to RRC_inactive. |
| - Huawei | - No | - Seems not needed, we have PPI there. |
| - Samsung |  | - We proposed and supported it in the first round. But after reviewing SA2 spec further, it might not be required. So if needed, it could be discussed further based on the contribution in the next RAN3 meeting. |
| - ZTE | - Maybe Yes | - We are fine to discuss it next meeting. |
| - CATT | - No | - Similar situation as SS, after check the spec 36.423, the PPI always is sent from UP to CP. |

- Moderator Summary 5:

- It can be discussed at the next meeting based on contribution.

  - New Cause value over S1AP “Reject paging” needed?

- A6: Please indicate your view and reasoning.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - No | - This new cause value would not add useful information. |
| - Radisys | - No | - Agree with E/// |
| - Huawei | - No strong view |  |
| - Samsung | - No | - We don’t see the value. |
| - ZTE | - No |  |
| - CATT | - No |  |

- Moderator Summary 6:

- It is agreed that the “reject paging” cause value in S1AP is not needed.

  - New Cause value over NGAP “Release due to MUSIM” needed?

- A7: Please indicate your view and reasoning.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - No | - This new cause value would not add useful information. |
| - Radisys | - No | - Agree with E/// |
| - Huawei | - No strong view |  |
| - Samsung | - Yes | - We think it’s useful that RAN node knows the reason of the release. |
| - ZTE | - Yes | - Same view as SS. |
| - CATT | - No |  |

- Moderator Summary 7:

- It is proposed to discuss this at the next meeting.

  - MUSIM Gap over F1AP needed?

- A8: Please indicate your view and reasoning.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - No | - The MUSIM Gap is decided by gNB-CU. |
| - Huawei | - Yes | - We think it is more approximate to locate the gap configuration at the DU, since this is related to the UE dynamic scheduling. But anyway we are fine to wait for RAN2 further progress. |
| - Samsung |  | - The MUSIM Gap could be decided by the gNB-CU. However, RAN2 is still discussing it, so RAN3 should wait for RAN2’ progress. |
| - ZTE | - Maybe No | - Can discuss it next meeting. |

- Moderator Summary 7:

- Further discussion based on contribution and RAN2 progress.

  - TP proposed for TS 38.300 needed? (new addition in the Annex)

There is no input document on this topic. The TP is attached in the Annex.

The companies are welcome to comment. Alternative is that the interested company to bring the contribution at the next RAN3 meeting.

- A9: Please indicate your view and reasoning.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Radisys | - Yes | - We think this TP is needed. If companies agree we can submit this TP as a outcome of this discussion. |
| - Huawei | - Yes | - This looks reasonable, let’s see if some company volunteer to take this lead. |
| - Samsung | - Yes | - We also think it’s reasonable. |
| - ZTE | - Yes |  |
| - CATT | - Yes | - Need check whether RAN2 already has the CR/TP |

- Moderator Summary 8:

It is proposed to agree that the BL draft CR on 38.300 is needed (on Radisys). The content to be further discussed at the next meeting (Radisys to submit an input paper to RAN3#115.

# First round Discussions

## General

The WID Rapporteur had early discussed the Baseline CRs with the companies. The below CRs will be revised, if needed, to capture the agreement and serve as the Baseline CRs.

- Revise R3-220219	TS 36.413 ->ZTE

- Revise R3-220364	TS 38.413 -> Nokia

- Revise R3-220618	TS 38.423 -> Ericsson

- Revise R3-220677	TS 38.473) -> Huawei

For the Baseline CRs, moderator propose to apply the same as used in NR-IIoT (AI 21.1) that we use the same title, for example:

Introduction of MultiSIM support over NG /Xn/F1/S1

Proposal 1: for the BL CRs, we use “Introduction of MultiSIM support over NG  or Xn or F1 or S1”.

Any view?

| Company | Comment |
| --- | --- |
| Nokia | OK for us. |
| ZTE | Agree |
| vivo | Fine |
| Huawei | OK |
| Qualcomm | OK |
| CATT | OK |

## Impacts on S1AP

- R3-220219 presents the changes on S1AP. Any comments on this CR?

| Company | Comment |
| --- | --- |
| Nokia | The use of of an ID-Paging Record in the Paging message which is an asn1 sequence containing a Paging Cause is unusual in our protocol but looks like RAN2 encoding.. why is this needed? Should we consider that MME doesn’t support the feature if the eNB does? |
| ZTE | The IE Paging Cause/Paging Record is optional. If this IE is present, it means that MME supports Multi-USIM feature. The sub-IE Paging Cause included in this IE (refer to R3-220219) as optional IE indicates whether the Paging message is originated due to IMS voice. When this IE is present but the sub-IE is absent, it implies that the corresponding paging entry is for a service other than IMS voice. When this IE is present and the sub-IE included in this IE (refer to R3-220219) is present, it implies that the corresponding paging entry is for IMS voice service. It is based on RAN2 progress. |
| vivo | Agree with CR intention |
| Ericsson | Propose to use the Paging Cause that we will define for all the specification. Do not see the need for the new cause value “reject paging “. |
| Huawei | With regard to ZTE’s method which contains one optional paging cause IE within another optional Paging record IE, first I think it is quite an unusual way. Usually, when we only have one IE within some IE, the IE is mandatory. However, as ZTE explains, I do understand the motivation of the method and I think this way best matches the RAN2’s agreement among other contributions. |
| Radisys | We think just Paging cause IE is sufficient. Presence of this IE indicates IMS voice is supported, and Absence of the IE indicates, other services are supported. MME support for MUSIM need not be indicated in every Paging message. |
| Qualcomm | The “Paging Record” IE should be “Paging Cause” IE. “Paging Rejection” is not needed. |
| CATT | Paging cause IE can be used |

Proposal 2: The paging cause should be aligned for all the specifications.

Proposal 3: The baseline S1AP CR includes the “Paging Cause” in Paging message.

## Impacts on NGAP

- R3-220364 presents the changes on NGAP. Any comments on this CR?

| Company | Comment |
| --- | --- |
| Nokia | Peharps an editor’s note could be added whether we need a second codepoint “non-voice” to differentiate an AMF not using the feature from an AMF using the feature and indicating a non-voice service? |
| ZTE | Partial agree, including two comments as below. 1) In the our paper [R3-220220], we suggest the sub-IE “Paging Cause” is optional in the IE “Paging Cause”, detail reason can be seen in section 3.6. 2) In the IE “9.3.1.xy Support of Paging Cause”, we suggest to simply to “ENUMERATED (supported …)”. |
| vivo | Agree with CR intention |
| Ericsson | - Fine with “Paging Cause”. Think there is no need to have “non-voice” as the paging Cause goes directly to RRC. - Fine with “Support of Paging cause”, two comments, one if we need another code point “not support”, but since this is static, also fine with one code point; two: propose to add “This IE is used to notify whether the AMF and UE supports…” in 9.3.1.xy - Do not think that we need a new “Release due to MUSIM” |
| Huawei | Similar to comments in 3.2, if it is allowed to have one optional IE within another optional IE, we agree with ZTE. Other parts looks good. |
| Radisys | Partial agree In our paper R3-220855, we have used an IE Paging Cause Indication Support with enumerated value as (voice, …) which can be extended in future to support more options lie non-voice, etc. Hence we suggest to use this option. |
| Qualcomm | Looks good |
| CATT | Support this CR |

Proposal 4: The baseline NGAP CR includes the “Paging Cause” in Paging message.

Proposal 5: The baseline NGAP CR includes the “Paging Cause indication for voice services”

## Impacts on XnAP

- R3-220618 presents the changes on XnAP. Any comments on this CR?

| Company | Comment |
| --- | --- |
| Nokia | Why doesn’t the CR include a Paging Cause in the RAN Paging message? |
| ZTE | Same view with Nokia. We suggest to add Paging Cause to align with NGAP/S1AP/F1AP. |
| Samsung | The paging cause should be added in the RAN PAGING message. In our understanding, a paging cause for the RAN Paging is agreed in RAN2 and SA2. |
| Agree with Nokia and ZTE. |  |
| Ericsson | Paging Cause should be added. |
| Huawei | Same view as Nokia and ZTE. Paging Cause is clearly needed. |
| Radisys | In our paper R3-220855, we have included Paging Cause in the RAN Paging. Hence suggest adding Paging Cause to RAN Paging. We think Paging cause support indication in Paging Assistance data is not needed, as it will not be used in NG-RAN Node 2. If Paging cause is provided in RAN Paging it will be passed to UE. |
| Qualcomm | We should add “Paging Cause” instead of “Paging Cause Indication for Voice Service Supported” ins the Xn Paging message. |
| CATT | Paging cause should be added. But the Paging Cause Indication for Voice Service Supported in Assistance Data for RAN Paging is not needed as Radisys said. The paging node carry the paging cause implicit the UE support it |

Proposal 6: The baseline XnAP CR includes the “Paging Cause” in the RAN Paging message.

## Impacts on F1AP

- R3-220677 presents the changes on F1AP. Any comments on this CR?

| Company | Comment |
| --- | --- |
| Nokia | Paging Cuse is OK but the MUSIM Gap Config in the DU to CU RRC Information should be removed and first discuss which node decides the Gap config. In our view, we think it may be instead the role of CU to decide in preferred pattern depending on other types of gaps already configured. |
| ZTE | Agree with Nokia. We shall further think MUSIM Gaps |
| Samsung | The MUSIM Gap Config needs to be discussed after RAN2 progress. |
| vivo | Agree with the CR intention |
| Ericsson | Paging Cause is fine. MUSIM Gap should be removed. |
| Huawei | 1. The MUSIM Gap Config is just for capturing RAN2 agreements, but we are OK to remove it at this stage. 2. If it is allowed to have one optional Paging Cause IE within another optional Paging Cause IE, we are OK to change this part as well. |
| Radisys | Paging cause is ok. MUSIM gap needs to be discussed |
| Qualcomm | Gap should be decided by CU. |
| CATT | Agree with Nokia. |

Proposal 7: The Baseline F1AP CR includes the “Paging Cause”.

## Others

### Set 1

In R3-220220, Paging Recorder is proposed to the above specifications. What is your view on this proposal?

| Company | Comment |
| --- | --- |
| Nokia | Need clarification? |
| ZTE | I suggest that we shall discuss and then decide it. The difference between my paper and other papers is the sub-IE Paging Cause is mandatory or Optional. In the RAN2 in LS as above, RAN2 agreed to provide the paging cause value “voice” to UEs by extending the Paging message. The presence of the extension, which can be empty list if none of the UEs have paging cause “voice”, indicates that network supports transmission of paging cause for service indication. The IE Paging Cause/Paging Record is optional. If this IE is present, it means that MME supports Multi-USIM feature. The sub-IE Paging Cause included in this IE (refer to R3-220219) as optional IE indicates whether the Paging message is originated due to IMS voice. When this IE is present but the sub-IE is absent, it implies that the corresponding paging entry is for a service other than IMS voice. When this IE is present and the sub-IE included in this IE (refer to R3-220219) is present, it implies that the corresponding paging entry is for IMS voice service. It is based on RAN2 progress. |
| Samsung | The usage and the benefit are still unclear. We don’t understand the dependency on RAN2 for coding method in RAN3 |
| Ericsson | The proposed coding is to achieve that sometimes an empty structure is used? |
| Huawei | See our comments to 3.2 |
| Radisys | For the paging cause sub IE see our response in 3.2 Addition of paging cause to PDU and UE release and modify, needs to be discussed further. |
| Qualcomm | See answer to 3.2 |
| CATT | Need more discussion |

### Set 2

In R3-220649/50, there are two proposals:

Introduce the Paging Cause IE in the DL Data Notification message in E1AP. The Paging Cause IE is coded with one value, e.g. ‘Voice’. What is your view?

| Company | Comment |
| --- | --- |
| Nokia | Seems needed. |
| ZTE | Needed, so that E1AP BLCR is also needed. |
| Samsung | To support a paging cause for the RAN Paging, the gNB-CU-UP needs to provide a service indication to the gNB-CU-CP. |
| Vivo | E1AP BLCR is also needed |
| Ericsson | E1AP is impacted. |
| Huawei | Seems needed |
| Radisys | needed |
| Qualcomm | This is needed. CU-UP sets this value based PPI in GTP-U header. |
| CATT | It is needed. But we should assure the UP can obtain whether the data type is voice. |

Proposal 8: The Baseline E1AP CR includes the “Paging Cause” in DL Data Notification

Introduce the Paging Cause Indication IE in the Core Network Assistance Information for RRC INACTIVE IE in NGAP and in the Bearer Context Setup Request and the Bearer Context Modification Request messages in E1AP. The Paging Cause Indication IE is coded with one value, e.g. ‘supported’.

What is your view?

| Company | Comment |
| --- | --- |
| Nokia | Needs further discussion in our view. |
| ZTE | Need further discussion |
| Samsung | Only if the paging cause for voice service is supported for RAN paging, the gNB-CU-UP needs to provide a service indication to the gNB-CU-CP when a DL data for voice service arrives. It’s used for avoiding unnecessary indication from the gNB-CU-UP to the gNB-CU-CP. |
| Radisys | Agree with Samsung’s reasoning. But we think it is needed only when the UE is sent to inactive instead of sending the IE for every bearer setup and modify. Also we prefer to use the code point in our paper R3-220855 for future extension instead of “supported” |
| Qualcomm | This is needed |
| CATT | It is needed |

### Other

| Company | Comment |
| --- | --- |
| Radisys | We also think that update for Paging cause is needed in 38.300. TP for the same is provided in the Annexure. |

Moderator’s summary:

- After the first round, it is agreed that:

- It is agreed to name the Baseline CRs as “Introduction of MultiSIM support over NG or Xn or etc.”. Align the IE names in all the specification.

- Paging Cause is included in NGAP/XnAP/F1AP/E1AP/S1AP. The Coding is to be discussed.

- Paging Cause indication for voice services is included in NGAP. The  Coding is to be discussed.

- After the first round, the below topics need to be further discussed:

- “Paging Cause IE” coding:

  - one code point “voice” or

  - two code points “voice, non-voice”

  - “Paging Record” proposal in R3-220220

  - With extension mark “ …”

- “Paging Cause indication for voice services” coding:

  - one code point “supported” or “voice”

  - two code points “supported, not-supported”

  - With extension mark “ …”

- “Paging Cause indication for voice services” indicates UE support or “AMF and UE support”?

- Paging Cause indication for voice services over XnAP needed?

- Paging Cause indication for voice services over E1AP needed?

- New Cause value over S1AP Reject paging needed?

- New Cause value over NGAP Release due to MUSIM needed?

- MUSIM Gap over F1AP needed?

- TP proposed for TS 38.300 needed?

ZTE’s view:

- “Paging Cause”, one code point “voice” or two code points “voice, non-voice” vs “Paging Record” proposal in R3-220220

If IE Paging Cause/Paging Record is absent in the Paging/RAN Paging message, the RAN knows it is legacy Paging (i.e., not triggered due to Multi-SIM feature)

If IE Paging Cause/Paging Record is present in the Paging/RAN Paging message, the RAN knows it is Multi-SIM related Paging (i.e., triggered due to Multi-SIM feature)

If sub-IE Paging Cause is absent in the IE Paging Cause/Paging Record, the RAN knows it is for a service other than IMS voice.

If sub-IE Paging Cause is present in the IE Paging Cause/Paging Record, the RAN knows it is for IMS voice.

#### 9.3.3.xx	Paging Record

This IE indicates the paging cause for paging a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | O |  | ENUMERATED (voice, …) | Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If this field is not present but PagingRecord is present, it implies that the corresponding paging entry is for a service other than IMS voice. |

The following IE is existing in the current TS38.413.

#### 9.3.1.119      CN Assisted RAN Parameters Tuning

This IE provides information for assisting in parameters tuning of the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Expected UE Behaviour | O |  | 9.3.1.93 | This IE may be present in case the Core Network Assistance Information for RRC INACTIVE IE is not included and is ignored otherwise. |

9.3.1.140      Enhanced Coverage Restriction

This IE provides information on the restriction information of using Coverage Enhancement.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Enhanced Coverage Restriction | O |  | ENUMERATED (restricted, ...) | Indicates whether the UE is restricted to use coverage enhancement. Value “restricted” indicates that the UE is not allowed to use coverage enhancement. |

9.3.2.9          TNL Association Usage

This IE indicates the usage of the TNL association.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, …) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. |

# References

- R3-220099	Reply LS on RAN2 agreements for MUSIM (RAN2)

- R3-220100	LS on RAN2 agreements for paging with service indication (RAN2)

- R3-220120	LS on Paging Cause Indication for Voice Service Supported in RRC Inactive assistance information (SA2)

- R3-220219	Multi-USIM BLCR to TS 36.413 (ZTE, vivo)

- R3-220220	(TP for BLCR 38.413/38.423/38.473) Support of Multi-SIM (ZTE)

- R3-220363	Support for Multi-USIM devices (Nokia, Nokia Shanghai Bell)

- R3-220364	Support for Multi-USIM devices (Nokia, Nokia Shanghai Bell)

- R3-220467	Discussion of paging cause for MUSIM (Huawei)

- R3-220617	Paging Cause Indication for Voice Service Supported” in RRC Inactive (Ericsson)

- R3-220618	Support of MUSIM in XnAP (Ericsson)

- R3-220649	(TP for LTE_NR_MUSIM CR for TS 38.413) Discussion on supporting the paging cause for MUSIM UE (Samsung)

- R3-220650	(TP for LTE_NR_MUSIM BL CR for TS 38.463) Supporting the paging cause for MUSIM UE (Samsung)

- R3-220677	(CR to 38.473) Support for Multi-SIM devices (Huawei, vivo, CMCC)

- R3-220678	(CR to 38.413) Support for Multi-SIM devices (Huawei)

- R3-220679	(CR to 36.413) Support for Multi-SIM devices (Huawei,vivo,CMCC)

- R3-220680	(CR to 38.423) Support for Multi-SIM devices (Huawei,vivo,CMCC)

- R3-220855	(TP for NGAP, F1AP and XnAP on MUSIM) MUSIM Paging Service Indication Support (RadiSys, Reliance JIO)

# Annexure

TP for TS 38.300

### 9.2.2	Mobility in RRC_INACTIVE

#### 9.2.2.1	Overview

RRC_INACTIVE is a state where a UE remains in CM-CONNECTED and can move within an area configured by NG-RAN (the RNA) without notifying NG-RAN. In RRC_INACTIVE, the last serving gNB node keeps the UE context and the UE-associated NG connection with the serving AMF and UPF.

If the last serving gNB receives DL data from the UPF or DL UE-associated signalling from the AMF (except the UE Context Release Command message) while the UE is in RRC_INACTIVE, it pages in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s).

Upon receiving the UE Context Release Command message while the UE is in RRC_INACTIVE, the last serving gNB may page in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s), in order to release UE explicitly.

Upon receiving the NG RESET message while the UE is in RRC_INACTIVE, the last serving gNB may page involved UEs in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s) in order to explicitly release involved UEs.

Upon RAN paging failure, the gNB behaves according to TS 23.501 [3].

The AMF provides to the NG-RAN node the Core Network Assistance Information to assist the NG-RAN node's decision whether the UE can be sent to RRC_INACTIVE, and to assist UE configuration and paging in RRC_INACTIVE. The Core Network Assistance Information includes the registration area configured for the UE, the Periodic Registration Update timer, and the UE Identity Index value, and may include the UE specific DRX, an indication if the UE is configured with Mobile Initiated Connection Only (MICO) mode by the AMF, the Expected UE Behaviour, the UE Radio Capability for Paging and Paging Cause Indication Support. The UE registration area is taken into account by the NG-RAN node when configuring the RNA. The UE specific DRX and UE Identity Index value are used by the NG-RAN node for RAN paging. The Periodic Registration Update timer is taken into account by the NG-RAN node to configure Periodic RNA Update timer. The NG-RAN node takes into account the Expected UE Behaviour to assist the UE RRC state transition decision. The NG-RAN node may use the UE Radio Capability for Paging during RAN Paging. The NG-RAN node takes into consideration the Paging Cause Indication Support information to configure the Paging Cause in RAN Paging for an UE in RRC_INACTIVE state. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the Paging Cause may be included.

Skip unchanged text

### 9.2.5	Paging

Paging allows the network to reach UEs in RRC_IDLE and in RRC_INACTIVE state through Paging messages, and to notify UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state of system information change (see clause 7.3.3) and ETWS/CMAS indications (see clause 16.4) through Short Messages. Both Paging messages and Short Messages are addressed with P-RNTI on PDCCH, but while the former is sent on PCCH, the latter is sent over PDCCH directly (see clause 6.5 of TS 38.331 [12]).

While in RRC_IDLE the UE monitors the paging channels for CN-initiated paging; in RRC_INACTIVE the UE also monitors paging channels for RAN-initiated paging. A UE need not monitor paging channels continuously though; Paging DRX is defined where the UE in RRC_IDLE or RRC_INACTIVE is only required to monitor paging channels during one Paging Occasion (PO) per DRX cycle (see TS 38.304 [10]). The Paging DRX cycles are configured by the network:

1)	For CN-initiated paging, a default cycle is broadcast in system information;

2)	For CN-initiated paging, a UE specific cycle can be configured via NAS signalling;

3)	For RAN-initiated paging, a UE-specific cycle is configured via RRC signalling;

-	The UE uses the shortest of the DRX cycles applicable i.e. a UE in RRC_IDLE uses the shortest of the first two cycles above, while a UE in RRC_INACTIVE uses the shortest of the three.

The POs of a UE for CN-initiated and RAN-initiated paging are based on the same UE ID, resulting in overlapping POs for both. The number of different POs in a DRX cycle is configurable via system information and a network may distribute UEs to those POs based on their IDs.

When in RRC_CONNECTED, the UE monitors the paging channels in any PO signalled in system information for SI change indication and PWS notification. In case of BA, a UE in RRC_CONNECTED only monitors paging channels on the active BWP with common search space configured.

For operation with shared spectrum channel access, a UE can be configured for an additional number of PDCCH monitoring occasions in its PO to monitor for paging. However, when the UE detects a PDCCH transmission within the UE's PO addressed with P-RNTI, the UE is not required to monitor the subsequent PDCCH monitoring occasions within this PO.

If Paging Cause is included in the Paging message, a UE in RRC_IDLE or  RRC_INACTIVE state may use the Paging Cause as per TS 23.501[3]

End change