3GPP TSG-RAN WG3 #114bis-e							R3-221110

17-26 Jan 2022 Online

Agenda Item:	9.3.5.1

Source:	ZTE (moderator)

Title:	                Summary of Offline Discussion on CB: # 96_PC5QoS

Document for:	Approval

# Introduction

This paper summarizes the following email discussion:

CB: # 96_PC5QoS

- Check the details on 5QI->PQI and ASN.1 updates

- Other changes if agreeable

- Update the coversheet

(ZTE - moderator)

Summary of offline disc R3-221110

The deadline of the discussion is Friday, Jan 21st, 23:59:59 UTC.

# For the Chairman’s Notes

Propose the following:

TBD

# Discussion

## 5QI to PQI change

For R16 NR sidelink, PQI is defined and used as a reference to PC5 QoS characteristics. In addition, the one-to-one mapping of standardized PQI values to PC5 QoS characteristics is specified in TS 23.287.

### 5.4.4 Standardized PQI to QoS characteristics mapping The one-to-one mapping of standardized PQI values to PC5 QoS characteristics is specified in table 5.4.4-1. Table 5.4.4-1: Standardized PQI to QoS characteristics mapping PQI Value; Resource Type; Default Priority Level; Packet Delay Budget; Packet Error Rate; Default Maximum Data Burst Volume; Default Averaging Window; Example Services. 21; GBR; 3; 20 ms; 10-4; N/A; 2000 ms; Platooning between UEs – Higher degree of automation; Platooning between UE and RSU – Higher degree of automation. 22; (NOTE 1); 4; 50 ms; 10-2; N/A; 2000 ms; Sensor sharing – higher degree of automation. 23; 3; 100 ms; 10-4; N/A; 2000 ms; Information sharing for automated driving – between UEs or UE and RSU - higher degree of automation. 55; Non-GBR; 3; 10 ms; 10-4; N/A; N/A; Cooperative lane change – higher degree of automation. 56; 6; 20 ms; 10-1; N/A; N/A; Platooning informative exchange – low degree of automation; Platooning – information sharing with RSU. 57; 5; 25 ms; 10-1; N/A; N/A; Cooperative lane change – lower degree of automation. 58; 4; 100 ms; 10-2; N/A; N/A; Sensor information sharing – lower degree of automation. 59; 6; 500 ms; 10-1; N/A; N/A; Platooning – reporting to an RSU. 90; Delay Critical GBR; 3; 10 ms; 10-4; 2000 bytes; 2000 ms; Cooperative collision avoidance; Sensor sharing – Higher degree of automation; Video sharing – higher degree of automation. 91; (NOTE 1); 2; 3 ms; 10-5; 2000 bytes; 2000 ms; Emergency trajectory alignment; Sensor sharing – Higher degree of automation. NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications.. NOTE 1: For Standardized PQI to QoS characteristics mapping, the table will be extended/updated to support service requirements for other identified V2X services. NOTE 2: The PQIs may be used for other services than V2X. NOTE 3: A PQI may be used together with an application indicated priority, which overrides the Default Priority Level of the PQI.

The PQI is a special 5QI, but the standardized PQI values and the one-to-one mapping to QoS characteristics are different from 5QI values. The PQI values are not included in the 5QI to QoS characteristics mapping table, see the table in Annex. And even for the same value, in different tables it has different QoS characteristics. Therefore, moderator thinks it shall be clarified that we have two distinct tables for 5QI and PQI.

In section 9.3.1.126 in TS 48.473, for the Non Dynamic PQI Descriptor, if we use 5QI and refer to TS 23.501, we can’t find suitable PQI values/QoS parameters applicable for sidelink communication.

So, moderator suggests that the 5QI is changed to PQI to indicate the QoS Characteristics for a standardized or pre-configured PQI for sidelink. And the reference specification is changed to TS 23.287 [40] accordingly.

9.3.1.126	Non Dynamic PQI Descriptor

This IE indicates the QoS Characteristics for a standardized or pre-configured PQI for sidelink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PQI | M |  | INTEGER (0..255,...) | This IE contains the standardized or pre-configured PQI as specified in TS 23.287 [40] |
| QoS Priority Level | O |  | INTEGER (1..8,…) | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. |
| Averaging Window | O |  | 9.3.1.53 | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. |
| Maximum Data Burst Volume | O |  | 9.3.1.54 | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. |

Question 1: Do you agree to change 5QI to PQI and change the reference specification to TS 23.287?

| Company | Yes/No | View |
| --- | --- | --- |

## Reference spec for IE “Guaranteed Flow Bit Rate” and “Maximum Flow Bit Rate”

In 9.3.1.122 in TS 38.473, for IE “Guaranteed Flow Bit Rate” and “Maximum Flow Bit Rate”, they are used to indicate guaranteed bit rate and maximum bit rate for the PC5 QoS flow and referred to TS 23.501. But the related concept for PC5 QoS flow are specified in TS 23.287. On the other hand, in TS 23.287, it states that the GFBR and MFBR for PC5 QoS flow are defined as in clause 5.7.2.5 of TS 23.501.

The concern is whether the reference specification for IEs “Guaranteed Flow Bit Rate” and “Maximum Flow Bit Rate” shall be changed to TS 23.287.

TS 23.287 #### 5.4.2.2 PC5 Flow Bit Rates For GBR QoS Flows only, the following additional PC5 QoS parameters exist: - Guaranteed Flow Bit Rate (GFBR); - Maximum Flow Bit Rate (MFBR). The GFBR and MFBR as defined in clause 5.7.2.5 of TS 23.501 [6] are used for bit rate control on PC5 reference point over the Averaging Time Window. For PC5 communication, the same GFBR and MFBR are used for both directions.

9.3.1.122	PC5 QoS Parameters

This IE defines the QoS to be applied to a SL DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE PC5 QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic PQI |  |  |  |  | - |  |
| >>Non Dynamic PQI Descriptor | M |  | 9.3.1.126 |  | - |  |
| >Dynamic PQI |  |  |  |  | - |  |
| >>Dynamic PQI Descriptor | M |  | 9.3.1.127 |  | - |  |
| PC5 QoS Flow Bit Rates | O |  |  | Only applies for GBR QoS Flows. | - |  |
| >Guaranteed Flow Bit Rate | M |  | Bit Rate 9.3.1.22 | Guaranteed Bit Rate for the PC5 QoS flow. Details in TS 23.287 [40]. | - |  |
| >Maximum Flow Bit Rate | M |  | Bit Rate 9.3.1.22 | Maximum Bit Rate for the PC5 QoS flow. Details in TS 23.287 [40]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flows allowed towards one UE for NR sidelink communication, the maximum value is 2048. |

Question 2: Do companies think the reference specification for IEs “Guaranteed Flow Bit Rate” and “Maximum Flow Bit Rate” shall be changed to TS 23.287?

| Company | Yes/No | View |
| --- | --- | --- |

## ASN.1 change

If the answer to Question 1 is Yes, then we discuss whether the ASN.1 needs to be changed accordingly.

------------------------------------------------------------<text omitted>-----------------------------------------------

NonDynamicPQIDescriptor	::= SEQUENCE {

pQI						INTEGER (0..255, ...),

qoSPriorityLevel			INTEGER (1..8, ...)				OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { NonDynamicPQIDescriptor-ExtIEs } } OPTIONAL

}

------------------------------------------------------------<text omitted>-----------------------------------------------

Question 3: Do companies think the above ASN.1 change (change fiveQI to pQI) is needed?

| Company | Yes/No | View |
| --- | --- | --- |

In online, it is agreeable that the “PC5 Flow Bit Rates” is changed to “PC5 QoS Flow Bit Rates” for better reading of a superior specification. The concern is whether the ASN.1 needs to be changed accordingly.

------------------------------------------------------------<text omitted>-----------------------------------------------

PC5QoSParameters	::= SEQUENCE {

pC5-QoS-Characteristics				PC5-QoS-Characteristics,

pC5-QoS-Flow-Bit-Rates				PC5QoSFlowBitRates				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PC5QoSParameters-ExtIEs } }	OPTIONAL,

...

}

------------------------------------------------------------<text omitted>-----------------------------------------------

Question 4: Do companies think the above ASN.1 change is necessary?

| Company | Yes/No | View |
| --- | --- | --- |

Besides the above two ASN.1 changes,

Question 5: Do companies think any other ASN.1 change is needed?  If the answer is Yes, please give your view where should be changed.

| Company | Yes/No | View |
| --- | --- | --- |

## BC or NBC change

During online discussion, company queried whether the 5QI to PQI change is BC change or NBC change. For the 5QI to PQI change, since the different PQI/5QI values to QoS characteristics mapping table is referred to, it may be considered as a NBC change.

Question 6: Do companies think the attached CR is BC change or NBC change?

| Company | View |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-220378, CR to TS38.473: Correction on PC5 QoS parameters for NR V2X (ZTE)

# Annex

TS 23.501:

### 5.7.4	Standardized 5QI to QoS characteristics mapping

Standardized 5QI values are specified for services that are assumed to be frequently used and thus benefit from optimized signalling by using standardized QoS characteristics. Dynamically assigned 5QI values (which require a signalling of QoS characteristics as part of the QoS profile) can be used for services for which standardized 5QI values are not defined. The one-to-one mapping of standardized 5QI values to 5G QoS characteristics is specified in table 5.7.4-1.

Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms (NOTE 11, NOTE 13) | 10-2 | N/A | 2000 ms | Conversational Voice |
| 2 | (NOTE 1) | 40 | 150 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |
| 3 |  | 30 | 50 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Real Time Gaming, V2X messages (see TS 23.287 [121]). Electricity distribution – medium voltage, Process automation monitoring |
| 4 |  | 50 | 300 ms (NOTE 11, NOTE 13) | 10-6 | N/A | 2000 ms | Non-Conversational Video (Buffered Streaming) |
| 65 (NOTE 9, NOTE 12) |  | 7 | 75 ms (NOTE 7, NOTE 8) | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 66 (NOTE 12) |  | 20 | 100 ms (NOTE 10, NOTE 13) | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 67 (NOTE 12) |  | 15 | 100 ms (NOTE 10, NOTE 13) | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 75 (NOTE 14) |  |  |  |  |  |  |  |
| 71 |  | 56 | 150 ms (NOTE 11, NOTE 13, NOTE 15) | 10-6 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 72 |  | 56 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | 10-4 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 73 |  | 56 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | 10-8 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 74 |  | 56 | 500 ms (NOTE 11, NOTE 15) | 10-8 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 76 |  | 56 | 500 ms (NOTE 11, NOTE 13, NOTE 15) | 10-4 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 5 | Non-GBR | 10 | 100 ms NOTE 10, NOTE 13) | 10-6 | N/A | N/A | IMS Signalling |
| 6 | (NOTE 1) | 60 | 300 ms (NOTE 10, NOTE 13) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 |  | 70 | 100 ms (NOTE 10, NOTE 13) | 10-3 | N/A | N/A | Voice, Video (Live Streaming) Interactive Gaming |
| 8 |  | 80 | 300 ms (NOTE 13) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive |
| 9 |  | 90 |  |  |  |  | video, etc.) |
| 10 |  | 90 | 1100ms (NOTE 13) (NOTE 17) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |
| 69 (NOTE 9, NOTE 12) |  | 5 | 60 ms (NOTE 7, NOTE 8) | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 70 (NOTE 12) |  | 55 | 200 ms (NOTE 7, NOTE 10) | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |
| 79 |  | 65 | 50 ms (NOTE 10, NOTE 13) | 10-2 | N/A | N/A | V2X messages (see TS 23.287 [121]) |
| 80 |  | 68 | 10 ms (NOTE 5, NOTE 10) | 10-6 | N/A | N/A | Low Latency eMBB applications Augmented Reality |
| 82 | Delay-critical GBR | 19 | 10 ms (NOTE 4) | 10-4 | 255 bytes | 2000 ms | Discrete Automation (see TS 22.261 [2]) |
| 83 |  | 22 | 10 ms (NOTE 4) | 10-4 | 1354 bytes (NOTE 3) | 2000 ms | Discrete Automation (see TS 22.261 [2]); V2X messages (UE - RSU Platooning, Advanced Driving: Cooperative Lane Change with low LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 84 |  | 24 | 30 ms (NOTE 6) | 10-5 | 1354 bytes (NOTE 3) | 2000 ms | Intelligent transport systems (see TS 22.261 [2]) |
| 85 |  | 21 | 5 ms (NOTE 5) | 10-5 | 255 bytes | 2000 ms | Electricity Distribution- high voltage (see TS 22.261 [2]). V2X messages (Remote Driving. See TS 22.186 [111], NOTE 16, see TS 23.287 [121]) |
| 86 |  | 18 | 5 ms (NOTE 5) | 10-4 | 1354 bytes | 2000 ms | V2X messages (Advanced Driving: Collision Avoidance, Platooning with high LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 87 |  | 25 | 5 ms (NOTE 4) | 10-3 | 500 bytes | 2000 ms | Interactive Service - Motion tracking data, (see TS 22.261 [2]) |
| 88 |  | 25 | 10 ms (NOTE 4) | 10-3 | 1125 bytes | 2000 ms | Interactive Service - Motion tracking data, (see TS 22.261 [2]) |
| 89 |  | 25 | 15 ms (NOTE 4) | 10-4 | 17000 bytes | 2000 ms | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |
| 90 |  | 25 | 20 ms (NOTE 4) | 10-4 | 63000 bytes | 2000 ms | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |
| NOTE 1: A packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: It is required that default MDBV is supported by a PLMN supporting the related 5QIs. NOTE 3: The Maximum Transfer Unit (MTU) size considerations in clause 9.3 and Annex C of TS 23.060 [56] are also applicable. IP fragmentation may have impacts to CN PDB, and details are provided in clause 5.6.10. NOTE 4: A static value for the CN PDB of 1 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 5: A static value for the CN PDB of 2 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 6: A static value for the CN PDB of 5 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 7: For Mission Critical services, it may be assumed that the UPF terminating N6 is located "close" to the 5G_AN (roughly 10 ms) and is not normally used in a long distance, home routed roaming situation. Hence a static value for the CN PDB of 10 ms for the delay between a UPF terminating N6 and a 5G_AN should be subtracted from this PDB to derive the packet delay budget that applies to the radio interface. NOTE 8: In both RRC Idle and RRC Connected mode, the PDB requirement for these 5QIs can be relaxed (but not to a value greater than 320 ms) for the first packet(s) in a downlink data or signalling burst in order to permit reasonable battery saving (DRX) techniques. NOTE 9: It is expected that 5QI-65 and 5QI-69 are used together to provide Mission Critical Push to Talk service (e.g. 5QI-5 is not used for signalling). It is expected that the amount of traffic per UE will be similar or less compared to the IMS signalling. NOTE 10: In both RRC Idle and RRC Connected mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 11: In RRC Idle mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 12: This 5QI value can only be assigned upon request from the network side. The UE and any application running on the UE is not allowed to request this 5QI value. NOTE 13: A static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 14: This 5QI is not supported in this Release of the specification as it is only used for transmission of V2X messages over MBMS bearers as defined in TS 23.285 [72] but the value is reserved for future use. NOTE 15: For "live" uplink streaming (see TS 26.238 [76]), guidelines for PDB values of the different 5QIs correspond to the latency configurations defined in TR 26.939 [77]. In order to support higher latency reliable streaming services (above 500ms PDB), if different PDB and PER combinations are needed these configurations will have to use non-standardised 5QIs. NOTE 16: These services are expected to need much larger MDBV values to be signalled to the RAN. Support for such larger MDBV values with low latency and high reliability is likely to require a suitable RAN configuration, for which, the simulation scenarios in TR 38.824 [112] may contain some guidance. NOTE 17: The worst case one way propagation delay for GEO satellite is expected to be ~270ms, ,~ 21 ms for LEO at 1200km, and 13 ms for LEO at 600km. The UL scheduling delay that needs to be added is also typically two way propagation delay e.g. ~540ms for GEO, ~42ms for LEO at 1200km, and ~26 ms for LEO at 600km. Based on that, the 5G-AN Packet delay budget is not applicable for 5QIs that require 5G-AN PDB lower than the sum of these values when the specific types of satellite access are used (see TS 38.300 [27]). 5QI-10 can accommodate the worst case PDB for GEO satellite type. |  |  |  |  |  |  |  |

NOTE:	It is preferred that a value less than 64 is allocated for any new standardised 5QI of Non-GBR resource type. This is to allow for option 1 to be used as described in clause 5.7.1.3 (as the QFI is limited to less than 64).