3GPP TSG-RAN WG3 #114bis-e	R3-221296

17 - 26 Jan 2022

Online

Agenda item:	15.4

Source: 	ZTE

Title: 	(TP for BL CR of TS38.463) Alignment of MDT and QoE Measurements

Document for:   Agreement

# Introduction

This contribution provides TP for 38.463 of Alignment of MDT and QoE Measurements.

# 2. TP to BL CR of 38.463

- --------------------------------------------Start of Change -------------------------------------

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

<unchanged omitted>

If the QoE-MDT Alignment Information IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

- --------------------------------------------Next Change -------------------------------------

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

#### 8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

<unchanged omitted>

If the QoE-MDT Alignment Information IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

- --------------------------------------------Next Change -------------------------------------

### 9.2.2	Bearer Context Management messages

#### 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Security Information | M |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Serving PLMN | M |  | PLMN Identity 9.3.1.7 |  | YES | ignore |
| Activity Notification Level | M |  | 9.3.1.67 |  | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates the status of the Bearer Context | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List | M |  | DRB To Setup List E-UTRAN 9.3.3.1 |  | YES | reject |
| >>Subscriber Profile ID for RAT/Frequency priority | O |  | 9.3.1.69 |  | YES | ignore |
| >>Additional RRM Policy Index | O |  | 9.3.1.70 |  | YES | Ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List | M |  | 9.3.3.2 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE(8)) |  | YES | ignore |
| gNB-DU ID | O |  | 9.3.1.65 | Included whenever it is known by the gNB-CU-CP | YES | ignore |
| Trace Activation | O |  | 9.3.1.68 |  | YES | ignore |
| NPN Context Information | O |  | 9.3.1.84 |  | YES | reject |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.89 |  | YES | ignore |
| CHO Initiation | O |  | ENUMERATED (True, …) |  | YES | reject |
| Additional Handover Information | O |  | ENUMERATED(Discard PDCP SN, …) | If set to “Discard PDCP SN”, indicates that the forwarded PDCP SNs have to be removed | YES | ignore |
| Direct Forwarding Path Availability | O |  | 9.3.1.98 |  | YES | ignore |
| gNB-CU-UP UE E1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| QoE-MDT Alignment Information | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

- --------------------------------------------Next Change -------------------------------------

#### 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Security Information | O |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates the status of the Bearer Context | YES | reject |
| New UL TNL Information Required | O |  | ENUMERATED (required, …) | Indicates that new UL TNL information has been requested to be provided. | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Data Discard Required | O |  | ENUMERATED (required, …) | Indicate to discard the DL user data in case of RAN paging failure. | YES | ignore |
| CHOICE System | O |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List | O |  | DRB To Setup Modification List E-UTRAN 9.3.3.7 |  | YES | reject |
| >>DRB To Modify List | O |  | DRB To Modify List E-UTRAN 9.3.3.8 |  | YES | reject |
| >>DRB To Remove List | O |  | DRB To Remove List E-UTRAN 9.3.3.9 |  | YES | reject |
| >>Subscriber Profile ID for RAT/Frequency priority | O |  | 9.3.1.69 |  | YES | ignore |
| >>Additional RRM Policy Index | O |  | 9.3.1.70 |  | YES | ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List | O |  | PDU Session Resource To Setup Modification List 9.3.3.10 |  | YES | reject |
| >>PDU Session Resource To Modify List | O |  | 9.3.3.11 |  | YES | reject |
| >>PDU Session Resource To Remove List | O |  | 9.3.3.12 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE(8)) |  | YES | ignore |
| gNB-DU ID | O |  | 9.3.1.65 |  | YES | ignore |
| Activity Notification Level | O |  | 9.3.1.67 |  | YES | ignore |
| QoE-MDT Alignment Information | O |  | 9.3.1.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

- ------------------------------------------- Next Change -------------------------------------

#### 9.3.1.X	QoE-MDT Alignment Information

This IE provides QoE-MDT alignment information for gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoE-MDT Alignment Information list |  | 1 |  |  |
| > QoE-MDT Alignment information Item |  | 1..<maxnoofUEApplicationLayerMeas> |  |  |
| >>MCE IP Address | M |  | Transport Layer Address 9.3.2.4 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEApplicationLayerMeas | Maximum no. of UE application layer measurement. Value is FFS. |

- ------------------------------------------- Next Change -------------------------------------

### 9.4.4	PDU Definitions

<unchanged omitted>

IMPORTS

Cause,

CriticalityDiagnostics,

GNB-CU-CP-UE-E1AP-ID,

GNB-CU-UP-UE-E1AP-ID,

UE-associatedLogicalE1-ConnectionItem,

GNB-CU-UP-ID,

<unchaned omitted>

Extended-NR-CGI-Support-List,

DirectForwardingPathAvailability,

QoE-MDTAlignmentInformation,

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-CP-UE-E1AP-ID,

id-gNB-CU-UP-UE-E1AP-ID,

id-ResetType,

id-UE-associatedLogicalE1-ConnectionItem,

id-UE-associatedLogicalE1-ConnectionListResAck,

<unchanged omitted>

id-Extended-NR-CGI-Support-List,

id-DirectForwardingPathAvailability,

id-QoE-MDTAlignmentInformation,

maxnoofErrors,

maxnoofSPLMNs,

maxnoofDRBs,

maxnoofTNLAssociations,

maxnoofIndividualE1ConnectionsToReset,

maxnoofTNLAddresses,

maxnoofUEApplicationLayerMeas

<unchanged omitted>

-- Bearer Context Setup Request

--

-- **************************************************************

BearerContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextSetupRequestIEs} },

...

}

BearerContextSetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID						PRESENCE mandatory }|

{ ID id-SecurityInformation					CRITICALITY reject	TYPE SecurityInformation						PRESENCE mandatory }|

{ ID id-UEDLAggregateMaximumBitRate			CRITICALITY reject	TYPE BitRate									PRESENCE mandatory }|

{ ID id-UEDLMaximumIntegrityProtectedDataRate	CRITICALITY reject		TYPE BitRate							PRESENCE optional	 }|

{ ID id-Serving-PLMN						CRITICALITY ignore	TYPE PLMN-Identity								PRESENCE mandatory }|

{ ID id-ActivityNotificationLevel			CRITICALITY reject	TYPE ActivityNotificationLevel					PRESENCE mandatory }|

{ ID id-UE-Inactivity-Timer					CRITICALITY reject	TYPE Inactivity-Timer							PRESENCE optional  }|

{ ID id-BearerContextStatusChange			CRITICALITY reject	TYPE BearerContextStatusChange					PRESENCE optional  }|

{ ID id-System-BearerContextSetupRequest	CRITICALITY reject	TYPE System-BearerContextSetupRequest				PRESENCE mandatory }|

{ ID id-RANUEID								CRITICALITY ignore	TYPE RANUEID									PRESENCE optional }|

{ ID id-GNB-DU-ID							CRITICALITY ignore	TYPE GNB-DU-ID									PRESENCE optional }|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional }|

{ ID id-NPNContextInfo						CRITICALITY reject	TYPE NPNContextInfo								PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY	ignore	TYPE MDTPLMNList								PRESENCE optional}|

{ ID id-CHOInitiation						CRITICALITY reject	TYPE CHOInitiation								PRESENCE optional }|

{ ID id-AdditionalHandoverInfo				CRITICALITY ignore	TYPE AdditionalHandoverInfo						PRESENCE optional }|

{ ID id-DirectForwardingPathAvailability	CRITICALITY ignore	TYPE DirectForwardingPathAvailability		              PRESENCE optional }|

{ ID id-gNB-CU-UP-UE-E1AP-ID				CRITICALITY ignore	TYPE GNB-CU-UP-UE-E1AP-ID						PRESENCE optional }|

{ ID id-QoE-MDTAlignmentInformation	 	CRITICALITY ignore   TYPE QoE-MDTAlignmentInformation			              PRESENCE optional },

...

}

- ------------------------------------------- Next Change -------------------------------------

-- Bearer Context Modification Request

--

-- **************************************************************

BearerContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationRequestIEs} },

...

}

BearerContextModificationRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			       PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			       PRESENCE mandatory }|

{ ID id-SecurityInformation						CRITICALITY reject	TYPE SecurityInformation			       PRESENCE optional  }|

{ ID id-UEDLAggregateMaximumBitRate				CRITICALITY reject	TYPE BitRate						       PRESENCE optional  }|

{ ID id-UEDLMaximumIntegrityProtectedDataRate	CRITICALITY reject	TYPE BitRate					              PRESENCE optional	 }|

{ ID id-BearerContextStatusChange				CRITICALITY reject	TYPE BearerContextStatusChange		       PRESENCE optional  }|

{ ID id-New-UL-TNL-Information-Required			CRITICALITY reject	TYPE New-UL-TNL-Information-Required	       PRESENCE optional  }|

{ ID id-UE-Inactivity-Timer						CRITICALITY reject	TYPE Inactivity-Timer				       PRESENCE optional  }|

{ ID id-DataDiscardRequired					     CRITICALITY ignore	TYPE DataDiscardRequired				PRESENCE optional  }|

{ ID id-System-BearerContextModificationRequest	CRITICALITY reject	TYPE System-BearerContextModificationRequest	PRESENCE optional  }|

{ ID id-RANUEID									CRITICALITY ignore	TYPE RANUEID  					              PRESENCE optional }|

{ ID id-GNB-DU-ID								CRITICALITY ignore	TYPE GNB-DU-ID						       PRESENCE optional }|

{ ID id-ActivityNotificationLevel				CRITICALITY ignore	TYPE ActivityNotificationLevel			PRESENCE optional }|

{ ID id-QoE-MDTAlignmentInformation	 	          CRITICALITY ignore  TYPE QoE-MDTAlignmentInformation			PRESENCE optional },

...

}

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

- -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-QoSMonitoringDisabled,

id-PDCP-StatusReportIndication,

id-RedundantCommonNetworkInstance,

id-redundant-nG-UL-UP-TNL-Information,

id-redundant-nG-DL-UP-TNL-Information,

id-RedundantQosFlowIndicator,

id-TSCTrafficCharacteristics,

id-ExtendedPacketDelayBudget,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-AdditionalPDCPduplicationInformation,

id-RedundantPDUSessionInformation,

id-RedundantPDUSessionInformation-used,

id-QoS-Mapping-Information,

id-MDTConfiguration,

id-TraceCollectionEntityURI,

id-EHC-Parameters,

id-DAPSRequestInfo,

id-EarlyForwardingCOUNTReq,

id-EarlyForwardingCOUNTInfo,

id-AlternativeQoSParaSetList,

id-MCG-OfferedGBRQoSFlowInfo,

id-Number-of-tunnels,

id-DataForwardingtoE-UTRANInformationList,

id-DataForwardingtoNG-RANQoSFlowInformationList,

id-MaxCIDEHCDL,

id-ignoreMappingRuleIndication,

id-EarlyDataForwardingIndicator,

id-QoSFlowsDRBRemapping,

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofSPLMNs,

maxnoofMDTPLMNs,

maxnoofExtSliceItems,

maxnoofDataForwardingTunneltoE-UTRAN,

MaxnoofExtNRCGI,

maxnoofUEApplicationLayerMeas

<unchanged omitted>

QoE-MDTAlignmentInformation ::= SEQUENCE {

qoE-MDTAlignmentInformation-List	QoE-MDTAlignmentInformation-List,

iE-Extensions		ProtocolExtensionContainer { { QoE-MDTAlignmentInformation-ExtIEs} }	OPTIONAL,

...

}

QoE-MDTAlignmentInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoE-MDTAlignmentInformation-List::= SEQUENCE(SIZE (1.. maxnoofUEApplicationLayerMeas)) OF QoE-MDTAlignmentInformationItem

QoE-MDTAlignmentInformationItem ::= SEQUENCE {

mceIPAddres        		TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { { QoE-MDTAlignmentInformationItem-ExtIEs} }	OPTIONAL,

...

}

QoE-MDTAlignmentInformationItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

### 9.4.7	Constant Definitions

<unchanged omitted>

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofExtNRCGI 							INTEGER	::= 16384

maxnoofUEApplicationLayerMeas                                         INTEGER ::= FFS

-- **************************************************************

--

- -- IEs

--

-- **************************************************************

<unchanged omitted>

id-DataForwardingtoNG-RANQoSFlowInformationList					ProtocolIE-ID ::= 136

id-MaxCIDEHCDL													ProtocolIE-ID ::= 137

id-ignoreMappingRuleIndication									ProtocolIE-ID ::= 138

id-DirectForwardingPathAvailability								ProtocolIE-ID ::= 139

id-QoE-MDTAlignmentInformation									ProtocolIE-ID ::= xxx

- --------------------------------------------End of Change -------------------------------