3GPP TSG-RAN WG3 #114-bis-e	R3-221018

Online, 17 – 26 Jan 2022

Agenda Item:	10.2.1.5

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on SON MLB (CB #SONMDT3)

Document for:	Approval

# Introduction

CB: # SONMDT3_LoadBalance

- Turn WAs to agreements? Continue the discussion on the open issues from last meeting

- One eNB working in EN-DC should also be possible to get aware of the load information of potential target PSCell from other eNBs? Introducing ““Aggregated NR CAC”?

- Try to close this topic, capture agreements and provide the TPs if agreeable

(Nok - moderator)

Summary of offline disc R3-221018

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion (1st round)

## Per-beam Mobility Setting Change

It has been discussed for a long while and gained significant support at the #114 meeting. However, it has been questioned how CHO is to be addressed with the proposed mechanism. Now, in [6], it is explained that CHO shall not be subject to the mobility setting changes and thus the fact that it can’t be supported in case of per-beam MSC is not a problem.

Question 1-1: Considering the support at the #114 meeting and the explanations provided in [6], can RAN3 agree to enable the feature without CHO support?

| Company | Answer | If the answer is negative, please, explain why not. |
| --- | --- | --- |
| Nokia | No | We disagree with the arguments given in [6] that CHO and MSC are orthogonal. Actually, they seem very related: MSC is to amend mobility settings to balance load, while CHO is one of the forms of mobility. However, this is not a strong objection. We will accept per-beam MSC without CHO, if everybody else is fine to enable it. |

The issue of CHO is addressed also in [1], where it is explained that since nothing prevents using MSC for CHO currently, such limitations shall not be added for enhancements to the MSC. Two options are proposed there to include CHO in the per-beam MSC:

- Either we send an LS to RAN2 to ask for the needed enhancement to the measurement framework, so that the CHO criteria can be made beam-dependent; or

- Instead of negotiating per-beam mobility trigger, RAN3 enhances MSC so that the involved nodes can negotiate switching off overloaded beam.

Question 1-2: Considering that CHO can be used with the classic MSC, do you agree to address the problem of CHO in per-beam MSC? If yes, which of the methods above is preferred?

| Company | Answer | If the answer is negative, please, explain why not. |
| --- | --- | --- |
| Nokia | Yes (1 or 2, or both) | As explained above, we think CHO shall be part of the per-beam MSC. Of the two ways to enable it, we think option 2 is the easiest and fastest to implement (purely RAN3’s feature). However, sending an LS to RAN2 is also fine. RAN3 may also consider both options: switching off a beam can be a “max decrease/increase” of the mobility triggering point. |

Finally, also in [1], the solution for the F1AP that enables the DU to inform the CU about the HO triggering point for selected beams is discussed again. At #114 meeting and before, it was not considered needed for the per-beam MSC. At this meeting, it is explained it is useful also for static adjustment of the triggering point.

Question 1-3: Considering that [1] explains that the per-beam HO triggering point may be useful to help the DU to adjust the load in the static manner (i.e. not necessarily as a part of the per-beam MSC), would you consider it useful solution?

| Company | Answer | If the answer is negative, please, explain why not. |
| --- | --- | --- |
| Nokia | Yes |  |

## Per-slice Mobility Setting Change

It has been discussed for a long while and gained significant support at the #114 meeting. However, it has been questioned whether it is technically feasible considering that a UE may use services from different slices. Now, in [5], it is explained how this can work.

Question 2-1: Considering the support at the #114 meeting and the explanations provided in [5], can RAN3 agree to enable the feature?

| Company | Answer | If the answer is negative, please, explain why not. |
| --- | --- | --- |
| Nokia | Yes / neutral | We’re fine to enable it, though we encourage RAN3 to see if the proposed handling of the slices can be made even simpler. |

## Load in aggregated cells

It has been discussed for a long while and at #114 an agreement was reached:

RAN3 agrees to work on a solution as light as possible for informing about other cells that are relevant to UEs served by a cell and that can be configured as PSCell or SCell for the UE.

Now, solutions are proposed to implement the agreement:

- In [3], it is proposed to report CAC of the cells that are possible SgNB cells to the reporting cell. There two variants of the solution:

  - The CAC is reported per each possible SgNB (the reporting node may optimise the report to avoid duplications); or

  - A combined CAC per all possible SgNBs is provided.

- In [6], it is proposed that a CAC thresholds are set and the reporting node include a list of possible SgNBs (or candidates for CA) that meet the CAC threshold criteria.

- In [1], it is discussed that the load shall be reported per possible SgNB cell (as in solution 1 above), but the list shall be limited to the cells relevant for the UEs that may possibly be subjects of the MLB HOs. The actual signalling is not proposed though.

Question 3-1: Which of the methods proposed now is preferred?

| Company | Answer | Any comments or enhancement suggestions? |
| --- | --- | --- |
| Nokia | 3 | We proposed 3, however, without details yet. We think that thresholds-based filtering of solution 2 does not answer the need: the reported cells may still be not relevant for the UEs that may be subject of MLB mobility. On the other hand, opt. 1a may result in too large lists, while 1b offers very little information (perhaps it should be “minimum” CAC of all cells?). |

## Enhancements to the load reporting

It has been discussed for a while to enhance the load reporting with indication of stop, pause and resume. At #114 meeting, it was widely supported to enable at least the stop indicator. This is further discussed in [6]. However, in [10], the need for such enhancement is questioned.

This enables us to define following options:

- Full support for stop/pause/resume is enabled;

- Only stop indicator is added;

- No enhancement.

Question 4-1: Considering the support at the #114 meeting and the explanations provided in [6] (but also objections from [10], can RAN3 agree to enable full or partial support for stop/pause/resume?

| Company | Answer | Any comments or enhancement suggestions? |
| --- | --- | --- |
| Nokia | Yes (option 2) or no | We think the arguments exchanged before still hold. Therefore, “stop” is the only possible enhancement – if any is needed. |

## New load measurement

There are proposals to enable new load measurements:

- Inactive UEs:
It was discussed before and widely supported. The main concern was if the number of inactive UEs is to be reported per cell or per node. In [6,9] it is proposed to enable the information per cell. Furthermore, in [9], it is proposed to add the information on the overall capacity for storying UE contexts. In [10], however, it is explained that the information on the number of inactive UEs is not needed.

- PRB usage for MIMO:
It is a new proposal explained in [9]. This information is proposed to be reported per cell.

Question 5-1: Considering the arguments provided in [6,9,10], please, comment which of the enhancement can be agreed?

| Company | Answer | Any comments or enhancement suggestions to the supported option? |
| --- | --- | --- |
| Nokia | 1, if any | As discussed at #114 meeting, we still consider that the capability for storing inactive UEs is node’s capability, not cell’s (there seem to be no context fetch procedure enabled on F1AP, is there?). Regarding 2, we do not quite understand the motivation (i.e. what load it is supposed to measure and in which node)? |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-220155, Per-beam load balancing and load information from aggregated cells (Nokia, Nokia Shanghai Bell)

- R3-220156, [SON, TP to BL CR to TS 38.473]  The information on the per-beam HO triggering point (Nokia, Nokia Shanghai Bell)

- R3-220242, Discussion on PSCell MLB (CATT)

- R3-220252, Discussion on per-SSB and per-slice Mobility Settings Change (NEC)

- R3-220317, (TP for SON BLCR for 38.423) Load Balancing Enhancements (Huawei)

- R3-220576, (TP for SON BL CR for TS 38.423) MLB enhancements (Ericsson)

- R3-220577, (TP for SON BL CR for TS 36.423) MLB enhancements (Ericsson)

- R3-220711, (TP for 36.423) Support of PSCell MLB (CATT)

- R3-220870, (TP for SON BLCR to 38.423) Remaining issues on MLB (CMCC)

- R3-220885, Further Discussion on Load Balancing Enhancements in NR (ZTE, China Telecom)