3GPP TSG-RAN WG3 #115-e	R3-222411

21st February – 3rd March 2022

E-Meeting

Agenda Item:	30.2

Source:	Ericsson - Moderator

Title:	Summary of Offline Discussion – MultiSIM

Document for:	Approval

# Introduction

This is the SOD document for the following come back:

CB: # 19_MultiSIM

- Continue the discussion on remaining open issues

- Capture agreements and provide TPs/CRs if agreeable

(E/// - moderator)

Summary of offline disc R3-222411

Deadline by 23rd, Wednesday 12h UTC (before the online session)

# For the Chairman’s Notes

Agreements (from the First round):

- It is

# First round discussion:

## Remaining Issue 1: “Paging Cause support indication” IE: naming and Coding

By input document [1] ZTE, [3] Nokia, [5] Huawei, [9] Ericsson, [10] Samsung,

It is proposed to use the same naming and coding as in SA2 TS 23.501, naming “Paging Cause Indication for Voice Service”, coding “supported, …”

By the input document [12] CATT:

It is proposed to use naming “Paging Cause support indication”, coding “voice …”

Question 1: It is proposed to use naming “Paging Cause Indication for Voice Service”, coding “supported, …” Please indicate your view.

| - Company | - Comments |
| --- | --- |
| - Ericsson | - Support the proposal. - It is much easier to read the specification when SA2 TS and RAN TS using the same terminology. |

- Moderator Summary 1:

- Proposal 1: xx

## Remaining Issue 2: “Paging Cause” IE: Coding

There are two Options presented at the RAN3#114bis-e:

- Paging Cause IE Coding.  Choose from the below two Options:

  - Option 1: “voice, …”

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | M |  | ENUMERATED (voice, …) |  |

  - Option 2: A new IE structure as in below: This IE indicates the paging cause for paging a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | O |  | ENUMERATED (voice, …) | Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If this field is not present, it implies that the corresponding paging entry is for a service other than IMS voice. |

Option 1: By input document [5] Huawei, [9] Ericsson, [12] CATT:

It is proposed that the coding of ‘Paging Cause IE’ should be ‘ENUMERATED (voice, …).

Option 2: By input document [1] ZTE, [10] Samsung,

It is proposed to use the “Paging Record” proposal.

Option 3: By input document [3] Nokia,

It is proposed for the paging cause in Paging message, use the name “Paging Cause” and encode in an ENUMERATED (IMS voice, non IMS voice).

The discussion in RAN3 is related to that RAN2 has agreed “to provide the paging cause value “voice” to UEs by extending the Paging message. The presence of the extension, which can be empty list if none of the UEs have paging cause “voice”, indicates that network supports transmission of paging cause for service indication”,

It is clear from RAN2 that the empty list is used to indicate if the network supports transmission of paging cause, to differentiate from the Rel 16 network, which would not support the transmission.

Question 2: The presence of the “RAN2 list” indicates the gNB/eNB supports the transmission of paging cause for service indication. If the service is "voice", the list will include the Paging cause "voice". If the service is other than IMS voice, the list will be empty.

Do you agree? Answer in Yes/No with comment.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - Yes | - We support the above understanding. - gNB/eNB knows if it supports the transmission of the paging cause for service indication. CN only needs to indicate if the paging cause is for “voice”. |

Three Options are listed here for clarity:

Option 1: (if to introduce a new IE, otherwise it is defined directly as ENUMERATED)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | M |  | ENUMERATED (voice, …) |  |

Option 2: A new IE is defined. The Semantics describes more details.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | O |  | ENUMERATED (voice, …) | Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If this field is not present, it implies that the corresponding paging entry is for a service other than IMS voice. |

Option 3: (if to introduce a new IE, otherwise it is defined directly as ENUMERATED)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | M |  | ENUMERATED (iMS voice, non IMS voice) |  |

Question 3: In the above three options, what is your preferred option? Answer in Choice with comments.

| - Company | - Choice | - Comments |
| --- | --- | --- |
| - Ericsson | - Option 1 | - The empty list in RAN2 specification is related to if gNB/eNB support the transmission of paging cause, there is no need to involve CN. - The solution that CN only needs to indicate “voice” in the paging cause is also beneficial in the RAN sharing scenario. |

- Moderator Summary 2:

- Proposal 2: xx

## Remaining Issue 3: New Cause value over NGAP “Release due to MUSIM”,

By input document [3] Nokia discussed Cause for switching:

In case of NAS Leave, the AMF will send NG Release Command to gNB, If we don’t have a new dedicated cause value, this could hit KPI i.e. this is a specific intentional release, not a failure release.

It is proposed to introduce a new specific cause value in TS 38.413 for release due to MUSIM NAS Leave.

By input document [9] Ericsson, the benefit to introduce a new cause value is not seen.

Question 4:  Do you see the need to differentiate the Context Release due to MUSIM NAS leave? Answer in Yes/No with comments.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Ericsson | - No | - NAS leave is a NAS based solution. If AMF makes a decision to release UE context, it is said to use a legacy release. We have not differentiated all the other causes for AMF release, it is thus difficult to motive this new release cause. |

- Moderator Summary 3:

- Proposal 3: xx

  - MUSIM gap configuration from DU to CU

By input document [5] Huawei,

- It is proposed to Introduce a MUSIM GapConfig IE in the DU to CU RRC Information in F1AP to specify the MUSIM gap configuration.

Question 5:  Should gNB-DU signal MUSIM Gap to gNB-CU via F1AP? Answer in Yes/No with comments.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |

- Moderator Summary 4:

- Proposal 4: xx

  - Remove the paging cause from DL DATA NOTIFICATION in E1AP

By the input documents [5] Huawei and [12] CATT: we already agree add the Paging Cause in DL DATA NOTIFICATION in E1AP. But from SA2 specification TS23.501 [2] the paging cause should interpret from Paging Priority Indicator (PPI) which already include in the message DL DATA NOTIFICATION. So the CU-CP also can derive the paging cause from Paging Priority Indicator (PPI).

For a UE in RRC-Inactive, the NG-RAN should provide the Voice Service Indication in the RAN Paging message only when there is Paging Cause Indication for Voice Service indication in the UE context and detects the downlink data which triggers the RAN Paging message is related to voice service based on the Paging Policy Indicator, in the header of the received downlink data, as specified in clause 5.4.3.2

It is proposed to remove/rediscuss the paging cause from DL DATA NOTIFICATION in E1AP

Question 6: Should the Paging cause from DL DATA NOTIGICATION in E1AP be removed? (Please provide Yes/No and comments)

| - Company | - Yes/No | - Comments |
| --- | --- | --- |

- Moderator Summary 5:

- Proposal 5: xx

# References

- R3-221792	(TP for BLCR Introduction of MultiSIM support over S1) Support of Multi-SIM	ZTE

- R3-221793	(TP for BLCR Introduction of MultiSIM support over NG/Xn/F1) Support of Multi-SIM	ZTE

- R3-221820	(TP for TS 38.413) Conclusions on Multi-USIM devices	Nokia, Nokia Shanghai Bell

- R3-221821	(TP for TS 38.473) Conclusions on Multi-USIM devices	Nokia, NOkia Shanghai Bell

- R3-221926	Discussion on MUSIM leftover issues 	Huawei

- R3-221927	(TP to BL CR 38.473) MUSIM gap configuration	Huawei

- R3-221930	Introduction of Multi-USIM Support for TS 38.300 (RadiSys, Reliance JIO, Ericsson)	draftCR

- R3-222035	TP on Introduction of MultiSIM support over Xn	Ericsson

- R3-222036	MUSIM reaming issues	Ericsson

- R3-222117	(TP for LTE_NR_MUSIM CR for TS 38.413) Discussion on open issues for MUSIM UE 	Samsung

- R3-222118	(TP for LTE_NR_MUSIM CR for TS 38.473) Discussion on supporting MUSIM gap configuration		Samsung

- R3-222215	Discussion on Multi-SIM	CATT

- R3-222216	(TP to BLCR for TS 38.463) Multi-SIM CATT