3GPP TSG-RAN3 Meeting #115-e	R3-22xxxx

21 Feb. – 3 Mar. 2022

Title: 	TP for SON BLCR for TS 38.423: Support of SON for NR-U

Source: 	Samsung

Agenda item:	10.5

Document for:	Other

# 1	Introduction

This contribution provides a TP for TS38.423 to support SON for NR-U.

# TP for SON BLCR to TS 38.423

============ First Change ==============

#### 9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >> RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>NR-U Channel List |  | 0..1 |  |  |  |  |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  |  |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | The NR-U channel utilised in the last reporting period |  |  |
| >>>>Channel occupancy time percentage | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding cell. Value 100 corresponds to the duration between consecutive reporting. |  |  |
| >>>>Percentage of Successful LBT (FFS) | M |  | INTEGER (0..100) | Calculated as Total Number of Successful LBT over total Number of LTB multiplied by 100. |  |  |
| >>>>LBT Sensing Duration (FFS) | M |  | INTEGER (0..100,…) | The percentage of time during which LBT procedure is carried out in DL. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U channel IDs in a cell. Value is xx. In this release, the actual maximum number is 4. |

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| <unchanged part is omitted> |  |  |  |  |  |  |
| CSI-RS Transmission Indication | O |  | ENUMERATED (activated, deactivated, ...) | This IE indicates the CSI-RS transmission status of the given cell. | YES | ignore |
| SFN Offset | O |  | 9.2.2.75 |  | YES | Ignore |
| NR-U Channel List |  | 0..1 |  |  |  |  |
| >NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >> NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) |  | – |  |
| >>NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) |  | – |  |
| >>Bandwidth | M |  | ENUMERATED (10MHz, 20MHz, 40MHz, 60 MHz, 80 MHz, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNR-UChannelIDs | Maximum no. NR-U channel IDs in a cell. Value is xx. In this release, the actual maximum number is 4. |

============ End of Change ==============