3GPP TSG-RAN WG3 #116-e								R3-223717

9th – 19th May 2022 Online

Agenda Item:	9.3.6

Source:	China Telecom (moderator)

Title:	Summary of Offline Discussion on CB#4 PRACH Coordination between LTE and NR

Document for:	Approval

# Introduction

CB: # 4_PRACH

- Turn the current WA into an agreement? No further standardization is needed in RAN3 or other working groups to support LTE-NR co-existence scenario under solution 1?

- Introduce a new indication “Only Resource Coordination” in Served NR Cell Information IE and NR Neighbour Information IE in EN-DC X2 Setup/Configuration Update? Specify in 36.300 that an EN-DC X2 interface can be configured to be used for resource coordination only?

- Close this topic, capture agreements and provide CRs if agreeable

(CT - moderator)

Summary of offline disc R3-223717

# For the Chairman’s Notes

Propose the following:

# Discussion

## First Round

The progress on resource Coordination between LTE and NR at RAN3 #115-emeeting are listed below:

WA: Solution 1 is agreed to support resource coordination between LTE and NR SA sites.

Whether there is any spec impact on RAN3 and other WG will be discussed at next meeting.

At this meeting, we have three discussion papers, as well as corresponding draft CRs. For the sake of discussion, we paste the proposals from four discussion papers as below:

In the paper from Nokia [1], the proposals are:

Observation 1: A gNB acting as an en-gNB supports an EN-DC X2 interface.

Observation 2: In case a gNB can act as an en-gNB, the Protected E-UTRA Resource Indication IE in E-UTRA-NR Cell Resource Coordination procedure can be re-used to protect PRACH Resources in case of an LTE-NR collocated scenario.

Observation 3: The No EN-DC attribute in the NCR can be reused so that X2 interface is not utilized for EN-DC purposes.

Observation 4: PRACH resource coordination in a LTE-NR co-existence scenario can be achieved without the need of any further standardization under solution 1.

Proposal 1: We propose to turn the current WA into an agreement:

Solution 1 is agreed to support resource coordination between LTE and NR SA sites.

Proposal 2: No further standardization is needed in RAN3 or other working groups to support LTE-NR co-existence scenario under solution 1.

In the paper from China Telecom [2], the proposals are:

Proposal 1: to introduce a new indication “Only Resource Coordination” in Served NR Cell Information IE and NR Neighbour Information IE in EN-DC X2 Setup/Configuration Update.

Proposal 2: the corresponding stage 2 texts on “Only Resource Coordination” is also needed in TS37.340.

Proposal 3: to introduce a new attribute” Only Resource Coordination” in ANR function of TS36.300.

Proposal 4: it is need to send a LS to SA5 on support of the new attribute “Only Resource Coordination” in EN-DC.

In the paper from Ericsson [4], the observations and proposals are:

Proposal 1: Adoption of Solution 1 implies no changes to the current standard

Proposal 2: As an enhancement, it is proposed to specify in 36.300 that an EN-DC X2 interface can be configured to be used for resource coordination only

### To turn the WA on solution 1 into an agreement

In Solution 1, since SA gNB also acts an en-gNB, it can establish an EN-DC X2 interface to enable the coordination of resources between co-channel sharing LTE and NR cells. In RAN3#115e meeting, the following WA on solution1 was agreed:

Solution 1 is agreed to support resource coordination between LTE and NR SA sites.

The contribution [1] proposed to turn the above WA into an agreement. In light of all contributions submitted in this meeting support solution 1, moderator proposes to agree this proposal.

Q1: do companies agree to turn the WA on Solution 1 into agreement?

Please provide any view / comments on this topic and the question in bold below:

| Company | Yes/No | Comment |
| --- | --- | --- |

### Standardization impact

#### TS36.423

Regarding the standardization impact on X2 interface, operator considers the SA gNB in solution 1 acts a special “en-gNB” which only supports the following procedure in X2 interface:

- EN-DC X2 Setup

- EN-DC Configuration Update

- EN-DC X2 Removal

- E-UTRA - NR Cell Resource Coordination

Given that the Configured TAC and 5GS-TAC could not be used to indicate the mode “Only Resource Coordination”, [2] proposes to introduce a new indication “Only Resource Coordination” in Served NR Cell Information IE and NR Neighbour Information IE to inform LTE eNB about the sender node is a special en-gNB which only supports global EN-DC procedures and E-UTRA-NR Cell Resource Coordination.

In contrast to the view in [2], [1][4] considers Solution 1 implies no changes to the current standard. This is because the specifications already allow a physical node hosting a gNB to also host an en-gNB. For that, the physical node is able to establish an EN-DC X2 interface with any neighbour eNB and to run the E-UTRA – NR Cell Resource Coordination procedure over it.

Question 2: which option do companies prefer?

- Option 1: to introduce a new indication in X2 interface

- Option 2: No changes to the current standard

please provide any view / comments on this topic below:

| Company | Option1/Option2 | Comment |
| --- | --- | --- |

#### TS37.340

The contribution [2] considers the “only resource coordination” is also a new functionality for en-gNB. Therefore, it proposes to add the corresponding stage 2 texts on “only resource coordination” in TS37.340.

Question 3: do companies agree to the stage 2 texts on “only resource coordination” in TS37.340?

| Company | Yes/No | Comment |
| --- | --- | --- |

#### TS36.300

Per TS36.300, the existing NCR from a source E-UTRA cell to a target NR cell means that eNB controlling the source cell knows the NCGI and PCI of the target cell. If an NCR from a source E-UTRA cell to a target E-UTRA cell exists, the eNB controlling the source cell has information whether the target E-UTRA cell has an existing NCR to a target NR cell for performing EN-DC.

The contributions [1][2][4] consider the attribute of NCR can be used to configure the EN-DC X2 interface for resource-coordination-only procedures. On which attribute could be used to indicate resource-coordination-only, there are two options according to the contributions submitted in this meeting:

- Option 1: [2][4] proposes to introduce a new field “Only Resource Coordination” in the NCR which when checked, neighbour relation shall only use the X2 interface to coordinate resources between the source and the target cell.

- Option 2: [1] proposes to reuse the existing attribute “No EN-DC” which when checked, he Neighbour Cell Relation shall not be used by the eNB for EN-DC.

Moderator would like to invite companies to provide your views on which solution(s) is acceptable.

Question 4: which option do companies prefer?

| Company | Option1/Option2 | Comment |
| --- | --- | --- |

#### To send a LS to SA5?

The contribution [2] considers SA5 related specification shall be updated to support the new attribute “Only Resource Coordination”. Therefore, a LS to SA5 is needed.

Question 5: do companies agree to send a LS to SA5?

| Company | Yes/No | Comment |
| --- | --- | --- |

## Second Round (if needed)

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-223176 Standards impacts of Resource Coordination between LTE and NR (Nokia, Nokia Shanghai Bell)

- R3-223320 Support of EN-DC X2 connection between SA gNB and LTE eNB (China Telecom)

- R3-223329 Support of the coordination of resources between co-channel sharing LTE and NR cells (China Telecom)

- R3-223406 Conclusions on PRACH Coordination Between LTE and NR (Ericsson)

- R3-223507 introduction of new attributes Only Resource Coordination in ANR (China Telecom)

- R3-223519  introduction of only resource coordination in EN-DC (China Telecom)