3GPP TSG-RAN WG3 #116-e	R3-223762

Online, 9 - 19 May 2022

Agenda Item:	9.3.9

Source:	NEC (moderator)

Title:	Summary of Offline Discussion on CB: # 57_PDCPCount

Document for:	Approval

# Introduction

CB: # 57_PDCPCount

- Evaluate the solutions

- Provide CRs if agreeable

(NEC - moderator)

Summary of offline disc R3-223762

The 1st round of discussion is set to deadline on 13th May (Friday) 23:59 UTC.

The 2nd round of discussion (if needed) is set to deadline on TBC

# For the Chairman’s Notes

[TBD]

# Discussion (1st round)

Refer to discussion in R3-223181:

If the handover involving Full Configuration, the 38.300 specifies that the PDCP SN and HFN are reset.

During the inter-gNB-DU handover, target gNB-DU may decide to generate CellGroupconfig using full configuration, the target gNB-DU indicate the Full Configuration IE as “true” in the F1AP: UE CONTEXT SETUP RESPONSE message. In this case, the PDCP COUNT (SN + HFN) will need to be reset. When the same gNB-CU-UP is kept, the gNB-CU-CP need to indicate to the gNB-CU-UP to reset the PDCP COUNT.  The question is how to do that in E1AP.

During the online discussion, some possible solutions were raised:

Solution 1:  The proposed solution in R3-223181 (with proposed CR in R3-223182, 3183, 3184) i.e in BEARER CONTEXT MODIFICATION REQUEST message the PDU Session Resource To Modify List IE to release the relevant DRBs (set the DRB To Remove List IE)and also to setup the same DRBs (set the DRB To Setup List IE). Furthermore, because of the need to give the target gNB-DU’s TNL information, then need to newly add the DL UP Parameter IE in the DRB To Setup List IE.

By this solution 1, all are done in one Bearer Context Modification procedure. The specification impact of this solution 1 would be to add the DRB To Setup List IE in the PDU Session Resource To Modify List IE and to add some procedure text (see CR in R3-223182, 3183, 3184)

Solution 2:  The gNB-CU-CP initiates the same Bearer Context Modification procedure two times i.e. gNB-CU-CP first to send E1AP: BEARER CONTEXT MODIFICATION REQUEST message the PDU Session Resource To Modify List IE to release the relevant DRBs  (set the DRB To Remove List IE)and also to setup the same DRBs (set the DRB To Setup List IE). After this first Bearer Context Modification procedure is complete, then gNB-CU-CP initiates again the same Bearer Context Modification procedure to give the target gNB-DU’s TNL information to the gNB-CU-UP (set the DL UP parameter IE in the DRB to Modify List IE).

The solution 2 use the existing IE. The specification impact may need to add some procedure text to describe the usage. However this solution 2 need to execute two times the Bearer Context Modification procedure so a concern is that this will increase handover delay which may create negative impact on the handover.

Solution 3: Use of the PDCP SN Status Information IE i.e. in BEARER CONTEXT MODIFICATION REQUEST message the PDU Session Resource To Modify List IE, set the PDCP SN and HFN as “0” in the PDCP SN Status Information IE of the DRB To Modify List IE .

This solution 3 use the existing IE. In E1AP 9.3.1.58 currently explain that “This IE contains information about PDCP PDU transfer status of a DRB”, so this is currently not for the purpose to reset the PDCP SN value. Then the specification impact would be to add new usage of the PDCP SN Status Information IE. further consequence, if any, may need to analysis.

## Companies to provide valuable comments for the possible solutions

Q1: which solution (solution 1, 2, 3) do you see is feasible, and also give comment from your analysis.

| Company | Comment |
| --- | --- |

Moderator Summary :

Q2: any other solution or other comments?

| Company | Comment |
| --- | --- |

Moderator Summary :

# Discussion (2nd round, if needed)

# Conclusion, Recommendations [if needed]

If needed

# References

- [1] R3-223181 Handling of PDCP COUNT reset in CU-UP for inter-gNB-DU Handover (NEC, ZTE)

- [2] R3-223182, Handling of PDCP COUNT reset in CU-UP for inter-gNB-DU Handover (NEC, ZTE) Rel-15 CR to 38.463

- [3] R3-223183, Handling of PDCP COUNT reset in CU-UP for inter-gNB-DU Handover (NEC, ZTE) Rel-16 CR to 38.463

- [4] R3-223184, Handling of PDCP COUNT reset in CU-UP for inter-gNB-DU Handover (NEC, ZTE) Rel-17 CR o 37.483

- Annex: 38.463v15.10.0 relevant part

#### 9.3.3.11	PDU Session Resource To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Security Indication | O |  | 9.3.1.23 | This IE is not used in this release. | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | - | - |
| >NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >PDU Session Data Forwarding Information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Network Instance | O |  | 9.3.1.62 | This IE is ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 0..1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flow Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | O |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | O |  | 9.3.1.38 |  | - | - |
| >>>DRB Data forwarding information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >>>PDCP SN Status Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP SN Status in the response message. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status to the target gNB-CU-UP. | - | - |
| >>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>Cell Group To Add | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Modify | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Remove | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.25 | Overrides previous mapping information. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>Old QoS Flow List - UL End Marker expected | O |  | QoS Flow List 9.3.1.12 | Indicates that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. | YES | reject |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >S-NSSAI | O |  | 9.3.1.9 |  | YES | reject |
| >Security Indication Modify | O |  | Security Indication 9.3.1.23 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.1.58	PDCP SN Status Information

This IE contains information about PDCP PDU transfer status of a DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP Status Transfer UL |  | 1 |  |  | – |  |
| >Receive Status Of PDCP SDU | O |  | BIT STRING (SIZE(1.. 131072)) | The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | – |  |
| >UL COUNT Value | M |  | PDCP Count 9.3.1.35 | PDCP-SN and Hyper Frame Number of the first missing UL SDU | – |  |
| PDCP Status Transfer DL |  | 1 |  |  | – |  |
| >DL COUNT Value | M |  | PDCP Count 9.3.1.35 | PDCP-SN and Hyper Frame Number that the target NG-RAN node (handover) or the NG-RAN node to which the DRB context is transferred (dual connectivity) should assign for the next DL SDU not having an SN yet. | – |  |

#### 9.3.1.35	PDCP Count

This IE include the PDCP Count information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| >PDCP SN | M |  | INTEGER (0 .. ..2PDCP_SN_Size-1) | The PDCP SN Size is provided in the PDCP Configuration IE. |
| >HFN | M |  | INTEGER (0 .. 232-PDCP_SN_Size-1) | The PDCP SN Size is provided in the PDCP Configuration IE. |