3GPP TSG-RAN WG3 Meeting #116-e	R3-223725

- 9th May - 19th May 2022

Online

Agenda Item:	17.3

Source:	Ericsson

Title:	SoD on CB: # Slice2_Others

Document for:	Discussion, Decision

# Introduction

CB: # Slice2_Others

- Check the LS from SA5 and send reply LS if needed

- Check the corrections for UE-Slice_MBR

(E/// - moderator)

Summary of offline disc R3-223725

Structure of the discussion:

First round comments to be provided by Friday the 13th at 12UTC

Second round comments to be provided by Tuesday the 17th  at 13UTC

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

## Discussion on Reply LS on Clarification of SCG outside RA for slicing

In the LS in [1] RAN3 asked the following question to SA2:

RAN3 is discussing a solution for resource optimisation for a slice 1 in a gNB cell called Multi Carrier Resource Sharing solution. This solution assumes that the gNB hosts another cell with different frequency and overlapping coverage where the same slice 1 is available. It consists of setting up DC using SCG resources on the “other cell” for offloading the PDU sessions of slice 1 of some or all UEs.

RAN3 is not sure if gNB can select such “other cell” outside of the RA (registration area) of the concerned UE as SCG resource (i.e. the secondary cell group in Dual Connectivity.).

In [2] SA2 replied that:

[…]

as the SCG is not used for paging nor considered when triggering Mobility Registration Updates by NAS, there is no functional issue to use a SCG cell outside the RA.

On the principle of potentially using a cell that does not support the network slice the UE is using. SA2 indicates that only cells of TAs defined to support the used network slices shall be allowed, and that such restriction applies also to SCG. SA2 does not foresee any impacts on SA2 specifications.

In [3] RAN3 sent an LS to SA5 asking the following questions:

Q1: How dynamically (i.e. with which frequency) can the modification to slice resource re-partitioning be performed?

Q2: is the solution shown in the figure above feasible?

To this, SA5 replied with the LS in [4] stating that:

Views of SA5: There are no limitation posed by the standard on how dynamic the process of  slice resource re-partitioning can be. This is considered to be dependent on the use case and on implementation.

Views of SA5: Yes. The solution proposed by RAN3 is feasible based on NRM and measurements defined in SA5 TSs.

In relation to the discussions in the LS exchange with SA2 and SA5, RAN3 agreed the CR in [8], where RAN3 followed the guidelines contained in the LSs from SA2 and SA5 in [2] and [4].

In [7] it is proposed to reply to SA2 (with SA5 in Cc), stating that RAN3 has taken the inputs from SA2 into account, and that RAN3 has converged on the agreements captured in [8].

The moderator believes that the reply LS in [7] would need to be revised, to at least add SA5 in the “To” list and to state that the LS is a reply also to the LS in [4] from SA5.

Companies are invited to provide their view on whether the LS in [7] can be revised (including SA5 in the “To” list and making it a reply also to the LS in [4]) and agreed.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Ericsson | Yes | It is appropriate to inform SA2 and SA5 of the progress in RAN3, especially because two LS threads were opened on this topic. |

## Discussion on presence of UE-Slice-MBR

In [5] a discrepancy between tabular and ASN.1 has been detected for the presence of the UE-Slice-MBR. The discrepancy consists of the UE-Slice-MBR being optional in ASN.1 and conditional in the tabular.

[5] states the following in the “reason for change”:

The gNB-DU UE Slice Maximum Bit Rate List IE was introduced to indicate the maximum aggregate UL bit rate per slice.

in the UE CONTEXT SETUP REQUEST message, this IE is conditional of the DRB setup as “C-ifDRBSetup”, but is optional in ASN.1.

Note that the gNB-DU UE Aggregate Maximum Bit Rate Uplink IE in the UE CONTEXT SETUP REQUEST message is also conditional of “C-ifDRBSetup”, and conditional in ASN.1.

Companies are invited to express their views on whether [5] is agreeable and if not on how to resolve the identified discrepancy

| Company | Comment |
| --- | --- |
| Ericsson | The UE AMBR was introduced since the very beginning of 5G and for that its presence has been made conditional to the setup of a DRB. Namely, the UE AMBR becomes mandatory if at least one DRB is setup. However, the UE-Slice-MBR was introduced only in Rel17 and its support is not mandatory. The latter is confirmed in TS38.413, which states the presence of the UE Slice Maximum Bit Rate List IE as “optional”. The same is confirmed also in TS23.501, which states the following: A UE subscription information may include an optional Slice Maximum Bit Rate for the UE (Subscribed UE-Slice-MBR) for an S-NSSAI, which applies for 3GPP access type only. Therefore, it is easy to understand from the above that a DRB may be setup over the F1 but the UE-Slice-MBR may not be present for the slice associated to the DRB. For this reason, the correction in [5] is not correct. In fact, the the correction in [5] implies that whenever a DRB is setup, the UE-Slice-MBR shall be present, which in turns implies that support of the UE-Slice-MBR is mandatory whenever at least one DRB is setup. This contradicts existing specifications. The optional presence of the UE-Slice-MBR in ASN.1 is correct and the identified discrepancy in 38.473 should be resolved by making the UE-Slice-MBR presence optional in the tabulars as well. |

In [6] a discrepancy concerning the criticality of the UE-Slice-MBR has been detected. The reason for change states that:

In the S-NODE ADDITION REQUEST and RETRIEVE UE CONTEXT RESPONSE messages, the criticality of the S-NG-RAN node UE Slice Maximum Bit Rate IE/ UE Slice Maximum Bit Rate List IE is set to “reject”, while in other messages, e.g., in HANDOVER REQUEST/ S-NODE MODIFICATION REQUEST message, it is set to “ignore”.

Note that in the BLCR CR0745, the criticality of this IE is set to “ignore” in the corresponding Tabular.

Companies are invited to present their views on whether the CR in [6] can be agreed

| Company | Comment |
| --- | --- |
| Ericsson | Yes, it can be agreed. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-216237, LS on Clarification of SCG outside RA for slicing (RAN3)

- R3-222738, Reply LS on Clarification of SCG outside RA for slicing (SA2)

- R3-216238, Response to LS Reply LS on Enhancement of RAN Slicing (RAN3)

- R3-223061, LS on Response to LS on Enhancement of RAN Slicing from RAN3 (SA5)

- R3-223469, Correction of the presence of UE-Slice-MBR (Huawei, CATT, CMCC)

- R3-223470, Correction of the criticality of UE-Slice-MBR (Huawei, CATT, CMCC)

- R3-223405, Reply LS to Reply LS on Clarification of SCG outside RA for slicing (Ericsson)

- R3-222665, (TP for TS 38.300 and TS38.423) Handling of Slice Resource Shortage (Nokia, Nokia Shanghai Bell)