3GPP TSG-RAN WG3 #117-bis-e	R3-225930

10th – 18th Oct 2022, Online

Agenda Item:	17.2

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on CB: # NTN1_ServiceContinuity

Document for:	Approval

# Introduction

CB: # NTN1_ServiceContinuity

- Cell ID for NTN mobility, Uu cell ID and/or Rel-17 defined mapped cell ID?

- Multiple TACs over Xn?

- Any enhancement on time-based CHO, e.g., Xn based and/or NG based CHO, NTN Cell Coverage Stop Time?

- Enhancements on Feeder Link Switch-over?

- Capture agreements and provide CRs if agreeable

- LS to RAN2?

(HW - moderator)

Summary of offline disc R3-225930

The discussion has two phases:

Phase 1: Identify potentially achievable agreements for online discussion.

Phase 2: Based on first round discussion, discuss the remaining issues and CRs if needed.

The deadline for Phase 1 is Tuesday, Oct 11th, 6:00 am UTC. This allows the moderator to prepare the proposals for Tuesday online session.

# For the Chairman’s Notes

Propose to capture the following Agreement:

…

# Phase 1 Discussion

## Issue 1: Clarification on Cell Identifier in hand-over signalling

In the last meeting, one remaining issue is whether we specify clearly it is Uu cell ID used in handover signalling or we rely on R17 mechanism, where the mapping relationship is known and no need to specify the cell ID type. In [1], a detailed analysis has been given to compare the two approaches, where it point out that specify clearly it is Uu cell ID used has following advantages:

- Supports Inter Vendor Operability - Mapped Cell ID is OAM configured value which is vendor specific unlike Uu Cell ID. Hence the mapped cell Id can be understood only by Intra vendor gNBs.

- Uu Cell ID is transparent and Unambiguous - Internal translation of Uu Cell ID reported by UE in Measurement Report to Mapped Cell ID for Handover message is not needed. The Uu Cell ID reported by UE can be sent in Handover message too.

- Identification of Beam ID – Uu Cell ID is mapped to the beams in a NTN cell. When target receives the Uu Cell ID, target can identify the beam where the UE can handed over. But this is not possible with Mapped Cell ID.

- Similar to TN handover – There is only one cell ID which is Uu Cell ID used in the existing TN handovers. Hence it is preferred to follow the same.

In addition, contributions ([2], [4]) mention that it is difficult for source gNB to know whether the geographical area corresponds to a Mapped Cell ID is served by more than 1 Uu cell from target gNB in some scenarios, so Uu cell is preferred. Papers in [4] and [5] also prefer to specify clearly it is Uu cell ID used in handover.

In the meanwhile, contribution [6] thinks there’s no extra stage 3 impact foreseen for this issue, and [7] suggests to use mapped cell ID during HO. The moderator notes that, the proposal of [7] relates to a quasi-earth fixed cell scenario, and the aim is to let UE consider itself to be in the same cell all the time.

In general, most companies ([1]-[6]) prefer to specify clearly it is Uu cell used in handover signaling.

Q1: Please share your view on the following proposal:

- Specify clearly it is Uu cell ID used in handover signaling for NTN.

| Company | Yes/No | Comment |
| --- | --- | --- |

## Issue 2: Signal multiple TACs for NTN cells at Xn setup and configuration update

Many contributions ([2], [3], [4] and [5]) observe both advantages and disadvantages to exchange multiple TACs via Xn interface. For example, if a single TAC is exchanged, it is difficult for source RAN node to judge whether it is suitable to handover UE to target cell. Also, RAN3 need to discuss which TAC should be chosen to be transferred via Xn. On the other hand, support of multiple TACs may lead to frequent NG-RAN node configuration update procedure, increasing signaling.

Contributions in [8], [9] think it is needed to support exchanging multiple TACs via Xn, while [6] has a different view.

Based on above, it seems no immediate agreements can be achieved regarding support multiple TACs, the moderator suggests to discuss the following issues:

Please share your view on the following question:

Q2.1 If only one TAC is transferred, which TAC should be transferred over Xn? And how to support the mobility restriction.

Q2.2 If multiple TACs are supported, how to support the dynamic change of TACs in earth-moving scenario?

| Company | Comment |
| --- | --- |

## Issue 3: Enhancement on time-based CHO

It has been agreed in the last meeting that:

Start time, duration are added in the signaling of time-based CHO.

The exchange of NTN Cell Coverage Stop Time between gNBs may be further discussed in future RAN3 meetings.

Enhancements for the support of CHO over NG for NTN-NTN hand-over should be discussed in this WI.

Time based CHO should be supported, the details are FFS

First of all, contribution [3], [4] and [7] all think there is no need to exchange the cell coverage stop time in the signaling of time-based CHO, because when the source node configure T1+T2 for time-based CHO, it can make sure T1+T2 will not exceed serving cell coverage stop time. [2] thinks the cell coverage stop time shall be exchanged, but the moderator notes that, the proposal is to exchange the Coverage Stop time via non-UE associated signaling, i.e. Xn setup and Configuration update procedure, which is related to another issue.

Therefore, it seems a consensus that there is no need to exchange the cell coverage stop time in the signaling of time-based CHO.

Q3.1: Please share your view on following proposals

- There is no need to exchange the cell coverage stop time in the signaling of time-based CHO.

| Company | Comment |
| --- | --- |

In [3] and [10], XnAP CRs have been provided to capture the agreements of time-based CHO. Although there are some tiny differences in the two CRs, e.g. IE structure and IE description, both contributions are working in the same direction and are similar. In the first round, the moderator would like to check whether the changes provided in the two sets of CRs are acceptable, and what type of enhancements are needed, if any.

Q3.2 Do you agree with the changes provided in [3] and/or [10]? Is there any enhancements needed?

| Company | Yes/No | Comment |
| --- | --- | --- |

In [7] and [8], it is proposed to support NG-based CHO. In addition, a CR [11] capturing the changes for supporting NG-based CHO is provided.

However, in [2] and [3], concerns are raised for supporting NG-based CHO. It seems the basic CHO functions are not supported in NG yet. To support NG-based CHO, a lot of changes are needed and may increase signaling consumption and problems, which includes at least the following aspects:

- Handover Preparation procedure needs to be enhanced to enable indicating conditional HO (and the start time/duration), and to enable replacing prepared CHO;

- A new NGAP procedure needs to be introduced to enable indicating handover success;

- A new NGAP procedure may need to be enhanced to enable CHO cancellation from the target side;

- A new NGAP procedure may need to be added to facilitate early data forwarding.

- Data forwarding

It is also mentioned in [2] and [8] that, SA2 should be involved if there is a need to support NG-based CHO.

Q3.3 Do companies acknowledge RAN3 should analyze the above mentioned impacts before supporting NG-based CHO, and SA2 is involved?

| Company | Yes/No | Comment |
| --- | --- | --- |

## Issue 4: Other mobility issues

In [8], it is proposed to introduce group handover over both Xn and NG interface to simplify NTN handover. The contribution in [7] also proposes to support group handover. However, the moderator notes that, in both contributions, the details of how group handover work are not given. The moderator would like to ask the following:

Q4. Do companies agree to support group handover over NG and Xn?

| Company | Yes/No | Comment |
| --- | --- | --- |

## Issue 5: Enhancements to feeder link switch over

In [9], it is proposed to support feeder link switch over enhancement based on Xn/NG signaling, and cell mapping information and NTN cell coverage stop time can be exchanged between gNBs for achieving the aim. In [8], it also propose to exchange cell coverage stop time via non-UE associated signaling to better support FLSW. [8] Also propose to exchange a Hard or Soft Feeder link Switch over indication via XN Setup procedure and Config Update procedure between the neighbouring gNBs.

Contribution [6] propose to define a new non-UE Xn procedure for feeder link switch, to exchange the necessary info between the gNBs, at least including satellite information and corresponding serving cell(s) information to be generated by the target gNB.

However, [7] thinks there are no strong motivations found to support decentralized coordination deployment.

Based on the above, the moderator would like to check companies’ views on the following questions.

Q5.1. Do companies agree it is needed to exchange cell coverage stop time via XN Setup procedure and Configuration Update procedure? How about also adding a ‘Hard or Soft Feeder link Switch over indication’?

| Company | Yes/No | Comment |
| --- | --- | --- |

Q5.2. Do companies agree to define a new non-UE Xn procedure for feeder link switch, to exchange the necessary info between the gNBs, at least including satellite information and corresponding serving cell(s) information to be generated by the target gNB?

| Company | Yes/No | Comment |
| --- | --- | --- |

## Issue 6: Others

In [12], a new proposal is given to study methods to enable an Xn interface over Uu (e.g., via service/feeder-links) and exchange a LS as necessary with RAN2.

Q6. Do companies agree with the above proposal to study methods to enable Xn interface over Uu?

| Company | Yes/No | Comment |
| --- | --- | --- |

# Phase 2 Discussion

# Conclusion, Recommendations

# References

- R3-225396, Cell Identifier for NTN handover (Qualcomm India Pvt Ltd)

- R3-225459, Discussion on the Mobility and Service Continuity Enhancements in NR NTN (Nokia, Nokia Shanghai Bell)

- R3-225692, Further discussion on mobility of NTN (Huawei)

- R3-225611, Further discussion on mobility issue for NR NTN (ZTE)

- R3-225711, Discussion on service continuity enhancement (Samsung)

- R3-225622, Discussion on Mobility enhancement for NR NTN (CATT)

- R3-225800, Discussion on mobility enhancement of NTN (CMCC)

- R3-225397, NTN Mobility and Feeder Link Switch Over Enhancements (Qualcomm India Pvt Ltd)

- R3-225427, Discussion on NTN mobility enhancements (China Telecommunication)

- R3-225580, CHO for NTN - XnAP Impacts (Ericsson, Thales, ZTE, Omnispace, TTP, Nokia, Nokia Shanghai Bell, CATT)

- R3-225579, CHO for NTN - NGAP impacts (Ericsson, Thales, ZTE, Omnispace, TTP, CATT)

- R3-225499, Xn interface enhancements in NTN (InterDigital )