3GPP TSG-RAN WG3 Meeting #118 	R3-226200

# Toulouse, France, EU, 14. - 18.11.2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1722 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional indicator for CHO-CPC coordination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At RAN3 #117, a solution was enabled that allows the MN and the SN to coordinate possible PSCell changes with ongoing CHO. Part of this solution concerns making the Data Forwarding Address Indication a mandatory step (it was optional and used only when the MN expected early data forwarding). This works, but two possible issues were identified: - It is unclear if the MN expects early data forwarding or not; - It is unclear how to handle the TEIDs if there are no SN-terminated bearers. To address that, a new code-point is proposed that enables separating the existing functionality from the new one and the handling of the TEIDs is clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CHO DC Indicator IE is extended so that the existing value can be handled in the same way as it was before. For coordination purposes only, a new code-point is added. The semantics are added to the list of PDU sessions to indicate the list shall be ignored if the CHO-DC Indicator IE is included and there are no SN-terminated bearers. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change concerns only one IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of the procedure may be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.15.2, 9.1.2.43, ASN.1 (9.3.5) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change, ommited text not changed

#### 8.3.15.2	Successful Operation

Figure 8.3.15.2-1: Data Forwarding Address Indication, successful operation

Figure 8.3.15.2-2: Data Forwarding Address Indication for Conditional Handover, successful operation

The new eNB initiates the procedure by sending a DATA FORWARDING ADDRESS INDICATION message to the old eNB.

For each E-RAB included in E-RABs Data Forwarding Address List IE, the new eNB indicates that it requests data forwarding of downlink packets to the GTP TEID indicated in the DL GTP Tunnel Endpoint IE.

If the DATA FORWARDING ADDRESS INDICATION message includes the CHO DC Indicator IE set to 'true', the SeNB (respectively, the en-gNB for EN-DC) shall, if supported, consider that the DATA FORWARDING ADDRESS INDICATION message concerns a Conditional Handover, and act as specified in TS 36.300 [15] for dual connectivity (respectively, act as specified in TS 37.340 [32] for EN-DC). If the CHO DC Indicator IE is set to 'true-coordination-only', the en-gNB shall, if supported, consider the that the XN-U ADDRESS INDICATION message concerns coordination of a triggered Conditional Handover with possible RRC reconfiguration with sync, and act as specified in TS 37.340 [32].

If the DATA FORWARDING ADDRESS INDICATION message includes the CHO DC Early Data Forwarding Indicator IE set to "stop", the SeNB (respectively, the en-gNB for EN-DC) shall, if supported and if already initiated, stop early data forwarding for the provided E-RABs Data Forwarding Address information.

If the DATA FORWARDING ADDRESS INDICATION message includes the CPC Data Forwarding Indicator IE set to “triggered”, the en-gNB for EN-DC shall, if supported, consider that the DATA FORWARDING ADDRESS INDICATION message concerns a Conditional PSCell Change, and act as specified in TS 37.340 [32]. If the CPC Data Forwarding Indicator IE is set to "early data transmission stop", the en-gNB shall, if supported and if already initiated, stop early data forwarding for the provided Data Forwarding Address information.

EN-DC

If the MeNB sends the message to the en-gNB, then the SgNB UE X2AP ID IE shall be included in the DATA FORWARDING ADDRESS INDICATION message, while the New eNB UE X2AP ID IE is ignored. The SgNB UE X2AP ID IE is used as the new UE ID.

Next change, ommited text not changed

#### 9.1.2.43	DATA FORWARDING ADDRESS INDICATION

This message is sent by the new eNB to indicate to the old eNB forwarding addresses for each E-RAB for which it admits data forwarding.

During a Conditional Handover with EN-DC or Dual Connectivity or Conditional PSCell Change, this message is also used to provide data forwarding related information. In case of EN-DC, the data forwarding related information is transferred from the eNB to the en-gNB, while in case of Dual Connectivity, the data forwarding related information is transferred from the MeNB to the SeNB.

Direction: new eNB  old eNB.

Direction: MeNB  en-gNB (Conditional Handover with EN-DC, Conditional PSCell Change), MeNB  SeNB (Conditional Handover with Dual Connectivity)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the new eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the new eNB | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the old eNB | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the old eNB | YES | ignore |
| E-RABs Data Forwarding Address List |  | 1 |  | This IE is ignored if the CHO DC Indicator IE is included and there are no SN-terminated bearers allocated. | YES | ignore |
| > E-RABs Data Forwarding Address Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| CHO DC Indicator | O |  | ENUMERATED (true, ..., true-coordination-only) | Indicating that the DATA FORWARDING ADDRESS INDICATION message is for a Conditional Handover. | YES | reject |
| CHO DC Early Data Forwarding Indicator | O |  | ENUMERATED (stop, ...) |  | YES | ignore |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated for EN-DC at the en-gNB. | YES | ignore |
| CPC Data Forwarding Indicator | O |  | ENUMERATED (triggered, early data transmission stop, ...) | Indicates that the DATA FORWARDING ADDRESS INDICATION message is for a Conditional PSCell Change. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

Next change, ommited text not changed

CHO-DC-Indicator ::= ENUMERATED {true, ..., true-coordination-only}

Remaining text not changed