3GPP TSG-RAN WG3 #118	R3-226364

Toulouse, France, 14th – 18th November 2022

Source:	CATT

Title:	Discussion on XnAP impacts of non-UE-associated metrics

Agenda Item:	12.2.2.1

Document for:	Discussion and decision

- Introduction

This discussion paper focuses impact on XnAP of AI/ML of non-UE-associated metrics IEs.

- Discussion

  - Container-based metric IE design

Although we have agreed that DC and disaggregated scenario are considered after there are some agreements on basic aggregated scenario, we should finally support all of these scenarios. Therefore we propose discussing whether we should introduce “container-based metric IE design” so as to minimise specification workload especially when specifying DC and disaggregated scenarios (as well as later enhancements).

Following is a rough summary on what measurement metric IE for AI/ML may be introduced for what RAN3 interface (both Non-UE-associated and UE-associated IEs are included here for convenience):

| New metric IE / IE group | XnAP non-UE or HO | XnAP SN to MN | XnAP MN to SN | E1AP UP to CP | F1AP DU to CU | F1AP CU to DU |
| --- | --- | --- | --- | --- | --- | --- |
| [Non-UE] Current own energy efficiency (traffic and/or energy consumption) and energy state | Yes | - | - | Yes | Yes | - |
| [Non-UE] Predicted own energy efficiency (traffic and/or energy consumption) | Yes | - | - | - | - | - |
| [Non-UE] Other non-UE performance KPIs | Yes | - | - | - | - | - |
| [Non-UE] Predicted own resource status | Yes | - | - | - | - | - |
| [UE-associated] Current UE traffic | Yes | Yes | Maybe | Yes | Maybe | - |
| [UE-associated] Predicted UE traffic | Yes | Maybe | Maybe | - | - | Maybe |
| [UE-associated] Predicted UE location (trajectory) | Yes | - | Maybe | - | - | Maybe |
| [UE-associated] UE location information of HO-ed UE as feedback | Yes | Maybe | - | - | Maybe | - |
| [UE-associated] Other UE performance metrics, e.g. bitrate, packet loss, packet delay | Yes | Yes | - | Maybe | - | - |

Although the tabular is only an initial though and may deviate quite much from the final agreements, one thing is clear: it is quite common that one metric IE needs to be introduced into multiple RAN interfaces.

Considering that RAN AI/ML is still premature now and thus it is likely that the IEs introduced in Rel-18, especially predictions, will face future enhancement (e.g. when one find that some enhancement can improve the AI/ML output very much), it may be a suitable way that those IEs are defined as containers within XnAP (where they are all necessary), and other specs quote these container directly without copying their definition.

Such design may provide an additional benefit that these containers can be delivered in a transparent manner so the version of the node in middle does not matter.

Such design will be named “container-based metric IE design” for convenience.

Proposal 1: Consider using “container-based metric IE design” when adding new IEs into RAN3 specs, i.e. defining new metrics (either statistical or analytical) as containers in XnAP, and other specs quote these containers directly without copying their definition.

Note that UE AP IDs should not be included with the container, since it is different among interfaces and thus cannot be delivered transparently. Cell ID, on the contrary, can be included.

Following we will discuss these non-UE-associated metrics one by one, focusing their impact on XnAP.

  - Current own energy efficiency

In RAN3#117-e meeting we agreed to introduce metrics on energy efficiency, but the detail was left as FFS. Last meeting we discussed them for another turn but failed too.

Energy efficiency for NG-RAN is defined in Section 6.1.1 of TS 28.310 as EEMN,DV=DVMNECMN, where DVMN is the data volume (i.e. traffic) and ECMN is the energy consumption. As implied in TR 37.817, AI/ML’s goal is to optimise the overall energy efficiency of the coverage of a gNB and its neighbours combined, i.e. a sum of DVMN divided by a sum of ECMN:

The current energy-saving schemes are vulnerable to potential issues listed as follows: - (other bullets omitted.) - Actions that may produce a local (e.g., limited to a single RAN node) improvement of Energy Efficiency, while producing an overall (e.g., involving multiple RAN nodes) deterioration of Energy Efficiency.

Mathematically, one cannot get the overall DV/EC quotient by operating individual DV/EC quotients, e.g.:

- Case A: Node1: DV=200, EC=20, DV/EC=10; Node2: DV=80, EC=80, DV/EC=1; overall DV/EC=2.8.

- Case B: Node1: DV=800, EC=80, DV/EC=10; Node2: DV=20, EC=20, DV/EC=1; overall DV/EC=8.2.

- Although Node3 receives DV/EC=10 from Node1 and DV/EC=1 form Node2, it is still not sure whether the overall DV/EC is 2.8 or 8.2.

Therefore what should be delivered over RAN interfaces should be the data volume and the energy consumption separately, rather than the quotient. Another approach is to deliver only the EC, but we think the total DV can still be helpful to deduce whether a rise in total EC is caused by a rise of total DV or wrong action, i.e. the overall DV/EC metric is better than the total EC metric to be used as an input for calculating the “reward”.

It was ever proposed that we can introduce only an abstract percentage, but we don’t consider it a good idea as delivering a percentage is not much less costly than delivering both the data volume and the energy consumption, whereas may incur interoperability problem. And percentage cannot be added either.

Proposal 2: In order to get the total energy efficiency of a node and its neighbour combined, the data volume (i.e. traffic) and the energy consumption should be delivered separately over RAN interfaces.

However there is not any necessity to get the precise per-QoS-level or per-slice data volume as defined in TS 28.552. And for the case of split gNB, it is actually impossible to get any per-F1-U/Xn-U data volume as F1-U/Xn-U tunnels are allocated per DRB and the gNB-CU-UP is never told whether a 38.425 tunnel is an F1-U one or an Xn-U one. A simple per-node “total DL data volume” and “total UL data volume” may be the most practical solution. The error caused by Xn-U tunnels is negligible as our goal is to optimise the energy efficiency of many RAN nodes combined rather than one node.

Proposal 3: The data volume provided over RAN interfaces should be the per-node “total DL data volume” and “total UL data volume” of the providing node, which is the most practical solution.

For the energy consumption metric ECMN, the most natural though to follow the “Active energy consumption over a period of time” defined in Annex A.1 of ETSI ES 202 336-12, which seems to be a per-node metric. It was ever proposed that delivering a per-cell metric would be much more helpful, but that may not be feasible as pointed out by some companies. For safety we still propose defining it as a per-node metric (and this costs fewer bits for sure).

Proposal 4: The energy consumption provided over RAN interfaces should be a per-node metric of the providing node.

  - Predicted own energy efficiency

For predicted own energy efficiency, we think the predicted data volume and the predicted energy consumption of the providing node should be provided separately likewise.

Proposal 5: It should be supported for a node to deliver both its current and its predicted energy efficiency metrics (i.e. data volume + energy consumption) over RAN interfaces.

For the case of neighbour energy efficiency, we agreed in RAN3#117 meeting that:

Predicted resource status information of neighbouring NG-RAN node(s) generated by the current NG-RAN node is internally used, and no standard impacts.

We think the prediction on neighbour energy efficiency should be used internally as well, i.e. there is no impact on specifications.

  - Accuracy and confidence

In our understanding every quantised prediction (e.g. energy consumption, UE location expressed in geographical coordinates) can be accompanied with an accuracy IE, whereas every non-quantised prediction (e.g. UE location expressed as CGI) can be accompanied with a confidence IE. For the former type, including also the confidence may also be beneficial, which can form a probability distribution along with the accuracy, but we do not think it necessary to consider at this release.

Proposal 6: Every quantised prediction (e.g. energy consumption, UE location expressed in geographical coordinates) can be accompanied with an accuracy IE, whereas every non-quantised prediction (e.g. UE location expressed as CGI) can be accompanied with a confidence IE.

  - Validity time

Last meeting we discussed how to express the validity time but no agreement was achieved. In TS 23.288 the validity time is always expressed in a form of period (absolute time + duration), either indicated in the request message (for what period the prediction is requested, namely “Analytics target period”) or provided along the prediction (namely “Analytics target period subset”). We may follow this in RAN3 specs.

Proposal 7: The method to indicate validity time used in TS 23.288 can be a baseline of how to indicate it in RAN specs.

- Conclusion

Proposal 1: Consider using “container-based metric IE design” when adding new IEs into RAN3 specs, i.e. defining new metrics (either statistical or analytical) as containers in XnAP, and other specs quote these containers directly without copying their definition.

Proposal 2: In order to get the total energy efficiency of a node and its neighbour combined, the data volume (i.e. traffic) and the energy consumption should be delivered separately over RAN interfaces.

Proposal 3: The data volume provided over RAN interfaces should be the per-node “total DL data volume” and “total UL data volume” of the providing node, which is the most practical solution.

Proposal 4: The energy consumption provided over RAN interfaces should be a per-node metric of the providing node.

Proposal 5: It should be supported for a node to deliver both its current and its predicted energy efficiency metrics (i.e. data volume + energy consumption) over RAN interfaces.

Proposal 6: Every quantised prediction (e.g. energy consumption, UE location expressed in geographical coordinates) can be accompanied with an accuracy IE, whereas every non-quantised prediction (e.g. UE location expressed as CGI) can be accompanied with a confidence IE.

Proposal 7: The method to indicate validity time used in TS 23.288 can be a baseline of how to indicate it in RAN specs.

Based on the proposal, we draft 2 Stage 3 TPs [1][2].

- Reference

[1] R3-226367; TP on TS 38.423 for AI/ML.

[2] R3-226366; TP on TS 38.420 for AI/ML.