3GPP TSG-RAN WG3 Meeting #118	R3-226507

14 November – 18 November 2022

Toulouse, France

Title: 	Open points on validity time and prediction accuracy

Source: 	Ericsson

Agenda item:	12.2.2.1

Document Type:	Discussion and Decision

# Introduction

In the previous meeting, the following FFS that were common for all use cases were captured in the meeting minutes:

It is FFS whether a node requesting a prediction includes timing information in order to indicate for which time a prediction is requested.

Whether there is a need for prediction accuracy at a receiving node is FFS.

In the following, we discuss these open points and put forward our proposals.

# Validity time

During the RAN3#117-e meeting, RAN3 agreed on two types of predictions to be signaled over Xn: i) predicted resource status information and ii) predicted UE trajectory (on a per cell-granularity). Furthermore, in the last RAN3#117-bis-e meeting, the following agreement was captured:

How to indicate validity time (e.g., implicitly with a new prediction when the previous prediction becomes invalid, explicitly with every prediction in the AI/ML output or by the request to the prediction) shall be discussed on a case by case basis.

Therefore, RAN3 agreed that validity time should be independently analyzed for each agreed signaled prediction. In the case of predicted UE trajectory, RAN3 has already agreed during the RAN3#117-bis-e meeting on how validity time is signaled:

Cell-based UE Trajectory prediction is provided as a list of cells into the future, each of which is indicated together with an expected time of stay into the cell.

This agreement is in line with the nature of the UE trajectory prediction, which RAN3 agreed to signal during handover using the existing procedures. This prediction is signaled proactively (i.e., without a request for the specific handover) since the target RAN node does not know when a UE from another RAN node is moving towards its coverage. The target RAN node of a handover cannot control the movements of the UE either; hence, the expected time of stay in the cell (i.e., the validity time of the prediction) needs to be signaled by the source RAN node.

Furthermore, the UE trajectory prediction is an assistance information that the target RAN node may choose to ignore; present-day systems carry out successful handovers without this type of assistance information.

Observation 1:  UE trajectory prediction is an assistance information not requested by the receiving node which can be safely ignored; thus, signaling its validity time, in the form of predicted time the UE stays in each cell, is a valid solution.

Predicted resource status information is a different type of assistance information. First, we note that this information is always sent by request; it would be a waste of resources to signal this prediction to every neighboring RAN node constantly. Second, this information is requested to fulfil a specific purpose; if a RAN node needs the predicted average number of active UEs (one of the agreed elements for predicted resource status information) during the next 60 seconds, it is not useful that it receives the average for the next 10 seconds, even if this validity time is signaled with the prediction. Therefore, a RAN node would always state the validity time of the predicted resource status information in the request for this information.

Observation 2:  Predicted resource status information is an assistance information that is requested for a specific purpose; thus, the requesting node needs to specify its validity time in the request.

Proposal 1:  The validity time of requested AI/ML assistance information needs to be signaled in the request.

One obvious way to signal the validity time as part of an AI/ML information request, without adding any extra information, is to deduce the validity time from the reporting period in case predictions are reported periodically.

Proposal 2:  For periodic reporting the validity time of requested AI/ML assistance information can be deduced from the reporting period.

When the validity time is signaled in the request message for the predicted resource status information, both RAN nodes are aware of the validity time of the predictions. Therefore, there is no need to signal the validity time of the predictions jointly with the predictions.

Observation 3:  If the validity time is present in the request for predicted information, there is no need to signal it with the predictions themselves.

Proposal 3:  The validity time of requested AI/ML assistance information should not be signaled jointly with the prediction information.

Regarding the reporting of AI/ML assistance information like the predicted resource status information, RAN3 agreed:

Reporting options for the new procedure used for AI/ML Related Information to be evaluated on a case-by-case basis. Possible reporting options are one-time and periodic reporting.

In the case of periodic reporting, it has been explained that the validity time of the predictions can be linked to the reporting periodicity. On the other hand, the request for a one-time reporting needs to explicitly state the validity time of the requested prediction.

Proposal 4:  For one-time reporting of AI/ML assistance information, the validity time should be explicitly signaled in the request.

# Prediction accuracy

In machine learning, the term “accuracy” normally refers to “classification accuracy” and it is equal to the ratio of correct predictions over the total number of predictions made (usually using a testing dataset). This terminology has several disadvantages for the work in 3GPP, as we explain below.

First, it is only useful for classification problems, e.g., determining the next cell in UE trajectory prediction, but it cannot be applied to regression problems, e.g., predict the average number of active UEs during the next 60 seconds. This is because accuracy only measures errors in a binary way (i.e., error or no error) which is useful for classification problems. In regression problems, we are interested in a more fine-grained error measure that describes how close the prediction is to the actual value; for example, a prediction of 20 active UEs when the true value is 22 is better than a prediction of 5 active UEs.

Using “accuracy” as an umbrella term for “performance metrics” of AI/ML algorithms is deceiving, since there are many possible metrics. To name a few, for classification problems, we have accuracy, precision, recall, and F1-score, while for regression problems, we have (root) mean square error and mean absolute error.

Observation 4:  There are multiple different performance metrics for AI/ML algorithms and standardizing them would result in a complex reporting procedure.

Second, the performance metrics described above are computed using a testing dataset after the AI/ML algorithm is trained. Even with strict data management policies, where the available dataset is properly partitioned into a training and a testing dataset early in the development of an AI/ML algorithm, it is not uncommon that there are certain similarities between the training and testing datasets coming from the same source. For example, assume two AI/ML algorithms; algorithm A is trained with data coming from operator A while algorithm B is trained using data from operator B. Both algorithms might achieve good testing performances (e.g., good accuracy if it were a classification problem) in their respective testing datasets, but they could perform much worse if they are tested with the other operator’s data.

With that, it should be clear that the prediction “accuracy” that may be provided by an AI/ML algorithm together with its prediction outputs may be in itself inaccurate and therefore difficult to use in a multivendor environment. Its usage might lead to errors, e.g., where a receiving node uses the accuracy metric to take important decisions, while the accuracy provided does not reflect the real error the prediction may be affected by.

Observation 5:  The reported performance metrics (e.g., accuracy) of AI/ML algorithms depend on the particular testing datasets used to compute them.

Finally, the aforementioned performance metrics are constant for each particular AI/ML model as long as the model is not modified in any way. This is because, in order for the performance metric, e.g., accuracy, to be reliable, it needs to be calculated on the basis of a large amount of training data. Assuming that there was some value in signaling performance metrics, there is thus no benefit in signaling the performance with each reported prediction as some companies proposed during the last meeting. A more fined-grained performance metric, one which is not unique for the whole model, would be more inaccurate due to the reduced number of samples used to compute it. This could be achieved, for example, by partitioning the input space in different regions and computing the performance metric for each region. In the extreme, one could train a different AI/ML model to estimate the performance of each individual input; this would be a very inaccurate and not useful information.

Observation 6:  The performance metrics are only useful when computed with many samples, and thus they are valid for the whole AI/ML model. Therefore, there is no need to signal them with each different model output.

Proposal 5:  The accuracy of predictions should not be signaled with the predictions.

A more robust solution is that each RAN node employing predictions from other sources should themselves compute the performance of the predictions. This can be achieved by subscribing to the data sources that produce the true values of the predicted information and comparing them a posteriori. The advantage of this approach is that the computed performance metrics are done with the real data the network encounters.

Proposal 6:  The verification of the performance of AI/ML models is out of RAN3 scope.

# Conclusion

In this contribution, we discussed the open issues regarding validity time and prediction accuracy, and we made the following observations and proposals:

Observation 1:  UE trajectory prediction is an assistance information not requested by the receiving node which can be safely ignored; thus, signaling its validity time, in the form of predicted time the UE stays in each cell, is a valid solution.

Observation 2:  Predicted resource status information is an assistance information that is requested for a specific purpose; thus, the requesting node needs to specify its validity time in the request.

Proposal 1:  The validity time of requested AI/ML assistance information needs to be signaled in the request.

Proposal 2:  For periodic reporting the validity time of requested AI/ML assistance information can be deduced from the reporting period.

Observation 3:  If the validity time is present in the request for predicted information, there is no need to signal it with the predictions themselves.

Proposal 3:  The validity time of requested AI/ML assistance information should not be signaled jointly with the prediction information.

Proposal 4:  For one-time reporting of AI/ML assistance information, the validity time should be explicitly signaled in the request.

Observation 4:  There are multiple different performance metrics for AI/ML algorithms and standardizing them would result in a complex reporting procedure.

Observation 5:  The reported performance metrics (e.g., accuracy) of AI/ML algorithms depend on the particular testing datasets used to compute them.

Observation 6:  The performance metrics are only useful when computed with many samples, and thus they are valid for the whole AI/ML model. Therefore, there is no need to signal them with each different model output.

Proposal 5:  The accuracy of predictions should not be signaled with the predictions.

Proposal 6:  The verification of the performance of AI/ML models is out of RAN3 scope.