3GPP TSG-RAN WG3 Meeting #119	R3-230598

# Athens, Greece, 27 Feb – 3 Mar 2023

Agenda item:		12.2.2.2

Source:	ZTE

Title:	Further discussion on procedure used for feedback and mobility optimization

Document for:		Others

Introduction

In the last meeting, with respect to the feedback information, the discussion was divided to single UE and group UE. And the controversial part is how to request/response the feedback information. Following are the open issues left:

There seems to be agreement that reporting of AI/ML feedback is sent in a new class 2 procedure, but this agreement can be finalized when the stage 2 discussion finalized in the future. Whether this is the same class 2 message as already agreed for Data Reporting of AI/ML Related Information is FFS.

It is FFS the feedback is triggered via the handover request, or via a new class 1 procedure (same or different from the previously agreed for initiating the reporting of AI/ML Related Information).

In this contribution, we elaborate our analysis on the open issues above related to feedback information for AI/ML in the RAN, and provide the corresponding TP thereof.

Discussion

Feedback Procedure

In the last meeting, it was discussed how does source gNB get the UE performance feedback e.g. UL/DL throughput, packet delay, packet error rate? And there seems following two options on the table.

- Option 1: Using the Handover Preparation procedure to request and response feedback information, and then using the new class 2 procedure to report the feedback information.

- Option 2: Using the new procedure for AI/ML related information to request and response feedback information and then using the new class 2 procedure to report the feedback information.

The same part between option1 and option2 is to use the new class2 procedure to report the feedback information. However, the controversial issue here is whether the class 2 procedure is UE-associated procedure or non-UE associated procedure. As we know, based on the current TR37.817, the feedback information across three use cases, e.g., Energy Saving, Load Balancing, Energy Saving, involves non-UE related information and UE related information as follows:

- UE related information: UE performance, UE Performance (e.g., UL/DL throughput, packet delay, packet error rate, etc.)

- Non-UE related information: Energy Efficiency, Resource Status

- The feedback information involves non-UE related information and UE related information.

Based on the current agreement, the resource status information can be transferred through existing resource status procedure between NG-RAN nodes. However, no procedure now is used to transfer energy efficiency of NG-RAN node after the UE(s) handover. For AI/ML based energy saving and AI/ML based load balancing, a group of UE will be offloaded to the target NG-RAN node based on the AI/ML based decision. The source NG-RAN nodes should know the performance of the target NG-RAN node is good or not over a period of time after handover. The NG-RAN node needs know the performance of UE-associated and non-UE associated. In the previous meeting, it was agreed that

The new procedure is non-UE associated procedure. If needed, the procedure can be used to capture UE-associated information.

Hence, to transfer the feedback information, including non-UE related information and UE related information, the new class 2 procedure used to report feedback information should be non-UE associated procedure.

- The new class 2 procedure used to report feedback information should be non-UE associated procedure.

Regarding how to request the feedback information, option 1 is to enhance the existing procedure, which handover request message may include an indication as similar as the Report Characteristics IE to indicate the requested feedback information, and then the target NG-RAN node responds with handover request acknowledge message, while option 2 is to use the new procedure to request and response feedback information.

As mentioned above, the feedback information involve non-UE related information and UE related information. Since the handover request message is UE-associated signaling, if the source NG-RAN node needs to collect non-UE related information (e.g., energy efficiency), it should also add non-UE associated feedback request indication in the handover request message. It leads to the issue that the target NG-RAN node may response the same non-UE associated feedback information to the source NG-RAN node in the individual report message based on the handover request message. Take as energy efficiency feedback information as an example:

Figure 1: Using Handover Preparation Procedure to request feedback information

It can explicitly seen that the energy efficiency is repeatedly reported by UE1’s class2 reporting procedure and UE2’s class2 reporting procedure. With the respect to the non-UE related feedback information, namely the performance of the target NG-RAN node, it is similar over a period of time after a group of UE offload to the target NG-RAN node. If multiple UEs handover to the target NG-RAN node based on the AI/ML based decision, it will cause the redundancy of reporting.

- Using the Handover Request message to request the feedback information leads to the redundancy of reporting.

In addition, handover procedure is used to UE handover preparation, which is high priority. Basically, the feedback information request/response is used to configure the requested measurement as similar as resource status procedure. With respect to the resource status procedure, if the requested information can not be satisfied in the target NG-RAN node, it sends the Failure message with an appropriate cause value. Now back to the handover procedure, if the requested feedback information can not be satisfied, the target NG-RAN node may respond a failure indication in the acknowledge message rather than send the handover failure message, which leads to the issue that feedback information needs to use other procedure to request and response, otherwise, the source NG-RAN node can not achieve the feedback information.

- In the case that handover preparation is successful but feedback request in the handover preparation is failed, the other procedure used for feedback information needs to be triggered or the source NG-RAN node cannot achieve the feedback information.

Above all, using the existing handover procedure to request feedback information will cause some issues which can be avoided by using the new AI/ML related information procedure.

In the last meeting, it was agreed that the new procedure is used for AI/ML related information. And this procedure can also be used for requesting feedback information. Using new procedure can also separate the handover function and AI/ML function. Feedback information is not limited to what is agreed now. For the follow-up use cases, more feedback information will need to be carried between NG-RAN nodes. Hence, new procedure is much flexible for future extension.

- Introduce AI/ML function cannot cause negative impacts on the Handover Procedure. New procedure is much flexible for future extension.

In addition, the timing of the two procedure of class1 and class2 is not so close. Feedback is not reported until a long time after the handover occurs. Hence, there is no relationship between the class2 procedure to report feedback information and handover procedure.

- No relationship between the class2 procedure to report feedback information and handover procedure.

- Using new agreed procedure to request and response the AI/ML related feedback information.

In the last meeting, it was agreed that WA: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”. However, the feedback information will be triggered to report after the handover occurs, so the feedback information is different from predicted information. We propose to introduce the Usage Request IE in the request message, to indicate the type of usage for which the AI/ML related information is required.

- Introduce the Usage Request IE in the request message, to indicate the type of usage for which the AI/ML related information is required.

UE performance

It was agreed that UE performance may include following information:

Support the following UE performance information to be sent for feedback purposes: Average Packet Delay, Average UE Throughput DL, Average UE Throughput UL, Average Packet Error Rate.

Hence, the UE performance IE in the report message should include following:

- 9.2.3.Y	UE Performance

This IE indicates UE performance measurements.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  |  |  |
| Average UE Throughput UL | O |  |  |  |
| Average Packet Delay | O |  |  |  |

- Introduce UE performance IE in the report message involves Average UE Throughput DL IE,  Average UE Throughput UL IE, and Average Packet Delay IE.

Besides, the timing information of the feedback information should be included in the request message, as describe in our companion contribution. The feedback information may be collected by the target NG-RAN node after handover over a period time. The source NG-RAN node may know the handover execution time of the UE, so NG-RAN node can provide the timing information, e.g., start time or duration or end time, to the target NG-RAN node to collect and measure feedback information within a specified period of time.

- The AI/ML related information used for feedback information requires the timing information in the request message.

Acutal UE trajectory information

Regarding the feedback information, following is the other issue in the previous meeting that there is no consensus to add UE trajectory feedback at this time. It is not clear for some companies why the actual trajectory is feedback information. After UE handover to the target NG-RAN node, the actual UE trajectory may be cell hops. The source NG-RAN node may predict the cell hops in the target NG-RAN node over a period time, so the target NG-RAN node can provide these kind of actual trajectory to the source NG-RAN node. According to the actual trajectory, the source RAN node can monitor its model performance, evaluate whether the model needs update.

- Providing the actual UE trajectory over a period of time is beneficial for the source NG-RAN node to monitor its model performance.

Company put forward that NG-RAN node can make use of UE history information to derive feedback for UE trajectory. From technical perspective, the issue can not be resolved by UE coming back to the NG-RAN node who generates the predicted UE trajectory. We can not ensure that the same UE will connected to the original NG-RAN node again.

- Using HUI to derive feedback for UE trajectory prediction is not feasible.

- Introduce the actual UE trajectory in the class2 feedback reporting message, and introduce the actual UE trajectory indication in the request message.

Predicted UE trajectory

Single UE – Transfer of UE Trajectory Prediction

UE Trajectory Prediction is transferred to the target gNB via the Handover Request.

In the last meeting, it was agreed that cell-based UE Trajectory Prediction is transferred via existing HO signalling messages, and it is not precluded that other way to transfer the predicted cell-based UE trajectory. The handover request message should include the list of predicted UE trajectory.

- The handover request message should include the list of predicted UE trajectory IE.

The proposals are reflected in our TP [1] to 38.423 and 38.420.

Conclusion

We propose the following observations:

- The feedback information involves non-UE related information and UE related information.

- Using the Handover Request message to request the feedback information leads to the redundancy of reporting.

- In the case that handover preparation is successful but feedback request in the handover preparation is failed, the other procedure used for feedback information needs to be triggered or the source NG-RAN node cannot achieve the feedback information.

- Introduce AI/ML function cannot cause negative impacts on the Handover Procedure. New procedure is much flexible for future extension. New procedure is much flexible for future extension.

- No relationship between the class2 procedure to report feedback information and handover procedure.

- Providing the actual UE trajectory over a period of time is beneficial for the source NG-RAN node to monitor its model performance.

We propose the following proposals:

- The new class 2 procedure used to report feedback information should be non-UE associated procedure.

- Using new agreed procedure to request and response the AI/ML related feedback information

- Introduce the Usage Request IE in the request message, to indicate the type of usage for which the AI/ML related information is required.

- Introduce UE performance IE in the report message involves Average UE Throughput DL IE,  Average UE Throughput UL IE, and Average Packet Delay IE.

- The AI/ML related information used for feedback information requires the timing information in the request message.

- Using HUI to derive feedback for UE trajectory prediction is not feasible.

- Introduce the actual UE trajectory in the class2 feedback reporting message, and introduce the actual UE trajectory indication in the request message.

- The handover request message should include the list of predicted UE trajectory IE.

- References

- R3-230599, (TP to 38.423 and 38.420) AIRAN impact on Xn Interface, ZTE

- Annex TP to 38.423

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Target NG-RAN node UE XnAP ID IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node shall remove the existing prepared conditional HO identified by the Target NG-RAN node UE XnAP ID IE and the Target Cell Global ID IE. It is up to the implementation of the target NG-RAN node when to remove the HO information.

**************** skip unchanged part *******************

If the Time Synchronisation Assistance Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7].

If the QMC Configuration Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.300 [9].

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the Predicted UE Trajectroy IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that is the predicted UE trajectory information, and may use it for subsequent mobility optimization.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

- 8.4.AA AI/ML Information Reporting Initiation

- 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

- 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to "start"; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	 shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If the Registration Request IE is set to "start" in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates UE specific AI/ML related information reporting, the UE To Report List IE shall be included.

If NG-RAN node2 is capable to provide all requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message.

If the Partial Reporting Indication IE is contained in the AI/ML INFORMATION REQUEST message, the NG-RAN node2 is allowed to provide all or part of requested information and shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and include the Report Characteristics IE in the AI/ML INFORMATION RESPONSE message if the Partial Reporting Indication IE is set to "allowed".

If the Reporting Timing Information IE is contained in the AI/ML INFORMATION REQUEST message, the NG-RAN node2 shall provide the requested information within the specified time.

If the Request Type IE is contained in the AI/ML INFORMATION REQUEST message, this indicates the request type of AI/ML related information. The NG-RAN node2 shall consider the information reported as feedback information and report the requested information after specific AI/ML related decision, e.g. handover.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the AI/ML INFORMATION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements on. For each cell, NG-RAN node2 shall include in the AI/ML INFORMATION REPORT message:

- 	the Predicted Radio Resource Status IE, if the first bit, "Predicted Resource Status Periodic " of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1". If NG-RAN node2 is a gNB and if the cell for which Predicted Radio Resource Status IE is requested to be reported supports more than one SSB, the Predicted Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall include the requested SSB Area Radio Resource Status List IE; If the cell for which Predicted Radio Resource Status IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the requested Slice Radio Resource Status Item IE;

- 	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs Periodic" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1";

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections Periodic" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

-    the Predicted Energy Efficiency  IE, if the forth bit, "Predicted Energy Efficiency Periodic" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

-    the Energy Efficiency IE, if the fifth bit, "Energy Efficiency Periodic" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

-    the UE performance IE, if the sixth bit, " UE Performance Periodic" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

-    the UE Trajectory IE, if the seventh bit, " UE Trajectory Periodic" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to "1".

- 8.4.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: AI/ML Information Reporting Initiation, unsuccessful operation

If all of the requested AI/ML related information reporting cannot be initiated, NG-RAN node2 shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value.

If the Partial Reporting Indication IE is set to "not allowed" and part of the requested AI/ML related information reporting cannot be initiated, NG-RAN node2 shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value.

- 8.4.AA.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the AI/ML INFOMRATION RESPONSE message or the AI/ML INFORMATION FAILURE message, the NG-RAN node1 may reinitiate the AI/ML Information Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new AI/ML INFOMRATION REQUEST message is identical to the content of the previously unacknowledged AI/ML INFOMRATION REQUEST message.

If the NG-RAN node2 receives a AI/ML INFOMRATION REQUEST message which includes the Registration Request IE set to "add" or "stop" and if the NG-RAN node2 Measurement ID value received in the AI/ML INFOMRATION REQUEST message is not used, the NG-RAN node2 shall initiate AI/ML INFOMRATION FAILURE message with an appropriate cause value.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the AI/ML INFOMRATION REQUEST message then NG-RAN node2 shall initiate a AI/ML INFOMRATION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a AI/ML INFOMRATION REQUEST message which includes the Registration Request IE set to "start" and the NG-RAN node1Measurement ID IE corresponding to an existing on-going AI/ML information reporting, then NG-RAN node2 shall initiate a AI/ML INFOMRATION FAILURE message with an appropriate cause value.

- 8.4.BB	AI/ML Information Reporting

- 8.4.BB.1	General

This procedure is initiated by an NG-RAN node to report AI/ML related information accepted by the NG-RAN node following a successful AI/ML Information Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be reported using this procedure.

Editor’s Note: FFS content of AL/ML related information.

- 8.4.BB.2	Successful Operation

Figure 8.4.11.2-1: AI/ML Information Reporting, successful operation

NG-RAN node2 shall report the accepted AI/ML related information in AI/ML INFORMATION UPDATE message. The accepted AI/ML related information is the information that was successfully initiated during the preceding AI/ML Information Reporting Initiation procedure.

- 8.4.BB.3	Unsuccessful Operation

Not applicable.

- 8.4.BB.4	Abnormal Conditions

Void

- 9.1	Message Functional Definition and Content

- 9.1.1	Messages for Basic Mobility Procedures

- 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Predicted UE Trajectory | O |  | 9.2.3.Z |  | Yes | Ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

- 9.1.3.CC	AI/ML INFORMATION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Request Type | O |  | ENUMERATED(feedback, …) | Indicate the request type of AI/ML related information. | Yes | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add…) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Resource Status Periodic, Second Bit = Predicted Number of Active UEs Periodic, Third Bit = Predicted RRC connections Periodic Forth Bit = Predicted Energy Efficiency Periodic Fifth Bit = Energy Efficiency Periodic Sixth Bit = UE Performance Periodic Seveth Bit = UE Trajectory Periodic If the Request Type is set to “feedback”, only fifth to seventh bit can be used. F | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| UE to Report List |  | 0..1 |  | UE ID list to which the request applies | YES | ignore |
| >UE To Report Item |  | 1 .. <maxnoofUEsinNG-RANnode> |  |  |  |  |
| >> UE Assistant Identifier |  |  | NG-RAN node XnAP ID 9.2.3.16 |  |  |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Partial Reporting Indication | O |  | ENUMERATED(allowed, not allowed) | This IE indicates whether the partial reporting is allowed. | YES | ignore |
| Request Timing Information | O |  | 9.2.X |  |  |  |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or “add”. |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEsinNG-RANnode | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

- 9.1.3.DD	AI/ML INFORMATION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information, for all or part of   the objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Successful Reporting Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is able to report. First Bit = Predicted Resource Status Periodic, Second Bit = Predicted Number of Active UEs Periodic, Third Bit = Predicted RRC connections Periodic Forth Bit = Predicted Energy Efficiency Periodic Fifth Bit = Energy Efficiency Periodic Sixth Bit = UE Performance Periodic Seveth Bit = UE Trajectory Periodic FFS on the coding | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

- 9.1.3.EE	AI/ML INFORMATION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of or part of the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

- 9.1.3.FF	AI/ML INFORMATION REPORT

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resources Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| > UE Associated Info Result |  | 0..1 |  |  |  |  |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  |  |  |
| >> UE Assistant Identifier | M |  | NG-RAN node XnAP ID 9.2.3.16 |  |  |  |
| >> UE Performance | O |  | 9.2.3.Y |  |  |  |
| >> UE Trajectory | O |  | 9.2.3.Z |  |  |  |
| Energy Efficiency | O |  | INTEGER (0…FFS) |  |  |  |
| Predicted Energy Efficiency | O |  | INTEGER (0…FFS) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

<Unchanged Text Omitted>

- 9.2	Information Element definitions

<Unchanged Text Omitted>

- 9.2.3	General IE definitions

- 9.2.3.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the XnAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( Cell not Available, Handover Desirable for Radio Reasons, Handover Target not Allowed, Invalid AMF Set ID, No Radio Resources Available in Target Cell, Partial Handover, Reduce Load in Serving Cell, Resource Optimisation Handover, Time Critical Handover, TXnRELOCoverall Expiry, TXnRELOCprep Expiry, Unknown GUAMI ID, Unknown Local NG-RAN node UE XnAP ID, Inconsistent Remote NG-RAN node UE XnAP ID, Encryption And/Or Integrity Protection Algorithms Not Supported, Multiple PDU Session ID Instances, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple QoS Flow ID Instances, Switch Off Ongoing, Not supported 5QI value, TXnDCoverall Expiry, TXnDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve User Bit Rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, UP integrity protection not possible, UP confidentiality protection not possible, Resources not available for the slice(s), UE Maximum integrity protected data rate reason, CP Integrity Protection Failure, UP Integrity Protection Failure, Slice(s) not supported by NG-RAN, MN Mobility, SN Mobility, Count reaches max value, Unknown Old NG-RAN node UE XnAP ID, PDCP Overload, DRB ID not available, Unspecified, …, UE Context ID not known, Non-relocation of context, CHO-CPC resources to be changed, RSN not available for the UP, NPN access denied, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available, Measurement not Supported For The Object, UE Power Saving, Not existing NG-RAN node2 Measurement ID, Insufficient UE Capabilities, Normal Release, Value out of allowed range, SCG activation deactivation failure, SCG deactivation failure due to data transmission, Partial Reporting Failure) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure, O&M Intervention, Not enough User Plane Processing Resources, Unspecified, …) |  |

The meaning of the different cause values is specified in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Cell not Available | The concerned cell is not available. |
| Handover Desirable for Radio Reasons | The reason for requesting handover is radio related. |
| Handover Target not Allowed | Handover to the indicated target cell is not allowed for the UE in question. |
| Invalid AMF Set ID | The target NG-RAN node doesn’t belong to the same AMF Set of the source NG-RAN node, i.e. NG handovers should be attempted instead. |
| No Radio Resources Available in Target Cell | The target cell doesn’t have sufficient radio resources available. |
| Partial Handover | Provides a reason for the handover cancellation. The target NG-RAN node did not admit all PDU Sessions included in the HANDOVER REQUEST and the source NG-RAN node estimated service continuity for the UE would be better by not proceeding with handover towards this particular target NG-RAN node. |
| Reduce Load in Serving Cell | Load in serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| Resource Optimisation Handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Value out of allowed range | The action failed because the proposed Handover Trigger parameter change in the NG-RAN node2 Proposed Mobility Parameters IE is too low or too high. |
| Time Critical Handover | Handover is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| TXnRELOCoverall Expiry | The reason for the action is expiry of timer TXnRELOCoverall. |
| TXnRELOCprep Expiry | Handover Preparation procedure is cancelled when timer TXnRELOCprep expires. |
| Unknown GUAMI ID | The target NG-RAN node belongs to the same AMF Set of the source NG-RAN node and recognizes the AMF Set ID. However, the GUAMI value is unknown to the target NG-RAN node. |
| Unknown Local NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node does not recognise the local NG-RAN node UE XnAP ID. |
| Inconsistent Remote NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node considers that the received remote NG-RAN node UE XnAP ID is inconsistent.. |
| Encryption And/Or Integrity Protection Algorithms Not Supported | The target NG-RAN node is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
| Multiple PDU Session ID Instances | The action failed because multiple instances of the same PDU Session had been provided to the NG-RAN node. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown in the NG-RAN node. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknown in the NG-RAN node. |
| Multiple QoS Flow ID Instances | The action failed because multiple instances of the same QoS flow had been provided to the NG-RAN node. |
| Switch Off Ongoing | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving NG-RAN node in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
| Not supported 5QI value | The action failed because the requested 5QI is not supported. |
| TXnDCoverall Expiry | The reason for the action is expiry of timer TXnDCoverall. |
| TXnDCprep Expiry | The reason for the action is expiry of timer TXnDCprep |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. In the current version of this specification applicable for Dual Connectivity only. |
| Reduce Load | Load in the cell(group) served by the requesting node needs to be reduced. In the current version of this specification applicable for Dual Connectivity only. |
| Resource Optimisation | The reason for requesting this action is to improve the load distribution with the neighbour cells. In the current version of this specification applicable for Dual Connectivity only. |
| Time Critical action | The action is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where radio resources are likely to be dropped if the requested action is not performed. In the current version of this specification applicable for Dual Connectivity only. |
| Target not Allowed | Requested action towards the indicated target cell is not allowed for the UE in question. In the current version of this specification applicable for Dual Connectivity only. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. In the current version of this specification applicable for Dual Connectivity only. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. In the current version of this specification applicable for Dual Connectivity only. |
| Encryption Algorithms Not Supported | The requested NG-RAN node is unable to support any of the encryption algorithms supported by the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. In the current version of this specification applicable for Dual Connectivity only. |
| RRM purpose | The procedure is initiated due to node internal RRM purposes. In the current version of this specification applicable for Dual Connectivity only. |
| Improve User Bit Rate | The reason for requesting this action is to improve the user bit rate. In the current version of this specification applicable for Dual Connectivity only. |
| User Inactivity | The action is requested due to user inactivity on all PDU Sessions. The action may be performed on several levels: - on UE Context level, if NG is requested to be released in order to optimise the radio resources; or S-NG-RAN node didn’t see activity on the PDU session recently. - on PDU Session Resource or DRB or QoS flow level, e.g. if Activity Notification indicate lack of activity In the current version of this specification applicable for Dual Connectivity only. |
| Radio Connection With UE Lost | The action is requested due to losing the radio connection to the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Failure in the Radio Interface Procedure | Radio interface procedure has failed. In the current version of this specification applicable for Dual Connectivity only. |
| Bearer Option not Supported | The requested bearer option is not supported by the sending node. In the current version of this specification applicable for Dual Connectivity only. |
| UP integrity protection not possible | The PDU session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU session cannot be accepted according to the required user plane confidentiality protection policy. |
| Resources not available for the slice(s) | The requested resources are not available for the slice(s). |
| UE Maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum data rate for integrity protection supported by the UE. |
| CP Integrity Protection Failure | The request is not accepted due to failed control plane integrity protection. |
| UP Integrity Protection Failure | The procedure is initiated because the SN (hosting node) detected an Integrity Protection failure in the UL PDU coming from the MN. |
| Slice(s) not supported by NG-RAN | The failure is due to slice(s) not supported by the NG-RAN node. |
| MN Mobility | The procedure is initiated due to relocation of the M-NG-RAN node UE context. |
| SN Mobility | The procedure is initiated due to relocation of the S-NG-RAN node UE context. |
| Count reaches max value, | Indicates the PDCP COUNT for UL or DL reached the max value and the bearer may be released. |
| Unknown Old NG-RAN node UE XnAP ID | The action failed because the Old NG-RAN node UE XnAP ID or the S-NG-RAN node UE XnAP ID is unknown. |
| PDCP Overload | The procedure is initiated due to PDCP resource limitation. |
| DRB ID not available | The action failed because the M-NG-RAN node is not able to provide additional DRB IDs to the S-NG-RAN node. |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| UE Context ID not known | The context retrieval procedure cannot be performed because the UE context cannot be identified. |
| Non-relocation of context | The context retrieval procedure is not performed because the old RAN node has decided not to relocate the UE context. |
| CHO-CPC resources to be changed | The prepared resources for CHO or CPC for a UE are to be changed. |
| RSN not available for the UP | The redundant user plane resources are not available. |
| NPN Access denied | Access denied, or release is required, due to NPN reasons. |
| Report Characteristics Empty | The action failed because there is no measurement object in the report characteristics. |
| Existing Measurement ID | The action failed because the measurement ID is already used. |
| Measurement Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object. |
| Measurement not Supported For The Object | At least one of the concerned object(s) does not support the requested measurement. |
| UE Power Saving | The procedure is initiated to accommodate the preference indicated by UE to release the S-NG-RAN node for UE power saving purpose. |
| Not existing NG-RAN node2 Measurement ID | The action failed because the NG-RAN node2 Measurement ID is not used. |
| Insufficient UE Capabilities | The procedure can’t proceed due to insufficient UE capabilities. |
| Normal Release | The release is due to normal reasons. |
| SCG activation deactivation failure | The action failed due to rejection of the SCG activation deactivation request. |
| SCG deactivation failure due to data transmission | The SCG deactivation failure due to ongoing or arriving data transmission. |
| Partial Reporting Failure | The action failed because part of the requested measurement object cannot be provided. |

| Transport Layer cause | Meaning |
| --- | --- |
| Transport resource unavailable | The required transport resources are not available. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated "reject". |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated "ignore and notify". |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | NG-RAN node control processing overload. |
| Hardware Failure | NG-RAN node hardware failure. |
| Not enough User Plane Processing Resources | NG-RAN node has insufficient user plane processing resources available. |
| O&M Intervention | Operation and Maintenance intervention related to NG-RAN node equipment. |
| Unspecified | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol. |

<Unchanged Text Omitted>

- 9.2.3.X	Start Time and Duration

This IE contains the start time and/or duration for the validity time for AI/ML related information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Start Time | O |  | Relative Time 1900 9.2.36 |  |
| Duration | O |  | INTEGER (0..90060, …) | Unit: seconds |

- 9.2.3.Y	UE Performance

This IE indicates UE performance measurements.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | FFS |  |
| Average UE Throughput UL | O |  | FFS |  |
| Average Packet Delay | O |  | FFS |  |

- 9.2.3.Z	UE Trajectory

This IE indicates UE trajectory measurements.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Trajectory List |  | 1 |  |  |
| >UE Trajectory Item |  | 1..<maxnoofCellsUEMovingTrajectory> |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsUEMovingTrajectory | Maximum no. of cells of UE moving trajectory. Value is 16. |

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>