3GPP TSG-RAN3 Meeting #119	R3-230862

Athens, Greece, 27th February - 3rd March 2023

Agenda Item:	22.2

Source:	ZTE (Moderator)

Title:	Summary of Offline Discussion on CB: # 21_NCR

Document for:	Approval

# Introduction

CB: # 21_NCR

-  Work on stage2 and stage3 TP/CR based on agreements captured above

(ZTE - moderator)

Summary of offline disc R3-230862

# For the Chairman’s Notes

<TBD>

# Discussion

The following agreements on NCR have been made:

The NCR may be configured with a list of allowed and/or forbidden cells.

RAN validation is not supported in R18.

The NG-RAN node selects an appropriate AMF for a NCR accesses to the network with the assumption that the NCR indication is received from NCR in MSG5.

gNB-CU shall indicate the NCR authorization info explicitly to gNB-DU once a NCR device is authorized by UE context setup request message and UE context modification request message.

It is proposed to base the BL CRs as follows to progress at this meeting.

Stage 2(TS 38.300) ZTE: based on R3-230729

Stage 2(TS 38.401) Nokia: based on R3-230180

Stage 3(TS 38.473) HW: based on R3-230101

Please allocate the tdoc numbers and  modify the CRs based on RAN3 agreements.

Please provide comments before Athens time 8:30am, March 3rd .

In addition, considering the NCR WI is going to be completed at the end of RAN3#119 and RAN3 agreements clearly have high impact to the RAN2 and SA2, an LS which is used to inform other WGs (e.g. RAN2, SA2) the RAN3 progress on NCR WI shall be sent by RAN3 in this meeting. A draft LS has been uploaded to the folder.

Q0: Comments/remarks regarding draft LS out.

| Company | Comment |
| --- | --- |
| Ericsson | We agree to send the LS. RAN1 and SA5 need to be in “to” because we have captured some agreements made by RAN1 which may impact SA5 (OAM requirements). A further editorial is needed (see our proposed update of the draft LS). |
| CATT | Fine with the LS, also ok with Ericsson’s revision. |

Moderator Summary : TBD

Q1: Comments/remarks regarding Stage 2 (TS 38.300)

| Company | Comment |
| --- | --- |
| Ericsson | Sec. X.Y: 1) It does not seem correct to mention configuration/implementation in normative text. So, we would propose to remove the sentence about gNB checking whether NCR camps on the correct cell, For the same reason, we propose to change the statement about characterization of beam information into a note, like in our corresponding TP (0469): “NOTE: How to characterize the beam information is up to implementation.” “(e.g. declaration from NCR vendor) is redundant” Sec. X.Y1: 2) The figure needs to be updated according to the current RAN3 agreements: remove the NCR Supported Indicator from step 2 and the NCR Indication info from step 9. 2bis: Step 1 to step 2: needs to be removed according to latest RAN3 agreements. 3) Step 3 to step 7: shall be sent -> is sent (this is a stage 2 description) 4) Step 8: reword according to RAN3 agreement: “The serving gNB selects an appropriate AMF for the NCR” 5) Step 9: not needed according to latest RAN3 agreements 5) Step 10: reword to “AMF provides authorization information to the gNB” Please see corresponding changes in our proposed updates to the draft, which includes various editorials we spotted. (NB it was not possible to update the figure) |
| CATT | Fine with Ericsson’s revision. |

Moderator Summary : TBD

Q2: Comments/remarks regarding Stage 2 (TS 38.401)

| Company | Comment |
| --- | --- |
| Ericsson | No need to capture the OAM requirements also here: they are already captured in TS 38.300. Then, also NOTE 1 is not needed. Please see corresponding change in our proposed update to the draft, which also includes a few editorials. |
| CATT | Fine with E///’s revision, OAM related business have already covered in 38.300 OAM part. One minor thing, which terminology should be used for the stage 2, “NCR” or “Network Controlled Repeater”? |

Moderator Summary : TBD

Q3: Comments/remarks regarding Stage 3 (TS 38.473)

| Company | Comment |
| --- | --- |
| Ericsson | Current TP includes the cell barring indication from CU to DU, which is implicitly “ported” from IAB. We are not in principle opposed to such a functionality, but we wonder whether it’s strictly needed for an NCR. Furthermore, a couple of minor changes seem beneficial: 1) the semantics indication for 9.3.1.XXX does not seem needed: it simply repeats the description above the tabular 2) the description above the tabular for 9.3.1.XXX should simply say “This IE provides the authorization status of the Network Controlled Repeater.” |
| CATT | Fine, add CATT to the support companies. |

Moderator Summary : TBD

Q4: Other comments?

| Company | Comment |
| --- | --- |

Moderator Summary : TBD

# References

- R3-230729	(TPs for NCR BL CR 38413 and pCR 38300) NCR left issues with draft LS (ZTE)	other

- R3-230730	(TPs for NCR BL CR for TS 38401 and TS 38473)NCR network architecture (ZTE)	other

- R3-230714	Why RAN validation is still necessary (Intel Corporation)	discussion

- R3-230715	[Draft] Reply LS on NCR validation (Intel Corporation)	LS out To: SA3 CC: RAN2, SA2, SA5

- R3-230100	(TPs to TS 38.300, TS 38.413 BL CRs) Support of Network-Controlled Repeater (Huawei)	other

- R3-230101	Support of Network-Controlled Repeater (Huawei, Nokia, Nokia Shanghai Bell)	CR1109r, TS 38.473 v17.3.0, Rel-18, Cat. B

- Rev in R3-230804

- R3-230179	Discussion on Identification and authorization of Network Controlled Repeater (Nokia, Nokia Shanghai Bell)	discussion

- R3-230180	Support of Network-Controlled Repeater (Nokia, Nokia Shanghai Bell, Huawei)	CR0274r, TS 38.401 v17.3.0, Rel-18, Cat. B

- R3-230152	(TP for TS 38.300 BL CR) Support of NCR management (CATT)	other

- R3-230153	Support of NCR authorization in F1AP (CATT)	CR1110r, TS 38.473 v17.3.0, Rel-18, Cat. B

- R3-230231	Management of Network-Controlled Repeater (Qualcomm Inc.)	discussion

- R3-230272	(TP to TS38.413 on NCR) Support of Network-Control Repeater (NCR) (NEC, KDDI Corporation, Rakuten Mobile, Inc)	other

- R3-230468	Remaining Open Issues on NCR (Ericsson LM)	discussion

- R3-230469	Further Stage 2 Impacts for NCR (Ericsson LM)	other

- R3-230534	Discussion on F1AP enhancement to support Network Controlled Repeater (Samsung)	discussion

- R3-230535	(TP for NR_netcon_repeater BL CR for TS 38.473) Authorization for Network Controlled Repeater (Samsung)	other

- R3-230644	Further Discussion on NCR Management (China Telecommunications)	discussion

- R3-230651	Support of Network Controlled Repeater (CMCC)	CR1133r, TS 38.473 v17.3.0, Rel-18, Cat. B

- R3-230749	Discussion on the network-controlled repeater management (TP to TS 38.413) (CMCC)	discussion