3GPP TSG-RAN WG3 Meeting #119bis-e                                                      	R3-231343

E-meeting, April 17th – April 26th, 2023

Agenda item:	10.2.5

Source:	Qualcomm Incorporated

Title:	SON enhancements for NR-U

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the open issues on SON enhancements for NR-U based on the discussion in the previous RAN3 meetings.

# Discussion

  - RLF Report enhancements for NR-U

FFS whether to include the following in RLF report:

a.        indications of number of consistent LBT failures and at which granularity (e.g., per BWP)

b.        EDT in UL (e.g., exact value, average, max)

c.        waiting time in uplink due to LBT

### Number of consistent LBT failures

RAN2 has made the following agreements:

Log “the number of LBT failures” in RA report, i.e., RACH attempts blocked by LBT

The definition of “the number of LBT failures” should be clarified

FFS how to log the number of LBT failures in RA report

- Indication per attempt if blocked by LBT indication

- Number of blocked attempts per SSB/CSI-RS

- Number of blocked attempts per RACH procedure

RAN2 is already discussing this topic and we therefore propose to leave it up to RAN2 discussion and not have duplicate discussions in RAN3.

Proposal 1: Leave it up to RAN2 to discuss details on how to log number of LBT failures and at what granularity.

### Energy Detection Threshold (EDT)

From TS 38.331, it is seen that a UE can use different values for EDT in UL, as long as this value is within a configured maximum EDT value. The exact EDT value used by the UE is quite random and even an averaged value would not give much information to the gNB. Also, different UEs might use different EDT thresholds, so getting an overall picture from different “averaged EDT” values from different UEs is also very cumbersome. Further, a UE is configured with energyDetectionConfig, so the network is aware of the max EDT value and other EDT related parameters, so asking the UE to report the EDT parameters back is redundant.

Observation 1: Specification allows a UE to use different values for Energy Detection Threshold (EDT) in UL (as long as this value is within a configured maximum EDT value), but not many UEs in actual implementation would use a “conservative” approach and select an EDT value less than the max allowed EDT value

Observation 2: Reporting the exact value of EDT used for each LBT attempt reveals UE implementation details and is also cumbersome to report for each LBT attempt

Observation 3: Even if UE reports the average value of the EDT in each LBT attempt, it is not clear how this is useful at the gNB, considering different UE implementations might report different averaged EDT value and taking an average of these averaged EDT values doesn’t give much knowledge to the gNB

Proposal 2: There is no need for UE to report any EDT value (exact value or average value) for optimizing UL LBT

### Waiting time in UL due to LBT

It was further discussed whether to add any LBT related time duration in RLF Report e.g., time between each LBT start and LBT success or time between each LBT start and a subsequent LBT failure or waiting time. Some companies argued that it is useful for the network to decide how the report should be used while doing MRO analysis, for example, if long time duration is spent for LBT, it may mean that the failure is mainly caused by channel occupancy.

We are not convinced that knowing this exact time can help much in MRO analysis (we already send an indication that HOF is due to consistent LBT failure) and requesting UE to compute this time during each LBT attempt is too much processing at the UE.

Observation 4: gNB can't optimize much knowing the exact timing related information related to LBT e.g., time between LBT start to LBT success/failure. Also requesting the UE to compute this time during each LBT attempt is too much processing at the UE.

Proposal 3: There is no need for UE to report any timing information related to LBT in RLF report

  - RA Report enhancements for NR-U

FFS whether there is need to add the following in RA Report

a.        information of LBT failures occurring during the RA procedure. FFS on the granularity of this information

b.        addition of EDT in UL from UE (and which value, e.g., exact value, average, max)

c.        addition of Measured RSSI

d.        addition of UL LBT duration time

UE also doesn’t send any measurements in RA Report today. So RSSI need to be sent as well in RA Report. RSSI in RLF report is sufficient.

Observation 5: Measured RSSI is already agreed to be included in RLF Report and no measurements are included in current RA-Report

LBT related timers e.g., LBT duration time was already discussed in section 2.1 and as we mentioned, there is not much benefit to include exact timer values.

Proposal 4: There is no need for UE to include measured RSSI and UL LBT duration time in RA-Report

# Conclusion

Proposal 1: Leave it up to RAN2 to discuss details on how to log number of LBT failures and at what granularity.

Observation 1: Specification allows a UE to use different values for Energy Detection Threshold (EDT) in UL (as long as this value is within a configured maximum EDT value), but not many UEs in actual implementation would use a “conservative” approach and select an EDT value less than the max allowed EDT value

Observation 2: Reporting the exact value of EDT used for each LBT attempt reveals UE implementation details and is also cumbersome to report for each LBT attempt

Observation 3: Even if UE reports the average value of the EDT in each LBT attempt, it is not clear how this is useful at the gNB, considering different UE implementations might report different averaged EDT value and taking an average of these averaged EDT values doesn’t give much knowledge to the gNB

Proposal 2: There is no need for UE to report any EDT value (exact value or average value) for optimizing UL LBT

Observation 4: gNB can't optimize much knowing the exact timing related information related to LBT e.g., time between LBT start to LBT success/failure. Also requesting the UE to compute this time during each LBT attempt is too much processing at the UE.

Proposal 3: There is no need for UE to report any timing information related to LBT in RLF report

Observation 5: Measured RSSI is already agreed to be included in RLF Report and no measurements are included in current RA-Report

Proposal 4: There is no need for UE to include measured RSSI and UL LBT duration time in RA-Report

# 4. References