3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231390

Online, 17 – 26 Apr, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0287 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Multicast session establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, Lenovo, ZTE, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the RAN3#119 meeting, the GTP-UL TEID info in the broadcast session establishment procedure had been deleted to align with stage 3 specification. However, for NG-U tunnel establishment procedure of multicast session, the current version of NGAP also does not specify the GTP UL TEID for NG-U unicast transport in Distribution Setup procedure. Therefore, the GTP UL TEID for NG-U unicast transport in steps 4/5 of Multicast session establishment procedure should be removed . - To provide the 5GC NG-U TNL address for multicast transport from gNB-CU-CP to gNB-CU-UP, the MC BEARER CONTEXT MODIFICATION REQUEST message should be used. In the current TS 38.401, the steps 5a/5b added in the RAN3#119 meeting shall include the NG-U TNL, which should be corrected to address for multicast transport instead of GTP UL TEID. And also steps 11/12 in the callflow is updated to only include the F1-U TNL DU info and remove the NG-U TNL info, while the description of steps 11/12 still include the NG-U TNL redundantly. Thus, the description of steps 11/12 should be aligned. - Wording issues, e.g. change "DU" to "gNB-DU". - Redundancy between step 10 and step 11/12. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the GTP UL TEID for NG-U unicast transport in steps 4/5 of Multicast session establishment procedure - Update the wording of "5GC UL TEID" to "5GC NG-U TNL address for multicast transport" in steps 5a/5b, and remove the wording of "and the 5GC NG-U TNL address " in steps 11/12 - Change "DU" to "gNB-DU". - Remove the redundancy between step 10 and step 11/12. - Typos fixed. Impact Analysis: This CR has an isolated impact towards the previous version of the specification, and will not cause any non-backward compatible issues. This CR only have impacts on the MBS multicast context establiment function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between the call flow of MBS multicast context establiment and the NGAP specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.15.1.2	Multicast MBS Session Context Establishment

Figure 8.15.1.2-1 illustrates an exemplified interaction of NGAP, E1AP, F1AP and RRC protocol functions for Multicast MBS Session Context Establishment with a UE joining an active multicast session as the first UE joining in its serving gNB.

Figure 8.15.1.2-1: Multicast MBS Session Context establishment

0.	For a certain multicast MBS session (TMGI), which is currently active, no multicast MBS session context is yet established in the gNB.

1.	The UE joins the multicast session.

NOTE 1:	NAS related details for PDU Session signalling are not shown in Figure 8.15.1.2-1.

2/3. The gNB-CU-CP establishes the multicast bearer context at the gNB-CU-UP, in order to retrieve unicast NG-U transport the GTP DL TEID.

4/5. The gNB-CU CP triggers the NGAP Distribution Setup procedure. For unicast transport, DL GTP TEID is provided to 5GC. For multicast transport, multicast address information is retrieved from the 5GC. Multicast session QoS parameters are provided by 5GC.

5a/5b. The gNB-CU-CP triggers the setup of MRB resources via the E1AP MC Bearer Context Modification procedure and provides the 5GC shared NG-U multicast address information if any, and Multicast session QoS parameters to the gNB-CU-UP.

6/7.	The gNB-CU-CP establishes the Multicast Context at the gNB-DU, providing MRB configuration. It may contain MBS Area Session ID information.

8/9. The gNB-CU-CP retrieves the MRB configuration for the joined UE from the gNB-DU via F1 UE Context Management procedures.

NOTE 2:	The interactions between the gNB-CU-CP and gNB-CU-UP for UE specific Berarer Context management before and after step 8/9 are not illustrated in this Figure.

10.	The gNB-DU triggers the establishment of F1-U resources, which are established either per gNB-DU or per cell or per MBS Area Session ID.

11/12. The gNB-CU-UP side of the F1-U and NG-U UP entity is established by means of the E1AP MC Bearer Context Modification procedure, providing the gNB-DU side F1-U TNL to the gNB-CU-UP, which provides the gNB-CU-UP side F1-U TNL address in return.

13.	The gNB-CU-UP side F1-U TNL address is provided to the gNB-DU.

14.	In case of NG-U multicast transport, the gNB-CU-UP joins the NG-U multicast group.

15.	The gNB successfully terminates the NGAP procedure for establishing the multicast session context.

16.	The gNB-CU-CP RRC-configures each UE which has joined the multicast group.

17.	The multicast MBS media stream is provided to the UEs.

On NG-U, in case of location dependent multicast MBS Sessions, multiple shared NG-U transport tunnels may need to be setup, one per Area Session ID served by the gNB.

In case of shared NG-U termination,

-	the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs.

-	at establishment of the MC bearer context in the gNB-CU-UP, the gNB-CU- CP may request the gNB-CU-UP to either apply the available MRB configuration of the shared NG-U termination, or to apply the MRB configuration requested by the gNB-CU-CP. The gNB-CU-UP provides the MRB configuration to the gNB-CU-CP if the MRB configuration requested by the gNB-CU-CP and the available MRB configuration of the shared NG-U termination are different.