3GPP TSG-RAN3 #119-bis-e	R3-231601

E-meeting, 17th April – 26th April 2023

Agenda Item:	8.1

Source:	Ericsson

Title:	Discussion on new attribute “Only Resource Coordination”

Document for:	Discussion, Decision

# 1. Introduction

RAN3 has received an LS from SA5 in S5-233142 where the following is requested:

SA5 understands that it is beneficial to use an EN-DC X2 interface only for coordination of resources between E-UTRA cells and NR cells controlled by different RAN nodes.

In order to maintain the consistency of the attributes of the NRCellRelation IOC in TS 28.541 with the definitions of RAN3, SA5 requires more detailed information about the usage and semantics of this new attribute, i.e., use case that motivates this attribute, which network function is supposed to set this attribute (e.g. by the neighboring eNB/gNB via X2, or by human operator via OAM, in gNB or eNB), the effect of this attribute on the system once it was set.

SA5 will take the response from RAN3 into account to specify the attribute requested by RAN3 if needed.

In this paper a summary of RAN3´s discussions leading to the agreement to request SA5 to introduce the new “Only Resource Coordination” attribute are recapped and a reply LS is proposed.

# 2. Discussion

During RAN3-117e, the following agreement was taken:

Agree to introduce a new attribute “Only Resource Coordination” in Neighbour Cell Relation to support resource coordination between LTE and NR SA sites.

This agreement was taken on the basis of a solution proposal that was reflected in another agreement taken earlier at RAN3-114e, namely:

Reuse Protected E-UTRA Resource Indication (from LTE to NR) + E-UTRA-NR cell resource coordination information to support resource coordinate between SA NR and LTE. Whether to exchange PRACH Configuration as resource coordination information is FFS.

The above agreements were based on a solution that RAN3 agreed to, then called “Solution 1”. Such solution is described below for convenience:

Solution 1: The assumption in Solution 1 is that an SA gNB can also support an EN-DC X2 connection. Namely an SA gNB can also act as an en-gNB. Therefore, the solution proposes to reuse the E-UTRA – NR Cell Resource Coordination function available over the EN-DC X2 interface to enable the coordination of resources between co-channel sharing LTE and NR cells.

The main concern from operators at the time of discussions in RAN3 was that it would be too costly to use the EN-DC X2 interface for resource coordination as in Solution1. This is because the EN-DC X2 interface supports many features and functions that would not be needed if the only purpose of establishing such interface is to run the E-UTRA – NR Cell Resource Coordination function.

This led to the agreement “to introduce a new attribute “Only Resource Coordination” in Neighbour Cell Relation to support resource coordination between LTE and NR SA sites”.

At RAN3-117e RAN3 also endorsed a CR to TS38.300, see R3-225158, where the new “Resource Coordination Only” attribute was added to the Neighbour Cell Relation and defined as follows:

Resource Coordination Only: If checked, the Neighbour Cell Relation shall use the X2 interface instance only to coordinate resources between the E-UTRA cell and the NR cell.

On the basis of the above information, answers can be derived to the questions asked by SA5.

Question 1: use case that motivates this attribute:

As explained above, the attribute is motivated by the need to coordination resources between co-channel sharing E-UTRA and NR cells, where E-UTRA cells and NR cells are served by different sites and where the site serving NR cells includes a gNB and an en-gNB. Under this deployment scenario, the assumption made by RAN3 is that the EN-DC X2 E-UTRA – NR Cell Resource Coordination procedure can lead to resource coordination between E-UTRA cells of the eNB and NR cells of the en-gNB and that, by means of implementation, the gNB co-located with the en-gNB would be also able to coordinate its resources with the E-UTRA cells.

Answer 1: : the attribute is motivated by the need to coordinate resources between co-channel sharing E-UTRA and NR cells, where E-UTRA cells are served by an E-UTRAN eNB in one site and NR cells are served by an NG-RAN gNB and an E-UTRAN en-gNB collocated at a different site. Under this deployment scenario, the assumption made by RAN3 is that the X2: E-UTRA – NR Cell Resource Coordination procedure can lead to resource coordination between E-UTRAN cells of the eNB and NR cells of the en-gNB and that, by means of implementation, the gNB co-located with the en-gNB would also be able to coordinate its resources with the E-UTRAN cells.

Question 2: which network function is supposed to set this attribute (e.g. by the neighboring eNB/gNB via X2, or by human operator via OAM, in gNB or eNB)

The reason for RAN3 to send an LS to SA5 asking SA5 to implement the changes in their specifications is that RAN3 envisaged the new attribute to be set by the OAM system. Whether this is the consequence of manual configuration or an automated configuration is out of RAN3 scope.

Answer 2: RAN3 envisaged the new attribute to be set by the OAM system at the eNB and at the en-gNB. In case of split RAN architecture, the attribute should be set at the CU-CP node of each of the eNB and en-gNB terminating the EN-DC X2 interface. Whether this is the consequence of manual configuration or an automated configuration is out of RAN3 scope.

Question 3: effect of this attribute on the system once it was set.

As described in the endorsed CR in R3-225158 sent to SA5 as part of the LS in R3-225206, the effect of the attribute on the system is that “If checked, the Neighbour Cell Relation shall use the X2 interface instance only to coordinate resources between the E-UTRA cell and the NR cell”. There is no other effect RAN3 foresaw for this attribute.

Answer 3: If the “Resource Coordination Only” attribute is checked, the eNB and en-gNB between which the EN-DC X2 interface is established shall use the X2 interface instance only to run basic procedures, e.g. EN-DC X2 Setup, and to coordinate resources between the E-UTRA cell and the NR cell. No more effects on the system were foreseen by RAN3.

Proposal: it is proposed to reply to SA5 with an LS containing the following answers

- Answer 1: the attribute is motivated by the need to coordinate resources between co-channel sharing E-UTRA and NR cells, where E-UTRA cells are served by an E-UTRAN eNB in one site and NR cells are served by an NG-RAN gNB and an E-UTRAN en-gNB collocated at a different site. Under this deployment scenario, the assumption made by RAN3 is that the X2: E-UTRA – NR Cell Resource Coordination procedure can lead to resource coordination between E-UTRAN cells of the eNB and NR cells of the en-gNB and that, by means of implementation, the gNB co-located with the en-gNB would also be able to coordinate its resources with the E-UTRAN cells.

- Answer 2: RAN3 envisaged the new attribute to be set by the OAM system at the eNB and at the en-gNB. In case of split RAN architecture, the attribute should be set at the CU-CP node of each of the eNB and en-gNB terminating the EN-DC X2 interface. Whether this is the consequence of manual configuration or an automated configuration is out of RAN3 scope.

- Answer 3: If the “Resource Coordination Only” attribute is checked, the eNB and en-gNB between which the EN-DC X2 interface is established shall use the X2 interface instance only to run basic procedures, e.g. EN-DC X2 Setup, and to coordinate resources between the E-UTRA cell and the NR cell. No more effects on the system were foreseen by RAN3.

# Conclusions

In this paper discussions leading to the definition of a new “Resource Coordination Only” attribute and endorsement of the CR in R3-225158 were summarised.

The paper proposes to reply to the LS from SA5 in S5-233142 as per the following proposal:

Proposal: it is proposed to reply to SA5 with an LS containing the following answers

- Answer 1: the attribute is motivated by the need to coordinate resources between co-channel sharing E-UTRA and NR cells, where E-UTRA cells are served by an E-UTRAN eNB in one site and NR cells are served by an NG-RAN gNB and an E-UTRAN en-gNB collocated at a different site. Under this deployment scenario, the assumption made by RAN3 is that the X2: E-UTRA – NR Cell Resource Coordination procedure can lead to resource coordination between E-UTRAN cells of the eNB and NR cells of the en-gNB and that, by means of implementation, the gNB co-located with the en-gNB would also be able to coordinate its resources with the E-UTRAN cells.

- Answer 2: RAN3 envisaged the new attribute to be set by the OAM system at the eNB and at the en-gNB. In case of split RAN architecture, the attribute should be set at the CU-CP node of each of the eNB and en-gNB terminating the EN-DC X2 interface. Whether this is the consequence of manual configuration or an automated configuration is out of RAN3 scope.

- Answer 3: If the “Resource Coordination Only” attribute is checked, the eNB and en-gNB between which the EN-DC X2 interface is established shall use the X2 interface instance only to run basic procedures, e.g. EN-DC X2 Setup, and to coordinate resources between the E-UTRA cell and the NR cell. No more effects on the system were foreseen by RAN3.

A draft reply LS mirroring the proposal above is available in the Annex

# Annex: Draft Reply LS

3GPP TSG-RAN3 #119-bis-e	R3-23xxxx

E-meeting, 17th April – 26th April 2023

Title:	Reply LS on introduction of a new attribute “Only Resource Coordination” to support source coordination between LTE and NR SA

Response to:	R3-231124/S5-233142

Release:	Rel-17

Work Item:	TEI-17

Source:	Ericsson (to be RAN3)

To:	SA5

Contact person:	Angelo Centonza

Angelo.centonza (at) Ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 thanks SA5 for the reply LS in S5-233142.

RAN3 understands that SA5 is asking three main questions to RAN3 concerning the new “Resource Coordination Only” attribute that RAN3 endorsed in the CR to TS38.300 in R3-225158, namely

Question 1: use case that motivates this attribute

Question 2: which network function is supposed to set this attribute (e.g. by the neighboring eNB/gNB via X2, or by human operator via OAM, in gNB or eNB)

Question 3: effect of this attribute on the system once it was set

RAN3 would like to answer these questions as follows:

Answer 1: the attribute is motivated by the need to coordinate resources between co-channel sharing E-UTRA and NR cells, where E-UTRA cells are served by an E-UTRAN eNB in one site and NR cells are served by an NG-RAN gNB and an E-UTRAN en-gNB collocated at a different site. Under this deployment scenario, the assumption made by RAN3 is that the X2: E-UTRA – NR Cell Resource Coordination procedure can lead to resource coordination between E-UTRAN cells of the eNB and NR cells of the en-gNB and that, by means of implementation, the gNB co-located with the en-gNB would also be able to coordinate its resources with the E-UTRAN cells.

Answer 2: RAN3 envisaged the new attribute to be set by the OAM system at the eNB and at the en-gNB. In case of split RAN architecture, the attribute should be set at the CU-CP node of each of the eNB and en-gNB terminating the EN-DC X2 interface. Whether this is the consequence of manual configuration or an automated configuration is out of RAN3 scope.

Answer 3: If the “Resource Coordination Only” attribute is checked, the eNB and en-gNB between which the EN-DC X2 interface is established shall use the X2 interface instance only to run basic procedures, e.g. EN-DC X2 Setup, and to coordinate resources between the E-UTRA cell and the NR cell. No more effects on the system were foreseen by RAN3.

# 2	Actions

To SA5

ACTION: 	RAN3 kindly asks SA5 to take the responses into account and to provide feedback if any. If the answers are sufficient, RAN3 asks SA5 to specify the requested attribute.

# 3	Dates of next RAN3 meetings

RAN3 Meeting #120	22-26 May 2023	                                       Incheon, KR

RAN3 Meeting #121	21-25 August 2023	                     Toulouse, FR