3GPP RAN WG3 Meeting #119bis-e	R3-231639

# Online, 17th – 26th April, 2023

Agenda Item:	20.2

Source:	Ericsson

Title:	Discussion on MT-SDT open issues

Document for:	Discussion

# Introduction

RAN3 have discussed in RAN3#118 meeting the MT-SDT and have made the following agreements:

MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling. Upon reception of DL SDT user data, the gNB-CU-UP may include the assistance information (e.g., Data size) in E1AP DL Data Notification message to gNB-CU-CP. FFS on MT-SDT indicator. When receiving DL SDT data, the anchor gNB may send MT-SDT information IE to the neighbour gNBs within the RNA, via XnAP RAN paging message. The encoding and the name of MT-SDT information IE is FFS. The gNB that receives MT-SDT information within the RNA takes into account this information received in the XnAP RAN PAGING message from the anchor gNB to decide whether to trigger MT-SDT Uu paging. Upon reception of MT-SDT information via XnAP RAN paging message from the anchor gNB-CU, the gNB-CU may send F1 MT-SDT information to the gNB-DU via F1AP Paging message. FFS on F1AP MT-SDT information FFS: whether Data volume should also be provided from the CU to the DU in F1AP PAGING message. FFS: whether and how XnAP RTRV UE CTXT REQ message (that carries MT-SDT resume indication) FFS: whether MT-SDT support indication in E1 Bearer Context procedure should be defined to enable the gNB-CU-UP to include the DL data size in the E1AP DL DATA NOTIFICATION message. FFS: whether receiving gNB-CU or receving gNB-DU decides MT-SDT Uu paging, if we agree that it is the receving gNB to make the final decision on triggering MT-SDT Uu paging. FFS: MT-SDT assistance information sent from the anchor in the XnAP: RAN paging message and other alignment with RAN2 progress.

In this paper, we provide our views on the open issues.

# Discussion

On the first agreement that “MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling” this is not fully correct, as it is the serving gNB can classifies the received DL user data and/or DL signalling to decide whether it can be used for ther MT-SDT session or not, since CN is not aware of SDT operation, there is no functionality in CN. It is needed to reformulate the proposal as follows:

- MT-SDT can be triggered by DL user data and/or DL signalling.

alternatively:

- MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling, as classified by the serving gNB.

Proposal 1: reformulate the agreement that MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling, as classified by the serving gNB

On the MT-SDT Information IE, the data size is sufficient, because first there is no need to signal an explicit indicator, the data size is enough, as an explicit indicator is not needed when the MT-SDT Information IE contains the data size, which will help the receiving node making decision of SDT.

Proposal 2: There is no need for an explicit MT-SDT indicator over Xn RAN PAGING message, the implicit data size in the IE is enough to make receiving node make decision of MT-SDT paging

We provide a TP to XnAP BL CR in the annex of this paper.

For F1AP, there is question whether the CU or DU makes decision of MT-SDT paging, in our understanding based on the MT-SDT volume size, and thus the assistance information that should be sent over F1. The question is about the entity that formulates the paging decision. As per definition, the CU filters the cells for paging based on many factors, such as cell characteristics, UE radio capabilities for paging, number of paging attempts, etc.. But it is the DU that handles the processing part when it comes to decide what should be sent and scheduled in the paging message over Uu. Thus, while CU decides and filters the cells for paging and sends the MT-SDT information, but for the paging decision over Uu, it is the DU’s task as it must consider the amount of data that needs to be sent and scheduled.

Observation 1: The CU decides and filters the cells for paging and sends the MT-SDT information, but for the paging decision over Uu, this is the DU task as it must consider the amount of data that needs to be sent and scheduled.

Therefore, it is proposed that DU makes decision of MT-SDT paging and the MT-SDT Information over F1 Paging contains the data volume size.

Proposal 3: DU makes decision of MT-SDT paging and the MT-SDT Information over F1 Paging contains the data volume size. Agree to TP to F1 BL CR in R3-231640

Furthermore, RAN2 has agreed that a New Resume cause in RRC resume will be introduced, one code point MT-SDT indication should be added in XnAP Resume Cause IE.

Proposal 4: Add new codepoint 'mo-SDT’ in RRC Resume Cause IE in XnAP

# Conclusions and Proposals

Our observations and proposals are summarized below.

Proposal 1: reformulate the agreement that MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling, as classified by the serving gNB

Proposal 2: There is no need for an explicit MT-SDT indicator over Xn RAN PAGING message, the implicit data size in the IE is enough to make receiving node make decision of MT-SDT paging

Observation 1: The CU decides and filters the cells for paging and sends the MT-SDT information, but for the paging decision over Uu, this is the DU task as it must consider the amount of data that needs to be sent and scheduled.

Proposal 3: DU makes decision of MT-SDT paging and the MT-SDT Information over F1 Paging contains the data volume size.

Proposal 4: Add new codepoint 'mo-SDT’ in RRC Resume Cause IE in XnAP

# Annex – TP to TS 38.423

>>>>>>START OF CHANGES<<<<<<<

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

If the Assistance Data for RAN Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

If the UE Radio Capability for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it to apply specific paging schemes.

If the Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 36.304 [34], and for eDRX or the UE_ID based subgrouping according to TS 38.304 [33]. When available, NG-RAN node1 may include the Extended UE Identity Index Value IE in the RAN PAGING message towards the NG-RAN node2.

When available, the NG-RAN node1 shall include the E-UTRA Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the E-UTRA Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN PAGING message towards the NG-RAN node2. If the UE specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the NR Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the NR Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the NR Paging eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

When available, the NG-RAN node1 shall include the Paging Cause IE in the RAN PAGING message towards the NG-RAN node2. If the Paging Cause IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.331 [10].

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the MT-SDT Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it to decide whether to initiate MT-SDT procedures towards the UE based on the data size information as described in  TS 38.300 [9].

<Next change>

#### 9.2.3.61	RRC Resume Cause

The purpose of the RRC Resume Cause IE is to indicate to the old NG-RAN node the reason for the RRC Connection Resume as received from the UE in the ResumeCause defined in TS 36.331 [14] and TS 38.331 [10].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC Resume Cause | M |  | ENUMERATED (rna-Update, ..., mo-Sdt) |  |

<Next change>

#### 9.2.3.xxx	MT-SDT Information

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Data size | M |  | FFS |  |

Editor’s note: the name and the encoding of the MT-SDT Information IE is FFS

>>>>>>NEXT CHANGE<<<<<<<

*** skip unchanged ***

MessageOversizeNotification ::= SEQUENCE {

maximumCellListSize								MaximumCellListSize,

iE-Extension				ProtocolExtensionContainer { {MessageOversizeNotification-ExtIEs}}	OPTIONAL,

...

}

MessageOversizeNotification-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MaximumCellListSize ::= INTEGER(1..16384, ...)

MT-SDT-Information ::= SEQUENCE {

Data-size					FFS,

iE-Extensions				ProtocolExtensionContainer { { MT-SDT-Information-ExtIEs} } OPTIONAL,

...

}

MT-SDT-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

--Editor’s note: the name and the encoding of the MT-SDT Information IE is FFS

}

*** skip unchanged ***

RRCResumeCause ::= ENUMERATED {

rna-Update,

...,

mo-Sdt

}

>>>>>>END OF CHANGEs<<<<<<<