3GPP TSG-RAN WG3 Meeting #119bis-e          	          R3-231808

Online, 17th Apr – 26th Apr 2023

Agenda item:	14.2

Source:	CMCC

Title:	(TP to TS 38.401) L1/L2 based Inter-Cell Mobility

Document for:	Approval

1	Introduction

This contribution provides the TP to TS 38.401 for L1/L2 based Inter-Cell Mobility based on the discussion paper.

- Text Proposal to TS 38.401 BLCR

--------------------------------------------------------------Start of the change---------------------------------------------------------

8.2.1.X	Intra-gNB-DU L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for L1/L2 triggered mobility. Figure 8.2.1.x-1 shows the intra-gNB-DU L1/L2 triggered mobility procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU L1/L2 Triggered Mobility

- The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 triggered mobility configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing the target candidate cells.

- If the gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell(s).

FFS for Step 3 and Step 4: either a single or multiple UE Context Modification procedure(s) should be used.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 triggered mobility configuration.

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the lower layer measurement result to the gNB-DU. The gNB-DU decides to execute L1/L2 inter-cell mobility.

- The gNB-DU sends the LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The gNB-DU signals the gNB-CU about the initiation of the L1/L2 triggered mobility command to the UE by L1/L2 Mobility Notification message including the Target cell ID.

- FFS: How the target gNB-DU detects the UE access.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

- The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with theUE CONTEXT MODIFICATION RESPONSE message.

--------------------------------------------------------------Next change -----------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for L1/L2 Triggered Mobility. Figure 8.2.1.Y-1 shows the inter-gNB-DU L1/L2 Triggered Mobility procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU L1/L2 Triggered Mobility

- The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 triggered mobility configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cells.

FFS for Step 3 and Step 4: either a single or multiple UE Context Setup procedure(s) should be used.

- If the candidate gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 triggered mobility configuration.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the lower layer measurement result to the source gNB-DU.

- The source gNB-DU decides to execute L1/L2 triggered mobility to a candidate target cell.

FFS: Notifying the LTM triggering decision to the other nodes as well.

- The source gNB-DU sends LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The source gNB-DU signals the gNB-CU about the initiation of the L1/L2 triggered mobility command to the UE by L1/L2 Mobility Notification message including the Target cell ID.

- FFS:  how the target gNB-DU detects the UE access and whether there is an F1 impact.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

- The gNB-CU sends the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of cells.

- The gNB-DU responds with the UE CONTEXT RELEASE COMPLETE message.

--------------------------------------------------------------End of the change----------------------------------------------------------