3GPP TSG-RAN WG3 #119bis-e	R3-231909

Online, 17th – 26th April 2023

Agenda Item:	9.1

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # 47_SubcarrierSpacingSSB

Document for:	Approval

# Introduction

CB: # 47_SubcarrierSpacingSSB

- Check the approved WI objectives and specs

- Check technical details

(moderator - Nok)

Summary of offline disc R3-231909

- This CB is related to TS36.413 CR “R3-231220, Correction for SubcarrierSpacingSSB”

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

## Issue 1 - SubcarrierSpacingSSB

TS36.413 CR (R3-231220) is proposed to address following misalignment between TS36.413 and TS36.331:

- Clarify kHz60 is not used in the speficication

- Add kHz480 and kHz960 introduced in Rel-17 TS36.331

- Correct the RRC reference

The main issue is the 2 new codepoints kHz480 and kHz960, which was introduced in TS36.331 per CR#4869 (Cat F, WI: NR_ext_to_71GHz-Core). The 71GHz WID (RP-222656) does not list any 36.xxx specs, but RAN agreed RAN2 TS36.xxx CRs in RAN Plenary #97. The TS36.306/331 CRs agreed in RAN#97 (RP-222525) is copied as below:

| 36.306 | 1856 | Rel-17 | UE Capability CR for HO from E-UTRA to FR2-2 | F | 17.1.0 | RAN2#119-e | R2-2209250 | ZTE Corporation, Sanechips | NR_ext_to_71GHz-Core |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36.331 | 4869 | Rel-17 | MeasConfig corrections for above 71 GHz operation | F | 17.1.0 | RAN2#119-e | R2-2209249 | ZTE Corporation, Sanechips | NR_ext_to_71GHz-Core |

On the other hand, the Inter System Measurement Configuration IE was introduced per Rel-16 NR_SON_MDT-Core WI.

Considering that RAN plenary agreed  RAN2 TS36.331 CR (Cat F) and the rule, RAN3 need to find a way to align with RAN2 spec TS36.331. Possible Options:

- Option 1: use Rel-16 “NR_SON_MDT-Core” and “TEI17” as WI code, and CR category “F” for this CR.  The reason is the Inter System Measurement Configuration IE is for SON, and to align with TS36.331.

- Option 2: use TEI17. The main reason is to align with TS36.331.

- Option 3: use 71GHz WI code, but this maybe reject by RAN plenary (even RAN plenary agreed RAN2 Cat F TS36.331 CR).

- Any other options?

Q1: Please share your view on above options.

| Company | Comment |
| --- | --- |
| Nokia | Option 1 or Option 2. Option 1 may be better. |
| Qualcomm | Either Option 1 or Option 2 work. |

In case Option 1 or Option 2 is adopted, a possible modification (for Option 1) to the CR cover page is as below:

| Work item code: | NR_ext_to_71GHz NR_SON_MDT-Core, TEI17 |  |  |  | Date: |  | 2023-03-29 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Category: | F |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is a misalignment between TS36.413 and TS36.331 on SubcarrierSpacingSSB - TS36.413 defines it as ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, ...) - Rel-17 TS36.331 defines 2 new codepoints kHz480 and kHz960 that was introduced in TS36.331 per NR_ext_to_71GHz-Core WI subcarrierSpacingSSB-r15 ENUMERATED {kHz15, kHz30, kHz120, kHz240}, subcarrierSpacingSSB-r17 ENUMERATED {kHz480, kHz960} OPTIONAL, -- Need OR kHz60 is not used in TS36.331. TS36.413 does not include the new Rel-17 codepoints kHz480 and kHz960. |  |  |  |  |  |
| Summary of change: |  | The changes are as follows: - Clarify kHz60 is not used in the speficication - Add kHz480 and kHz960 - Correct the RRC reference Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has impact under functional point of view. The impact can be considered isolated because it only affect the SON feature related to NR using subcarrierSpacingSSB kHz480 and kHz960. |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies and inaccuracies remain in the S1AP specification, and not align with TS36.331. |  |  |  |  |  |

Q2: Please share your view on above modification to the CR cover page, and the TP text

| Company | Comment |
| --- | --- |
| Nokia | Ok for the above modification in case Option 1 is adopted. In case Option 2 is adopted, WI code need to be changed to TEI17. |
| Qualcomm | Same view as Nokia. |

In case the new codepoints kHz480 and kHz960 are not agreed, the CR is still needed to address the other 2 issues

- Clarify kHz60 is not used in the speficication

- Correct the RRC reference

Q3: in case the new codepoints kHz480 and kHz960 are not agreed, are you ok to agree the CR to address below 2 issues with TEI17 WI code

- Clarify kHz60 is not used in the speficication

- Correct the RRC reference

| Company | Comment |
| --- | --- |
| Nokia | Yes. The CR is still needed to address the above issues. |
| Qualcomm | CR is needed otherwise FR2-2 numerology will not be supported and causes mis-alignment with RAN2. |

Summary

Potential proposals:

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-231220, Correction for SubcarrierSpacingSSB (Nokia, Nokia Shanghai Bell, Qualcomm, Huawei)