3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231861

Online, 17th – 26th Apr, 2023

Agenda Item:	9.2.1

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion: CB: # 4_R17SONMDT

Document for:	Discussion

# Introduction

This is the summary for the following comeback:

CB: # 4_R17SONMDT

- Check the corrections proposed

- Approve the CRs if agreeable

(moderator - HW)

Since this is a CB of many different individual CRs, the proposed way forward is to split the discussion in two phases.

- In the 1st phase we check whether the issue is confirmed and whether there is any immediate feedback on the proposed solution. This will be captured in section 3 below. 
Deadline: UTC 13:00 Thursday 20th April

- In the 2nd phase we take the CRs that have agreement on the issue from 1st phase and work out the details. The details will be captured in draft CRs on the reflector and a proposed set of CRs to be agreed will be captured under section 2 below. During this work, I would appreciate if you update the SoD with any new requested tdoc numbers in sections 2 so that people will know wheter a new draft exist.
Deadline: UTC 8:00 Tuesday 25th April

# For the Chairman’s Notes

[to be completed after round 2]

The conclusion after the 2nd round is to propose the following CR for agreement

R3-231267 rev [in xxxg] – agreed

R3-231268 rev [in xxxg] – agreed

R3-231546 rev [in xxxg] – agreed

R3-231547 rev [in xxxg] – agreed

R3-231548 rev [in xxxg] – agreed

R3-231549 rev [in xxxg] – agreed

R3-231685 rev [in xxxg] – agreed

R3-231686 rev [in xxxg] – agreed

R3-231687 rev [in xxxg] – agreed

R3-231725 rev [in xxxg] – agreed

R3-231726 rev [in xxxg] – agreed

R3-231727 rev [in xxxg] – agreed

R3-231728 rev [in xxxg] – agreed

R3-231729 rev [in xxxg] – agreed

R3-231190 rev [in xxxg] – agreed

R3-231191 rev [in xxxg] – agreed

R3-231611 rev [in xxxg] – agreed

R3-231612 rev [in xxxg] – agreed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion, Phase 1

## XN: Mobility Change procedure

| R3-231267 | Correction on Mobility Change procedure (Huawei, Nokia, Nokia Shanghai Bell, DT, Qualcomm, ZTE, Orange, Vodafone) | CR1011r, TS 38.423 v16.13.0, Rel-16, Cat. F |
| --- | --- | --- |
| R3-231268 | Correction on Mobility Change procedure (Huawei, Nokia, Nokia Shanghai Bell, DT, Qualcomm, ZTE, Orange, Vodafone) | CR1012r, TS 38.423 v17.4.0, Rel-17, Cat. A |

| Company | Issue Confirmed? | Immediate comments? |
| --- | --- | --- |
| Ericsson | Yes | Adding a “shall” for an optional IE, without conditions in the procedure text, makes the CR functionally NBC. This is also an unusual way to correct such issue. Prefer to change ASN.1. |

Moderator Summary [phase 1]:

[to be completed]

## Xn and F1: RACH Report

| R3-231546 | Correction on RACH Report IE (Ericsson) | CR1022r, TS 38.423 v16.13.0, Rel-16, Cat. F |
| --- | --- | --- |
| R3-231547 | Correction on RACH Report IE (Ericsson) | CR1023r, TS 38.423 v17.4.0, Rel-17, Cat. A |
| R3-231548 | Correction on RACH Report IE (Ericsson) | CR1154r, TS 38.473 v16.13.0, Rel-16, Cat. F |
| R3-231549 | Correction on RACH Report IE (Ericsson) | CR1155r, TS 38.473 v17.4.0, Rel-17, Cat. A |

| Company | Issue Confirmed? | Immediate comments? |
| --- | --- | --- |
| Ericsson | Yes | The intention of the Access and Mobility Procedure is to signal on a per UE basis the RACH reports corresponding to a specific UE and to cells served by the target node. It seems we added by mistake the full list of reports, which contains also reports that may not belong to the target node. The correction therefore is quite simple and it consists of changing the RA-ReportList-r16 into RA-Report-r16. In this way the source node includes in the RACH Report List IE items that consist of a RA Report and a UE Assistant Identifier. |

Moderator Summary [phase 1]:

[to be completed]

## E1: RESOURCE STATUS FAILURE message

| R3-231685 | Correction on RESOURCE STATUS FAILURE message over E1 in Rel-16 (ZTE, China Telecom, CMCC, Lenovo, China Unicom) | CR0717r, TS 38.463 v16.13.0, Rel-16, Cat. F |
| --- | --- | --- |
| R3-231686 | Correction on RESOURCE STATUS FAILURE message over E1 in Rel-17 (ZTE, China Telecom, CMCC, Lenovo, China Unicom) | CR0061r, TS 37.483 v17.4.0, Rel-17, Cat. A |

| Company | Issue Confirmed? | Immediate comments? |
| --- | --- | --- |
| Ericsson | Yes | 1/ Semantics description can be removed 2/ It is true that the condition cannot be checked in the same message, therefore this IE should not be conditional. However, it should stay as optional, as it may not be available at reporting node (node2). There is no need to align with other specs. |

Moderator Summary [phase 1]:

[to be completed]

## NG: Event-based Reporting for Inter-system

| R3-231687 | Correction on Event-based Reporting for Inter-system Resource Status Request (ZTE, Nokia, Nokia Shanghai Bell, China Telecom, CMCC, Lenovo) | CR0979r, TS 38.413 v17.4.0, Rel-17, Cat. F |
| --- | --- | --- |

| Company | Issue Confirmed? | Immediate comments? |
| --- | --- | --- |
| Ericsson | No | For the case of Event based reporting, the Number of Measurement Reporting Levels IE is intended to limit the amount of reporting when the cell load crosses the levels indicated by the IE. If a code point “none” is added, it can lead to confusion, as it can be interpreted in two ways: 1) every change in cell load triggers the reporting; or 2) there is never a crossing of cell load. |

Moderator Summary [phase 1]:

[to be completed]

## Xn: Trace Activation

| R3-231725 | Correction on Trace Activation IE (Huawei, Deutsche Telekom, Orange, CMCC) | CR1032r, TS 38.423 v16.13.0, Rel-16, Cat. F |
| --- | --- | --- |
| R3-231726 | Correction on Trace Activation IE (Huawei, Deutsche Telekom, Orange, CMCC) | CR1033r, TS 38.423 v17.4.0, Rel-17, Cat. A |

| Company | Issue Confirmed? | Immediate comments? |
| --- | --- | --- |
| Ericsson | Yes | Agree with the CRs, Please add Ericsson to the cosourcing companies |

Moderator Summary [phase 1]:

[to be completed]

## Xn: Area Scope IE in MDT Configuration

| R3-231727 | Discussion on the presence of area scope in MDT configuration (Huawei, CMCC, China Unicom) | discussion |
| --- | --- | --- |
| R3-231728 | Correction on the Area Scope IE in MDT Configuration (Huawei, CMCC, China Unicom) | CR1034r, TS 38.423 v16.13.0, Rel-16, Cat. F |
| R3-231729 | Correction on the Area Scope IE in MDT Configuration (Huawei, CMCC, China Unicom) | CR1035r, TS 38.423 v17.4.0, Rel-17, Cat. A |

| Company | Issue Confirmed? | Immediate comments? |
| --- | --- | --- |
| Ericsson | No | TS 32.422 states that the Area Scope is an optional IE and that if it is not included, it means that the area scope is PLMN-wide. TS 28.622 defines the SQ (support qualifier), which indicates conditionality for implementing an attribute. I.e. whether or not the MnS Producer (OAM) supports the attribute and under what conditions (if any). This does not define whether a value must be present. Specifically for ‘areaScope’ in 28.622, clause 4.3.30 defines the attribute as ‘Conditional Optional’: Attribute Name; S; isReadable; isWritable; isInvariant; isNotifyable. areaScope; CO; T; T; F; T. with the condition defined as follows: Name; Definition. areaScope (support qualifier); This attribute shall be present if MDT is supported.. The above means the attribute must be implemented if MDT is supported. Whether or not an attribute must contain a value is defined in the attributes table. For ‘areaScope’ per clause 4.4.1, its properties indicate it can be null: Attribute Name; Documentation and Allowed Values; Properties. areaScope; It specifies the area where data shall be collected.. List of eNB/list of gNB/eNB/gNB for RLF or RCEF. List of cells/TA/LA/RA for signalling based or management based Logged MDT. List of cells for management based Immediate MDT. List of cells or Tracking Area for QMC. Cell, TA, LA, RA are mutually exclusive.; type: AreaScope multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True. Therefore, OAM may or may not include a value for the Area Scope IE in the MDT configuration. If a value is not included then: - Over NG, S1 and X2, the “PLMN Wide” choice will be selected in the Area Scope of MDT IE, which is a mandatory IE - Over Xn, the Area Scope of MDT IE is not included, because the IE is optional. In this case, TS32.422 already states that absence of Area Scope means “PLMN Wide”. Hence, there is no issue with current specifications. |

Moderator Summary [phase 1]:

[to be completed]

## E1: Resource Status Update

| R3-231190 | Alignment of the tabular and ASN.1 definitions for the Resource Status Update (Nokia, Nokia Shanghai Bell, Huawei, ZTE, Samsung) | CR0711r, TS 38.463 v16.13.0, Rel-16, Cat. F Move to 9.2.1 |
| --- | --- | --- |
| R3-231191 | Alignment of the tabular and ASN.1 definitions for the Resource Status Update (Nokia, Nokia Shanghai Bell, Huawei, ZTE, Samsung) | CR0055r, TS 37.483 v17.4.0, Rel-17, Cat. A Move to 9.2.1 |

| Company | Issue Confirmed? | Immediate comments? |
| --- | --- | --- |
| Ericsson | Yes | Non-backward compatibility from a protocol pov needs further discussion 😊 I would consider that changing gNB-CU-UP-Measurement-ID from optional to mandatory is NBC. If that’s the case, why not changing the criticality to reject in ASN.1, which makes more sense than M/ignore |

Moderator Summary [phase 1]:

[to be completed]

## F1 and Xn: NR-U metrics

| R3-231611 | F1AP Rel-17 correction for NR-U metrics (Ericsson, Qualcomm Incorporated, Samsung, Nokia, Nokia Shanghai Bell) | CR1158r, TS 38.473 v17.4.1, Rel-17, Cat. F |
| --- | --- | --- |
| R3-231612 | XnAP Rel-17 correction for NR-U metrics (Ericsson, Qualcomm Incorporated, Samsung, Nokia, Nokia Shanghai Bell) | CR1026r, TS 38.423 v17.4.0, Rel-17, Cat. F |

| Company | Issue Confirmed? | Immediate comments? |
| --- | --- | --- |
| Ericsson | Yes | Without the clarifications, the value 100 in the semantics description for Channel occupancy time percentage DL IE is not clear. |

Moderator Summary [phase 1]:

[to be completed]