3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231918

Electronic meeting, April 16th – 25th 2023

Agenda Item:	9.2.4

Source:	CATT

Title:	MultiTrace - Summary of offline discussion

Document for:	Discussion and Decision

# 1	Introduction

The scope of the email discussion has been captured as followed:

CB: # 51_MultiTrace

- Confirm that the new Trace Session will be started,  then RAN3 can discuss whether anything needs to be updated to handle the previous Trace Session

- The relationship with MDT?

(moderator - CATT)

Summary of offline disc R3-231918

For the first round discussion, please provide your comments before 20:00 AM, UTC, 21st April, Friday.

In the second round, we could discuss on CR and LSout

# 2	For the chairman’s notes

To be captured later

# 3	Email discussion – 1st Round

Principle on handling of multiple trace messages

Currently, there are some ambugities on the behaviour of NG-RAN node for the case that it receives a second Trace Start message while there is already an active trace session ongoing[1].In 38.413,it is described that NG-RAN node would always start a new trace session when receving a new Trace Start message as described in 32.422.However, in 32.422,it is described that the NG-RAN node would ignore the second Trace Activation request message if the TR of the second Trace session is the same with the existing one and there is no description on the behaviour of NG-RAN node if TR of the second Trace Session is different with the existing one.

During the online discussion, it seems most companies think that NG-RAN node should start the new Trace session and deactive the existing one when it receives the second activation request of a Trace Session whose TR is different with the exsiting one.So,the moderator would like to have the following proposal：

Propsoal 1: NG-RAN node should start the new Trace session and deactive the existing one when it receives the second activation request of a Trace Session whose TR is different with the exsiting one

Question 1: Do you agree with above proposal?If not,please provide you comments.

| Company | Agree | Comment |
| --- | --- | --- |
| CATT | Yes |  |

Update on 38.413

If companies could reach consens on question 1, then it seems necessary to clarify in the specification that NG-RAN node would deactivate the exsiting Trace session if it receives a second Trace Start message whose TR is different with the existing one.Otherwise,there would be two trace sessions ongoing for one specific UE which is not supported in Rel-17.

The proposal from moderator is to update the stage 3 spec on Trace Start procedure as below:

The AMF initiates the procedure by sending a TRACE START message. Upon reception of the TRACE START message, the NG-RAN node shall initiate the requested trace session as described in TS 32.422 [11] and deactivate the ongoing trace session whose TR is different with the new received one, if any.

Propsoal 2: It is proposed to update the stage 3 spec as above.

Question 2: Do you agree with the proposed update on 38.413? Any comments?

| Company | Agree or not | Comments |
| --- | --- | --- |
| CATT | Yes |  |

LS to SA5

If RAN3 could reach consensus on question 1 and 2,then the LS to SA5 is just to inform SA5 on the agreement and also attach the related CR to guarantee the aligment between RAN3 and SA5.

Proposal 3: In the LS to SA5, inform SA5 on the agreement reached in RAN3 and also attach the related CR.

Question 3: Do you agree with the above proposal ?

| Company | Agree or not | Comments |
| --- | --- | --- |
| CATT | Yes |  |

Relationship with MDT

During the online discussion, one question raised is the relationship between Trace and MDT.Since  MDT is initially discussed under the framework of Trace procedure in both RAN3 and SA5.Also,MDT configuration is included in Trace Activation IE in 38.413.So,from moderator point of view,the above issue and principle discussed on trace procedure also applied to MDT i.e. if NG-RAN node receives a second MDT configraiton whose TR is different with the existing one,then the NG-RAN node would initiate the MDT as requested by the second one and deactive the existing MDT configuration.And another reason is that it is not supported to transfer two MDT configurations during HO procedure,so there could be only one active MDT configuration for one UE.So,unless there is some enhancement,multiple MDT configurations could not be supported in Rel-17.

Proposal4: In Rel-17, the principle discussed above also applied to MDT i.e. if NG-RAN node receives a second MDT configraiton whose TR is different with the existing one, then the NG-RAN node would initiate the MDT as requested by the second one and deactive the existing MDT configuration.

Question: Companies are invited to provide views on above proposal

| Company | Agree or not | Comments |
| --- | --- | --- |
| CATT | Yes |  |

# 3	Email discussion – 2nd Round

Discuss on CR and LSout

# 4	Conclusion

Result of the email discussion to be captured here

# 5	References

[11] R3-231281 Discussion on multiple trace activation CATT,ZTE, Huawei,CMCC,China Telecom