3GPP TSG-RAN WG3 #119bis-e							R3-231865

17th – 26th April 2023

Agenda Item:	9.2.4

Source:	China Telecom(moderator)

Title:	Summary of Offline Discussion on CB: # 8_NewAttribute

Document for:	Approval

# Introduction

This contribution is to kick off the following discussion:

CB: # 8_NewAttribute

- Clarify the use case and usage of this "Only Resource Coordination" attribute

- Provide reply LS to SA5

(moderator - CT)

Note: Two rounds of discussion are planned in this meeting.

The first round email discussion plan to be at the end of Thursday of 1st week.(Thursday UTC 18:00 2023-4-20).

The second round email discussion plan to be end before deadline of email discussion 2nd week, if needed. (Tuesday, 8:00 UTC, 2023-4-25)

# For the Chairman’s Notes

Agreement for discussion:

# First Round Discussion

RAN3 had discussed the scenario and solution for source coordination between LTE and NR SA in Release 17, and made the following agreements in RAN3#117e meeting:

Agree to introduce a new attribute “Only Resource Coordination” in Neighbour Cell Relation to support resource coordination between LTE and NR SA sites.

RAN3 also endorsed a CR to TS38.300, see R3-225158, where the new “Resource Coordination Only” attribute was added to the Neighbour Cell Relation and defined as follows:

- Resource Coordination Only: If checked, the Neighbour Cell Relation shall use the X2 interface instance only to coordinate resources between the E-UTRA cell and the NR cell.

Based on the above agreement and endorsed CR, a LS in R3-225206 on the conclusion of RAN3 was send to SA5. In this meeting, the Reply LS from SA5 in S5-233142 was received and the following questions need to be clarified by RAN3:

- Question 1: use case that motivates this attribute:

- Question 2: which network function is supposed to set this attribute (e.g. by the neighboring eNB/gNB via X2, or by human operator via OAM, in gNB or eNB)

- Question 3: effect of this attribute on the system once it was set.

In this meeting, 2 discussion papers and 2 draft reply LS were

### Answer to question 1

Three companies [2][4] agree to provide the following text for this answer:

A1:the attribute is motivated by the need to coordinate resources between co-channel sharing E-UTRA and NR cells, where E-UTRA cells are served by an E-UTRAN eNB in one site and NR cells are served by an NG-RAN gNB and an E-UTRAN en-gNB collocated at a different site. Under this deployment scenario, the assumption made by RAN3 is that the X2: E-UTRA – NR Cell Resource Coordination procedure can lead to resource coordination between E-UTRAN cells of the eNB and NR cells of the en-gNB and that, by means of implementation, the gNB co-located with the en-gNB would also be able to coordinate its resources with the E-UTRAN cells.

In addition, [2] also provide the following text:

A2: Given that the existing attribute, i.e, isENDCAllowed and isHOAllowed, could not be reused in this scenario, a new NRT attribute “Only Resource Coordination” then is agreed for this purpose while not activating other EN-DC X2 functions.

Based on above, moderator thinks at least the A1 can be captured in LS.

During the offline discussion in SA5, some companies had concerns on whether the existing attributes can be reused. Therefore, moderator thinks it is also need to capture A2 in LS.

Q1: Please provide your view on the answer.

| Company | Which answer(s) do you agree? A1,A1+A2, or other? | Comment |
| --- | --- | --- |

Conclusion：

### Answer to question 2

Three companies [2][4] provide the following answer:

A3: RAN3 envisaged the new attribute to be set by the OAM system at the eNB and at the en-gNB. In case of split RAN architecture, the attribute should be set at the CU-CP node of each of the eNB and en-gNB terminating the EN-DC X2 interface. Whether this is the consequence of manual configuration or an automated configuration is out of RAN3 scope.

One company [5] provides the following answer:

A4: The network function is supposed by human operator via OAM, the function apply for eNB and gNB.

All companies agree that the network function is supposed by human operator via OAM. Therefore, as a moderator, the A3 can be captured in LS.

Q2: Do you agree to capture A3 in LS?

| Company | Do you agree the answer. | Comment |
| --- | --- | --- |

### Answer to question 3

Three companies [2][4] provide the following answer:

A5: If the “Resource Coordination Only” attribute is checked, the eNB and en-gNB between which the EN-DC X2 interface is established shall use the X2 interface instance only to run basic procedures, e.g. EN-DC X2 Setup, and to coordinate resources between the E-UTRA cell and the NR cell. No more effects on the system were foreseen by RAN3.

One company [5] provides the following example to make SA5 to understand the effect in LS

A6: Example: An operator has deployed an LTE cell on 2 GHz, as well as an NR cell (TCI 1) using the same spectrum in the same area. The operator has also deployed NR capacity booster cells (TCI 2,TCI 3,TCI 4) on 28 GHz in the same area. The operator has chosen to operate NR cell 1 in stand-alone mode (SA mode).

Since TC1 will have interference impact on LTE cell, it is necessary to setup an EN-DC X2 connection between the TC1 (NG node ) and LTE cell( LTE node). While the EN-DC X2 connection only used for Resource Coordination purpose , for example ,to transfer resource status in order to mitigate the interference.

Q3: Please provide your view on the answer.

| Company | Which answer(s) do you agree? A5,A6,A5+A6, or other? | Comment |
| --- | --- | --- |

### Any other issue left

Q5: Please provide your view if anything missing.

| Company | Comments |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-231124 Reply LS on introduction of a new attribute “Only Resource Coordination” to support source coordination between LTE and NR SA (SA5, China telecom)

- R3-231579 [DRAFT] Reply LS on introduction of a new attribute "Only Resource Coordination" to support source coordination between LTE and NR SA (China Telecom, Huawei)

- R3-231580 Clarification of Use Case and the Effect of the new Attribute “Only Resource Coordination” (China Telecom, Huawei)

- R3-231601 Discussion on new attribute “Only Resource Coordination (Ericsson)

- R3-231702 Reply LS on introduction of a new attribute to support source coordination between LTE and NR SA (ZTE)