3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231993

- E-meeting, April 17th – April 26th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.422 | CR | 0016 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on TNLA Addition/Removal/Modification procedures (XnAP) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Nokia, Nokia Shanghai Bell, Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2023-03-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For SCTP association used for a single UE-associated signalling, it should not be changed unless the TNL binding of this association is updated, or this association is removed or failed, the current spec doesn’t reflec the latter case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that, for a single UE-associated signalling, the NG RAN nodes shall use one SCTP association and one SCTP stream and the SCTP association/stream should not be changed during the communication of the UE-associated signalling unless TNL binding update is performed or, this TNL association is deleted or failed . Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has limited impact under functional point of view. The CR clarifies the SCTP association can be changed when current SCTP association is failed or removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If a TNLA for UE associated signalling is removed or failed while UEs are still served by the TNLA, the receiver NG-RAN node is not able to signal any XnAP messages for these UEs because it is not able to reselect a TNLA for the affected Ues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.412 CR 0021 TS38.472 CR 0027 TS37.482 CR 0003 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev -: To update with impact analysis, wording of the change and TEI code. |  |  |  |  |  |  |  |  |

- First Change

Between one pair of two NG-RAN nodes:

-	a single pair of stream identifiers shall be reserved over an SCTP association for the sole use of XnAP elementary procedures that utilize non UE-associated signalling.

-	At least one pair of stream identifiers over one or several SCTP associations shall be reserved for the sole use of XnAP elementary procedures that utilize UE-associated signallings. However, a few pairs (i.e. more than one) should be reserved.

-	For a single UE-associated signalling, the NG RAN nodes shall use one SCTP association and one SCTP stream and the SCTP association/stream should not be changed during the communication of the UE-associated signalling unless current SCTP association is failed or removed, or TNL binding update is performed.

- End of Changes