3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231740

Electronic meeting, Apr. 17th – Apr. 26th, 2023

Title: 	(TPs for SON BLCR TS 38.401): naming update for RA report

Source: 	Huawei

Agenda item:	10.2.3

Document Type:	Other

# 1 Introduction

This document contains a TP for SON BLCR for 38.401 to reflect the agreements in RAN3 #119bis-e meeting.

# 2  Annex – TP for SON BLCR for TS 38.401

## 7.5	RACH Optimisation Function

The RACH Optimization Function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, RACH configuration conflict detection and resolution function is located at the gNB-DU. To perform RACH optimisation at gNB-DU, gNB-CU sends the RA report reported by the UE to gNB-DU via F1AP signalling. The gNB-DU signals the PRACH configuration per-cell to gNB-CU. The gNB-CU may forward a limited set of neighbour cell’s PRACH configurations received from neighbour gNBs and other gNB-DUs to the gNB-DU to resolve the configuration conflict.

RA Report retrieval

When a UE performs successful random access attempts which are only known by the gNB-DU (e.g., beam failure recovery, UL synchronization issue, scheduling request failure, no PUCCH resource available), the gNB-DU may inform the gNB-CU about the occurrences of successful random access procedures in the gNB-DU via a RACH Indication. The gNB-CU may then retrieve the RA Report from the UE(s) based on the RACH indication received via F1AP signalling from gNB-DU.