3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231740

Electronic meeting, Apr. 17th – Apr. 26th, 2023

Title: 	(TPs for SON BLCR for TS 38.30): RA report update

Source: 	Huawei

Agenda item:	10.2.3

Document Type:	Other

# 1 Introduction

This document contains a TP for SON BLCR for 38.300 to reflect the agreements in RAN3 #119bis-e meeting.

# 2 Annex – TP for SON BLCR for TS 38.300

### 15.5.3	Support for RACH Optimization

RACH optimization is supported by UE reported information made available at the NG RAN node as specified in TS 38.331 [12] and by PRACH parameters exchange between NG RAN nodes.

The contents of the RA report comprise of the following:

-	Contention detection indication per RACH attempt;

-	Indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts;

-	Indication whether the selected SSB is above or below the configured RSRP threshold per RACH attempt;

-	2-step RACH information as specified in clause 5.7.10.4 of TS 38.331 [12].

The UE operating in NR-DC, may also support SgNB RA report.

RA Report retrieval in case of dual connectivity:

In MR-DC, when UE performs a successful random-access procedure in the SN, the SN may inform the potential availability of RA Report in the UE to the MN via a RACH indication. The MN may then retrieve the RA Report from the UE based on the RACH indication received via XnAP signalling from SN.