3GPP TSG-RAN3 Meeting #119-bis-e	R3-23xxxx

Online, 17th April - 26th April 2023

3GPP TSG RAN WG1 #112		                                                                 R1-2302130

Athens, Greece, February 27th – March 3rd, 2023

Title:	Reply LS on RB set configuration

Response to:	R1-2300006(R3-226781)

Release:	Rel-17

Work Item:	NR_IAB_enh-Core

Source:	RAN1

To:	RAN3

Cc:

Attachment:	-

Contact Person:

Name:	Yuzhou Hu

E-mail Address:	huyuzhou@zte.com.cn

# Overall description

RAN1 thanks RAN3 for the LS R1-2300006(R3-226781) on RB set configuration where the following question was asked:

Question: RAN3 has uncertainty on how to define the start RB index for the Rel-17 IAB-DU HSNA resource configuration of an IAB-DU cell. RAN3 has two different understandings: - Understanding 1: the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the SCS-specific offset defined in the NR Carrier List IE based on the SCS included in the RB Set Configuration IE. - Understanding 2: the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the smallest SCS-specific frequency offset across all numerologies provided by the NR Carrier List IE for this cell. Note that in this understanding, the start RB index does not depend on the SCS value included in the RB Set Configuration IE in TS 38.473. In both understandings, the start RB is aligned with point A in case the NR Carrier List IE is not present. RAN3 kindly asks RAN1 to provide clarification on which of the two understandings is correct.

Answer: RAN1 has discussed the above question and would like to clarify that the modified understanding 2 as below is correct:

- The start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is the RB index of the lowest common RB with the reference SCS, which overlaps with the lowest usable RB  across all SCS-specific carriers provided by the NR Carrier List IE for this cell.

# Actions:

To RAN3:

RAN1 respectfully requests RAN3 to take the above information into account.

# Date of Next TSG-RAN1 Meetings:

RAN1#112bis-e			17th -26th Apr 2023	   			E-Meeting

RAN1#113			22th -26th May 2023	   			Incheon , KR