3GPP TSG-RAN Meeting #120	R3-232653

22th – 26th May 2023

Incheon, South Korea

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0061 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RESOURCE STATUS FAILURE message over E1 in Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, CMCC, Lenovo, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The semantic description of IE “Cause” is ambiguous, since the “Complete Failure Cause Information IE” is not defined in this specification. - The presence of the IE “gNB-CU-UP Measurement ID” in the E1AP RESOURCE STATUS FAILURE message is conditional in the IE tabular, while it is optional in ASN.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the unnecessary semantic description of IE “Cause” in the E1AP RESOURCE STATUS FAILURE message. - Change the presence of IE “gNB-CU-UP Measurement ID” in the E1AP RESOURCE STATUS FAILURE message as “optional” in IE tabular. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Ambiguous description of the semantic description. - Misalignment on the presence of IE “gNB-CU-UP Measurement ID” in the E1AP RESOURCE STATUS FAILURE message between IE tabular and ASN.1. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol point of view. The impact can be considered isolated because the change affects only the Resource Status Reporting Initiation function. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-231686 Rev1: Withdraw the correction on the presence of “gNB-CU-UP Measurement ID” IE, only changing it as “optional” in IE tabular. Rev2: Re-submission in RAN3#120. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< START OF CHANGE >>>>>>>>>>>>>>>>>>>>

9.2.1.21	RESOURCE STATUS FAILURE

This message is sent by the gNB-CU-UP to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | O |  |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics |  | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>