3GPP TSG-RAN3 Meeting # 120	R3-233071 (revision of R3-231610)

# Incheon, Korea, May 22nd - 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1157 | rev | 1 | Current version: | 17.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on F1AP for L2 U2N Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V., ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The Uu RLC Channel ID IE defined in TS 38.473 is not consistent with the Uu-RelayRLC-ChannelID IE defined in TS 38.331. Uu RLC Channel ID is defiend as BIT STRING (SIZE(16)), while Uu-RelayRLC-ChannelID is defined as INTEGER (1..maxLC-ID), where maxLC-ID is equal to 32. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the definition of Uu RLC Channel ID, adding a value range restriction which limits the value of the Uu RLC Channel ID to the range from 1 to 32. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistent definition of Uu RLC Channel ID will cause value overflow when downcasting a value of Uu RLC Channel ID greater than 32 to Uu-RelayRLC-ChannelID in the RRC configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.266 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

#### 9.3.1.266	Uu RLC Channel ID

This IE uniquely identifies a Uu Relay RLC channel for a L2 U2N Relay UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uu RLC Channel ID | M | 1..<maxnoofUuRLCChannels> | BIT STRING (SIZE (16)) |  |

End of Change