3GPP TSG-RAN WG3 Meeting #120	R3-233092

# Incheon, KR, May 22th – 26th 2023

Agenda Item:	10.2.2

Source:	Ericsson

Title:	(TP for SON BL CR for TS 36.300, 37.340) SON enhancements for Mobility Robustness Optimization

Document for:	Discussion and Decision

# 1	Introduction

During previous meetings, a stage-2 definition of Mobility Failure during Voice Fallback and its detection mechanism were agreed. In this paper we look into further changes needed to accommodate for Mobility Failure during Voice Fallback given the agreed definition.

MRO for Fast MCG Recovery was also discussed. In this contribution we will continue the discussion on the agreed scenarios and continue analyzing its RAN3 impacts. The forwarding of information about Fast MCG Recovery Failure to the SN (e.g. with RLF Report) will also be discussed.

The last topic discussed last meeting was MRO for CPAC. One remaining aspect for this topic was captured as to be continued during previous meetings: whether/how to support UHI for CPAC. A stage-2 solution is proposed, as well as some editorial changes on the new failure types definitions.

# 2	Discussion

## 2.1 MRO for IRAT Handover due to Voice Fallback

### 2.1.1 Background

In RAN3 #119-bis-e, a TP for SON BL CR 38.300 with the detection mechanism for Mobility Failure during Voice Fallback in NR [2] was agreed. The TP also improves the previously agreed failure definition for Mobility Failure during Voice Fallback.

Failure definition [2]:

Inter-system Mobility Failure during Voice Fallback: an RLF occurs shortly after a successful handover triggered due to Voice Fallback, or a failure occurs during an handover triggered due to Voice Fallback, from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node; the UE attempts to re-connect to a cell belonging to an E-UTRAN node, or an NG-RAN node.

Detection Mechanism [2]:

Inter-system Mobility Failure during Voice Fallback: in case the connection failure occurs during an inter-system handover for voice fall back from NR, the RLF Report from the UE includes a voice fallback indication.

Editor’s note 1: the name of the indication needs be refined when details are agreed in RAN2.

Editor’s note 2 The following detection mechanism is FFS: In case the connection failure occurs in the target LTE cell after a recent inter-system handover for voice fall back (i.e., the UE reported timer is smaller than the configured threshold, e.g., Tstore_UE_cntxt)”

It can be seen that the problem definition covers two different variants of the failure, namely

- Radio Link Failure after Voice Fallback: an RLF occurs shortly after a successful handover triggered due to Voice Fallback, and

- Mobility from NR Failure during Voice Fallback: a failure occurs during a handover triggered due to Voice Fallback.

It should be noticed that while Mobility from NR Failure during Voice Fallback would occur in NG-RAN and generate an NR RLF report, Radio Link Failure after Voice Fallback would occur in E-UTRAN and generate an LTE RLF report.

- Observation 1.1	Mobility from NR Failure during Voice Fallback would occur in NG-RAN and generate an NR RLF report.

- Observation 1.2	Radio Link Failure after Voice Fallback would occur in E-UTRAN and generate an LTE RLF report.

It can further be seen that the agreed detection mechanism in [2] can be used to detect Mobility from NR Failure during Voice Fallback, while the detection mechanism for Radio Link Failure after Voice Fallback is still FFS. The detection of RLF after voice fallback is further discussed in Section 2.1.2 below.

### 2.1.2 Detection of Radio Link Failure after Voice Fallback

In case of Radio Link Failure after Voice Fallback, for the network to become aware that a failure occurred shortly after mobility from NR due to voice fallback (to e.g. distinguish it from failures due to coverage), this needs to be indicated to the network. Since an LTE RLF Report is anyway generated, it would make sense to, in analogy with the NR RLF Report, also include a voice fallback indication in the LTE RLF Report.

- Proposal 1.1	Use the LTE RLF report to inform the network that a RLF occurred shortly after a successful handover triggered due to Voice Fallback.

- Proposal 1.2	Send an LS to RAN2 asking them to agree on how the network can be informed that a RLF occurred shortly after a successful handover triggered due to Voice Fallback, using the RLF report.

In the following, we assume that the network can become aware of a failure occurring shortly after mobility from NR due to voice fallback through the LTE RLF report.

In general, when RLF happens in an E-UTRA cell, the UE makes the LTE RLF Report available to NG-RAN nodes and eNB(s). If the LTE RLF Report is reported to a NG-RAN node, and the last serving node is an E-UTRAN node, the NG-RAN node may transfer it to the E-UTRAN node by triggering the Uplink RAN configuration transfer procedure over NG [3]. The E-UTRAN node can then detect that a failure occurred shortly after a successful handover triggered due to voice fallback.

In order for this to work however, the failure definition and detection mechanism needs to be introduced in the E-UTRAN specification TS 36.300. In Appendix A: TP to SON BL CR to TS 36.300, a text proposal for 36.300 including the problem definition and detection for Voice Fallback Triggered Handover Failure is given.

- Proposal 1.3	Agree on the text proposal to TS 36.300 on introducing the Voice Fallback Triggered Handover Failure type, and the detection of the same, given in Appendix A of this contribution.

With the introduction of Proposal 1.3, the following editor’s note 2: “The following detection mechanism is FFS: In case the connection failure occurs in the target LTE cell after a recent inter-system handover for voice fall back (i.e., the UE reported timer is smaller than the configured threshold, e.g., Tstore_UE_cntxt)” regarding when the connection failure occurs in the target LTE cell after a recent inter-system handover for voice fall back can be removed from 38.300.

- Proposal 1.4	Remove the editor’s note 2 from the Inter-system Mobility Failure during Voice Fallback failure detection mechanism in TS 38.300 BL CR.

### 2.1.3 Need for additional information exchange

In the case of RLF after voice fallback, i.e. when a RLF occurs shortly after a successful IRAT handover triggered due to voice fallback, the IRAT handover cannot be considered as too early. The NG-RAN node had to perform the handover to establish the call, even if radio conditions in source cell were good. There are no parameters to adjust to change the timing of the handover in that case.

Therefore, the RLF indication could be seen as an indication that the target E-UTRA cell coverage needs to be improved. Hence, Radio Link Failure after Voice Fallback can be detected by E-UTRAN, and any actions to improve the situation can also be taken in E-UTRAN. There is no need to inform NG-RAN.

- Observation 1.3	There is no need to inform NG-RAN about a Radio Link Failure after Voice Fallback.

For the case of Mobility from NR Failure during Voice Fallback, as earlier discussed this would occur in NG-RAN and generate an NR RLF report. Today, reporting of an NR RLF report to an E-UTRA node is not supported. Instead, a UE experiencing failure in NR and generating an NR RLF report will indicate the existence of the report to NR when coming back to NR.

- Observation 1.4	Reporting of an NR RLF report to an E-UTRA node is not supported, but the NR RLF report will be reported to an NG-RAN node when the UE comes back to NR.

The same principle should be applied when an NR RLF report is generated due to Mobility from NR Failure during Voice Fallback. Since the source PCell is already included in the RLF report, the NG-RAN node receiving the RLF report can forward the information to the node hosting the source PCell by using the FAILURE INDICATION message over Xn or by using the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG. This is already supported in specifications and requires no changes.

- Observation 1.5	The forwarding of the NR RLF information to the source node is already supported in specifications and requires no changes.

Considering Observation 1.3-1.5 it can be seen that no additional messages for reporting and forwarding of the RLF report or information therein are needed to support MRO for voice fallback.

- Proposal 1.5	No additional reporting and forwarding of the RLF report or information therein is needed to support MRO for voice fallback.

## 2.2 Fast MCG recovery

After many meetings discussing scenarios, there is only one remaining case to be agreed or abandoned:

- Case c: Fast recovery near failure case, i.e. UE receives the response message from MN via SN while T316 is running which almost expires but not yet.

Case c/ could be interesting to optimize T316 value. If T316 is too short, fast MCG failure may occur. If T316 is too big, and in case the MN does not take any action, there will be an addition delay before UE attempts RRC re-establishment. Therefore, this use-case may be considered.

Proposal 2.1: RAN3 to define solutions to support case c/

Before discussing the information needed by the network to optimize T316, it could be interesting to understand how the UE will report it. As for now, at MCG Failure, the RLF Report is generated in the UE. But the UE deletes it if fast MCG recovery is successful. An easy solution, to avoid generating a new report, would be to not delete it, and add the information needed for Fast MCG Recovery optimization in the RLF Report. This would not be an extra burden to the UE, as the RLF Report is generated in any case. And the network can benefit from the information contained in this report, and linked to the MCG Failure. This would also simplify the configuration of the UE participating to T316 optimization (e.g. no need for a new threshold or a new report).

Proposal 2.2: The RLF Report generated after the MCG failure is not deleted by the UE in case fast MCG recovery is successful

In order to support T316 optimization, the network needs to understand its value at the time of the successful recovery (e.g. if it has almost expired). It is therefore proposed to add the T316 elapsed time (i.e. recovery time if recovery was successful, configured value if not) to the RLF Report. This will of course be reported only if T316 has been configured.

Proposal 2.3: Add the T316 elapsed time to the RLF Report

As for all the SON reports generated by the UE, there is no mandate to keep the UE Context for 48h. This includes T316 configured value for that UE. Therefore, and to understand how close the UE was to RRC re-establishment, it is proposed to add the T316 configured value in the RLF Report. In case of fast MCG Recovery Failure, T316 elapsed time and the T316 configured value are the same. Therefore, there is no need to report the T316 configured value if it has expired. It should be reported only in case fast MCG Recovery was successful.

Proposal 2.4: Add the T316 configured value in the RLF Report, in case fast MCG Recovery was successful (i.e. T316 was running but did not expire)

During RAN3#119, it was agreed to add “SCG was deactivated or other cases that SCG is not available” as a possible fast MCG recovery failure cause. This agreement was sent to RAN2, and it is now up to RAN2 to decide how to signal it to the network (e.g. using RLF Report). This information (e.g. that SCG was deactivated at the time of failure) would also be useful in the SN, for example to not deactivate SCG in some cases, so that MCG Failure can be attempted in case of RLF in MCG. One possible way to achieve this goal would be to forward the RLF report (if RAN2 decides to add this information into the RLF Report) to the SN. But most of the information contained in the RLF Report will not be useful for the SN. Therefore, and to avoid signalling useless information, it is proposed to send only the SCG Status (i.e. SCG was deactivated or other cases that SCG is not available) to the SN in case SCG status was part of the problem leading to the failure.

Proposal 2.5: SCG status is needed in SN in case of fast MCG recovery failure due to SCG being deactivated or not available

So far RAN3 agreed on the following SCG failure type to be reported by the UE:

- t310-Expiry

- randomAccessProblem

- rlc-MaxNumRetx

However, there are many other SCG Failure types referenced in TS 38.331. All these failure types are included in SCGFailureInformation message and could not be sent by a UE to the network in the case of MCG failure simultaneous with the SCG failure. However, this information is of use to the network for failure analysis and DC operation optimization. Therefore, RAN3 shall agree to add any possible SCG failure type contained in RRC specification as possible failure types reported by the UE, in case of fast MCG recovery failure.

Proposal 2.6: Add any possible SCG failure type contained in RRC specification as possible failure types reported by the UE, in case of fast MCG recovery failure

## 2.3 MRO for CPAC

### 2.3.1 UHI for CPAC

During RAN3#118, the following topic was captured as “to be continued”:

whether/how to support UHI for CPAC

Also, during RAN3#119bis-e, the following understanding of the issue to be solved was captured:

During CPAC configuration, the value of the Time Stay IE for the source PSCell UHI, sent in S-NODE ADDITION REQUEST message, does not reflect the exact time the UE stayed in the source PSCell

Two solutions, as described in [1], were discussed. Solution A was aiming at stage-2 only impact, while stage-3 solution was described as solution B. At the end, it seemed that solution A was preferred by a majority of companies:

#### Solution A:

A similar solution was already agreed for UHI during CHO and this solution follows the same principle. This would consist in adding a note in TS 37.340 to indicate that the target SN needs to calculate the amount of time since arrival of the PSCell change request to when the UE performs RRC establishment. This will ensure that the SN updates the value of time stay in the latest PSCell entry (i.e. source PSCell) to accurately reflect actual time spent in the cell.

To accomplish this, it is required to add the note shown below in the section 10.5 of 37.340 v17.3.0:

- EN-DC: MN initiated conditional SN Change in Step 7.

- EN-DC: SN initiated conditional SN Change in step 12.

- MR-DC: MN initiated conditional SN Change in Step 7.

- MR-DC: SN initiated conditional SN Change in step 12.

Add the following NOTE:

NOTE: The target SN updates the time UE stayed in cell of the latest PSCell entry (the source PSCell) when the UE successfully accesses a candidate cell of the target SN. The updated value of the time UE stayed in source PSCell is equal to the value received from the MN during SN addition plus the time from receiving the SN addition request to completion of the Random Access Procedure with the UE.

Observation 3.1: Notes can be added to TS 37.340 to instruct the target SN to update time stayed IE in SCG UHI in case of conditional PSCell change

At the end of the meeting, the TP was almost agreed, but some companies wanted to continue the discussion on the details. Therefore, it is now proposed to agree solution A and the associated TP for TS 37.340

Proposal 3.1: Agree solution A for UHI for CPAC and notes in TS 37.340

### 2.3.1 New failure definitions

During RAN3#119bis-e, new failure definitions were defined in TS 37.340: Conditional PSCell addition or change failure. The CPC/CPA Execution to wrong PSCell failure definition has been split into 2 failure cases:

- Wrong target PSCell selection at the (candidate) target SN

- Wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA

For better readability, the title of the failure case should be added at the front of the definition, as proposed in Annex B.

Proposal 3.2: Agree editorial changes in BL CR for TS 37.340

Proposal 3.3: Agree to the TP in the Appendix B

# 3	Conclusion

- Observation 1.1	Mobility from NR Failure during Voice Fallback would occur in NG-RAN and generate an NR RLF report.

- Observation 1.2	Radio Link Failure after Voice Fallback would occur in E-UTRAN and generate an LTE RLF report.

- Proposal 1.1	Use the LTE RLF report to inform the network that a RLF occurred shortly after a successful handover triggered due to Voice Fallback.

- Proposal 1.2	Send an LS to RAN2 asking them to agree on how the network can be informed that a RLF occurred shortly after a successful handover triggered due to Voice Fallback, using the RLF report.

- Proposal 1.3	Agree on the text proposal to TS 36.300 on introducing the Voice Fallback Triggered Handover Failure type, and the detection of the same, given in Appendix A of this contribution.

- Proposal 1.4	Remove the editor’s note 2 from the Inter-system Mobility Failure during Voice Fallback failure detection mechanism in TS 38.300 BL CR.

- Observation 1.3	There is no need to inform NG-RAN about a Radio Link Failure after Voice Fallback.

- Observation 1.4	Reporting of an NR RLF report to an E-UTRA node is not supported, but the NR RLF report will be reported to an NG-RAN node when the UE comes back to NR.

- Observation 1.5	The forwarding of the NR RLF information to the source node is already supported in specifications and requires no changes.

- Proposal 1.5	No additional reporting and forwarding of the RLF report or information therein is needed to support MRO for voice fallback.

On Fast MCG Recovery:

Proposal 2.1: RAN3 to define solutions to support case c/

Proposal 2.2: The RLF Report generated after the MCG failure is not deleted by the UE in case fast MCG recovery is successful

Proposal 2.3: Add the T316 elapsed time to the RLF Report

Proposal 2.4: Add the T316 configured value in the RLF Report, in case fast MCG Recovery was successful (i.e. T316 was running but did not expire)

Proposal 2.5: SCG status is needed in SN in case of fast MCG recovery failure due to SCG being deactivated or not available

Proposal 2.6: Add any possible SCG failure type contained in RRC specification as possible failure types reported by the UE, in case of fast MCG recovery failure

On MRO for CPAC:

Proposal 3.1: Agree solution A for UHI for CPAC and notes in TS 37.340

Proposal 3.2: Agree editorial changes in BL CR for TS 37.340

Proposal 3.3: Agree to the TP in the Appendix B

# References

[1] R3-231585 - (TP for SON BL CR for TS 36.300, TS 38.300, TS 38.423) SON enhancements for MRO (Ericsson)

[2] R3-232151 - (TP for SON BL CR for TS 38.300) SON enhancements for Mobility Robustness (Ericsson)

[3] 3GPP TS 38.300 V17.4.0 (2023-03), NR and NG-RAN Overall Description; Stage 2

# Appendix A: TP to SON BL CR to TS 36.300

#### 22.4.2.2a	Connection failure due to inter-RAT mobility

One of the functions of Mobility Robustness Optimisation is to detect connection failures that occurred due to Too Early or Too Late inter-RAT handovers or inter-RAT Mobility Failure during Voice Fallback. These problems are defined as follows:

-	[Too Late Inter-RAT Handover] An RLF occurs after the UE has stayed in an E-UTRAN cell for a long period of time; the UE attempts to re-connect to a UTRAN cell.

-	[Too Early Inter-RAT Handover] An RLF occurs shortly after a successful handover from a UTRAN cell to a target cell in E-UTRAN; the UE attempts to re-connect to the source cell or to another UTRAN cell.

-	[Inter-RAT Mobility Failure during Voice Fallback] An RLF occurs shortly after a successful handover triggered due to Voice Fallback, or a failure occurs during an handover triggered due to Voice Fallback, from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node; the UE attempts to re-connect to a cell belonging to an E-UTRAN node, or an NG-RAN node.

The UE makes the RLF Report available to an eNB, when RLF happens in E-UTRAN and the UE re-connects to an eNB cell. Availability of the RLF Report at the RRC connection setup or at a handover to E-UTRAN cell is the indication that the UE suffered a connection failure and that the RLF Report from this failure was not yet delivered to the network.

The eNB receiving the RLF Report from the UE may forward the report to the eNB that served the UE before the reported connection failure using the RLF INDICATION message over X2 or by means of the eNB configuration transfer procedure and MME configuration transfer procedure over S1. If present in the RLF Report, the radio measurements may be used to identify lack of coverage as the potential cause of the failure. This information may be used to exclude those events from the MRO evaluation and redirect them as input to other algorithms.

Detection mechanisms for Too Late Inter-RAT Handover, Too Early Inter-RAT Handover, and Inter-RAT Mobility Failure during Voice Fallback are carried out through the following:

-	[Too Late Inter-RAT Handover]
The connection failure occurs while being connected to an LTE cell, and there is no recent handover for the UE prior to the connection failure i.e., the UE reported timer is absent or larger than the configured threshold, e.g., Tstore_UE_cntxt, and the first cell where the UE attempts to re-connect is a UTRAN cell.

-	[Too Early Inter-RAT Handover]
The connection failure occurs while being connected to an LTE cell, and there is a recent inter-RAT handover for the UE prior to the connection failure i.e., the UE reported timer is smaller than the configured threshold, e.g., Tstore_UE_cntxt, and the first cell where the UE attempts to re-connect and the cell that served the UE at the last handover initialisation are both UTRAN cells.

-	[Inter-RAT Mobility Failure during Voice Fallback]
The connection failure occurs while being connected to an LTE cell, and there is a recent inter-RAT handover for the UE prior to the connection failure i.e., the UE reported timer is smaller than the configured threshold, e.g., Tstore_UE_cntxt, during mobility from NR, and the RLF Report from the UE includes a voice fallback indication.

Editor’s note 1: the indication and its name first need to be agreed in RAN2.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure.

In case the failure is a Too Early Inter-RAT Handover, the eNB receiving the RLF INDICATION message may inform the UTRAN node by means of the eNB Direct Information Transfer procedure over S1. The information contains:

-	Type of detected handover problem (Too Early Inter-RAT Handover);

-	UE RLF Report Container: the RLF Report received from the UE, as specified in TS 36.331 [16];

-	Mobility Information (optionally, if provided in the last Handover Resource Allocation procedure from the UTRAN node);

# Appendix B: TP to SON BL CR to TS 37.340

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

## 10.5	Secondary Node Change (MN/SN initiated)

### 10.5.1	EN-DC

-- TEXT OMITTED --

MN initiated conditional SN Change

The MN initiated conditional inter-SN change procedure is used for CPC configuration and CPC execution.

-- TEXT OMITTED --

6a-6b.	The MN triggers the MeNB initiated SgNB Release procedure to inform the source SN to stop providing user data to the UE, and, if applicable, the address of the SN of the selected candidate PSCell to start data forwarding.

7a-7c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SgNB Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SgNB Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

NOTE x: The target SN updates the time UE stayed in cell of the latest PSCell entry (the source PSCell) when the UE successfully accesses a candidate cell of the target SN. The updated value of the time UE stayed in source PSCell is equal to the value received from the MN during SN addition plus the time from receiving the SN addition request to completion of the Random Access Procedure with the UE.

8.	The UE synchronizes to the PSCell indicated in the RRCConnectionReconfiguration* message applied in step 5.

-- TEXT OMITTED --

SN initiated conditional SN Change

The SN initiated conditional SN change procedure is used for CPC configuration and CPC execution.

The SN initiated conditional SN change procedure may also be initiated by the source SN, to modify the existing SN-initiated CPC configuration, or to trigger the release of the candidate SN by cancellation of all the prepared PSCells at the candidate SN and releasing the CPC related UE context at the candidate SN.

-- TEXT OMITTED --

11a-11b.	The MN triggers the MeNB initiated SgNB Release procedure to inform source SN to stop providing user data to the UE, and provide the address of the SN of the selected candidate PSCell and if applicable, start late data forwarding.

12a-12c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SgNB Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SgNB Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

NOTE x: The target SN updates the time UE stayed in cell of the latest PSCell entry (the source PSCell) when the UE successfully accesses a candidate cell of the target SN. The updated value of the time UE stayed in source PSCell is equal to the value received from the MN during SN addition plus the time from receiving the SN addition request to completion of the Random Access Procedure with the UE.

13.	The UE synchronizes to the PSCell indicated in the RRCConnectionReconfiguration* message applied in step 10.

-- TEXT OMITTED --

### 10.5.2	MR-DC with 5GC

-- TEXT OMITTED --

MN initiated conditional SN Change

The Conditional Secondary Node Change procedure is initiated by the MN for CPC configuration and CPC execution.

-- TEXT OMITTED --

6a-6c.	The MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and if applicable, triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell, to start late data forwarding.

7a-7c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SN Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

NOTE x: The target SN updates the time UE stayed in cell of the latest PSCell entry (the source PSCell) when the UE successfully accesses a candidate cell of the target SN. The updated value of the time UE stayed in source PSCell is equal to the value received from the MN during SN addition plus the time from receiving the SN addition request to completion of the Random Access Procedure with the UE.

8.	The UE synchronizes to the PSCell indicated in the RRCReconfiguration* message applied in step 5.

-- TEXT OMITTED --

SN initiated conditional SN Change

The SN initiated conditional SN change procedure is used for CPC configuration and CPC execution.

The SN initiated conditional SN change procedure may also be initiated by the source SN, to modify the existing CPC configuration, or to trigger the release of the candidate SN by cancellation of all the prepared PSCells at the candidate SN and releasing the CPC related UE context at the candidate SN.

-- TEXT OMITTED --

11a-11c.	The MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell and if applicable, starts late data forwarding.

12a-12c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SN Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

NOTE x: The target SN updates the time UE stayed in cell of the latest PSCell entry (the source PSCell) when the UE successfully accesses a candidate cell of the target SN. The updated value of the time UE stayed in source PSCell is equal to the value received from the MN during SN addition plus the time from receiving the SN addition request to completion of the Random Access Procedure with the UE.

13.	The UE synchronizes to the PSCell indicated in the RRCReconfiguration* message applied in step 10.

<<<<<<<<<<<<<<<<<<<< End of 1st Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED --

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

## 10.18	Self-optimisation for PSCell change

### 10.18.1	General

For analysis of PSCell change failure, the UE makes the SCG Failure Information available to the MN.

MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

### 10.18.2	PSCell change failure

One of the functions of self-optimization for PSCell change is to detect PSCell change failures that occur due to Too late PSCell change or Too early PSCell change, or Triggering PSCell change to wrong PSCell. These problems are defined as follows:

-	Too late PSCell change: an SCG failure occurs after the UE has stayed for a long period of time in the PSCell; a suitable different PSCell is found based on the measurements reported from the UE.

-	Too early PSCell change: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

In the definition above, the "successful PSCell change" refers to the UE state, namely the successful completion of the RA procedure.

### 10.18.X	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases:

-	Wrong target PSCell selection at the (candidate) target SN: if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA to a (candidate) target SN, but not one of the candidate PSCells selected by the (candidate) target SN;

-	Wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA: CPC/CPA Execution to wrong PSCell failure cases not covered by the “Wrong target PSCell selection at the (candidate) target SN” definition.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>