3GPP TSG-RAN WG3 Meeting #120	R3-233098

# Incheon, South Korea – May 22nd – May 26th 2023

Agenda Item:	10.2.5

Source:	Ericsson, Deutsche Telekom, Qualcomm Incorporated

Title:	  (TP for SON BL CR for TS 38.423) CAC per NR-U Channel

Document for:	Discussion, Approval

# Introduction

This contribution contains the TP for XnAP corresponding to the proposals in R3-233097.

# TP for XnAP - CAC per NR-U Channel

-----------------------------------------------  Start of XnAP Changes ------------------------------------------------------

#### 9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | The NR-U channel utilised in the last reporting period | – |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 corresponds to the duration between consecutive reporting. | – |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | – |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Energy Detection Threshold UL | O |  | INTEGER (-100..-50,…) | Average ED Threshold used for UL channel sensing as available at the gNB. Value is in dBm. | YES | ignore |
| >>>>Composite Available Capacity Group per NR-U Channel | O |  | 9.2.2.51 |  | YES | Ignore |

Editor’s note: FFS on whether the value for the Energy Detection Threshold UL in RESOURCE STATUS UPDATE can be obtained by the gNB in an implementation specific way and/or based on Energy Detection Theshold UL provided by the UEs.

-----------------------------------------------  Start of XnAP ASN.1 Changes ---------------------------------------------

-- 9.3.5	Information Element definitions

(unchanged part skipped)

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-TAINSAGSupportList,

id-earlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-ChannelOccupancyTimePercentageUL,

id-EnergyDetectionThresholdUL,

id-CompositeAvailableCapacityGroupNR-U-Channel,

maxEARFCN,

maxnoofAllowedAreas,

-----------------------------------------------  Next XnAP ASN.1 Change ---------------------------------------------

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						NR-U-ChannelID,

channelOccupancyTimePercentageDL	ChannelOccupancyTimePercentage,

energyDetectionThreshold			EnergyDetectionThreshold,

iE-Extension		ProtocolExtensionContainer { {NR-U-Channel-Item-ExtIEs} } 	OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-ChannelOccupancyTimePercentageUL	CRITICALITY ignore EXTENSION ChannelOccupancyTimePercentage PRESENCE optional}|
{ID id-EnergyDetectionThresholdUL	CRITICALITY ignore EXTENSION EnergyDetectionThreshold PRESENCE optional}|
{ID id-CompositeAvailableCapacityGroupNR-U-Channel	CRITICALITY ignore EXTENSION CompositeAvailableCapacityGroup PRESENCE optional},

...

}

-----------------------------------------------  Next XnAP ASN.1 Change ---------------------------------------------

-- 9.3.7	Constant definitions

(unchanged part skipped)

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																			ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-ChannelOccupancyTimePercentageUL				ProtocolIE-ID ::= xx1
id-EnergyDetectionThresholdUL					ProtocolIE-ID ::= xx2

id-CompositeAvailableCapacityGroupNR-U-Channel 	ProtocolIE-ID ::= xx3

END

-- ASN1STOP

-----------------------------------------------  End of XnAP Changes -------------------------------------------------------