3GPP TSG-RAN WG3 #120                                                                                              R3-233163

Incheon, Korea, 22th – 26rd May 2023

Agenda Item:	15.2

Source:	CATT，CBN

Title:	(TP for 38.401/38.413/38.473/37.483)Discussion on efficient MBS reception in RAN sharing scenario

Document for:	Approval

# 1	Introduction

In last RAN3 meeting, some agreements and work assumption are achieved on various aspects related to resource efficiency on MBS reception in RAN sharing scenario. In this contribution, we make further analysis based on the agreements and work assumption and provide corresponding observations and proposals.

- Discussion

Support of location dependent services

For location dependent broadcast service, it was discussed in last RAN3 meeting.There was the work assumption made as below：

WA: In case of location dependent broadcast services, the gNB deduces identical broadcast content from the MBS Associated Session ID and the MBS Service Area information provided by the participating 5GCs. (to be checked against the actual SA2 agreements / agreed CR text.)

Meanwhile, SA2 also captured the similar statement in agreed CR[2] as below:

For location dependent broadcast services, the shared NG-RAN is required to determine that the multiple broadcast MBS Sessions via different CNs deliver the same content for location-dependent MBS session with additionally considering the MBS Service Area.

So, based on the discussion in both SA2 and RAN3,the common understanding is that NG-RAN node could deduce whether the same MBS contents are delivered even the area session ID is different from different PLMNs. Here,we further analize the different cases for RAN sharing in location dependent MBS secanrios i.e. there is overlapping cells/TAs in the MBS areas from different PLMNs and no overlapping cells/TAs.

- There is overlapping cells/TAs in the MBS areas from different PLMNs

For example, cell 1,2,3,4 broadcast the same MBS content, and PLMN A decides to broadcast the MBS service in cell 1,2 and 3 while PLMN B decides to broadcast in cell 3 and 4. NG-RAN node deduces that PLMN A and PLMN B aimed to deliver the same broadcast content according to the overlapping cell 3. Therefore, when there is overlapping cells/TAs in the MBS areas from different PLMNs, NG-RAN node deduces identical broadcast content based on whether there is overlapping cells/TAs among the areas identified by different area session ID, and the overlapping cells/TAs could share the radio resources.

Proposal 1-1: When there is overlapping cells/TAs in the MBS areas from different PLMNs, NG-RAN node deduces identical broadcast content based on whether there is overlapping cells/TAs among the areas identified by different area session ID, and the overlapping cells/TAs could share the radio resources.

- No overlapping cells/TAs in the MBS areas from different PLMNs

It is possible that there is no overlapping cells/TAs in the MBS areas from different PLMNs, which is a rare case. For example, cell 1,2,3,4 broadcast the same MBS content while PLMN A only broadcast the MBS service in cell 1,2 and PLMN B only broadcast the MBS service in cell 3,4. Since no overlapping cells/TAs in the MBS areas between PLMN A and PLMN B, NG-RAN node could not deduces identical broadcast content. All nodes on the N3 interface could use one message, but the establishment of radio resources is per cell. When there is no overlapping cells/TAs, each cell needs to establish radio resources separately. And one more NG-U tunnel is established if NG-RAN node could not deduces identical broadcast content in the case of no overlapping cells/TAs. The benefit is only one NG-U tunnel saved, so it seems not necessary to consider the case with no overlapping cells/TAs. The optimization space is small.

Proposal 1-2: In case there is no overlapping cells/TAs in the MBS areas from different PLMNs, the benefit is only one NG-U tunnel saved, so it seems not necessary to consider this case. The optimization space is small.

How to introduce Associated Session ID

In RAN3#119 meeting, there was discussion on whether the associated session ID is unified among all area sessions for location dependent broadcast service. And we agreed to wait for the conclusion of SA2. Then according to the discussion in last SA2 meeting, the common undersantding is that there is only one Associated Session ID for one MBS session. With this, it is more suitable to introduce Associcated Session ID IE in MBS Session ID IE.

Proposal 2-1:It is proposed to include Associcated Session ID in MBS Session ID IE.

How to let gNB-DU be aware that multiple MBS sessions aimed to the same MSB service (and impact of E1AP)

On how to let gNB-DU be aware that multiple MBS sessions aimed to the same MSB service, based on the last meeting agreement and agreement in SA2, associated session ID IE which is transferred from CN to gNB-CU should also be provided to gNB-DU during Broadcast Context Setup procedure. And similar as NGAP, it is also useful to transfer the Associated Session ID IE included in MBS Session ID IE from gNB-CU to gNB-DU.

The changes of E1AP should be considered as well. To allow the gNB to select a subset of shared NG-U resources, it is also useful to transfer the Associated Session ID IE included in Global MBS Session ID IE from gNB-CU-CP to gNB-CU-UP.

Proposal 2-2: Similar as NGAP, it is also useful to transfer the Associated Session ID IE included in MBS Session ID IE from gNB-CU to gNB-DU.

Proposal 2-3: It is also useful to transfer the Associated Session ID IE included in Global MBS Session ID IE from gNB-CU-CP to gNB-CU-UP.

Impact of gNB implementation deciding the number of NG-U tunnels:

In the last RAN3 meeting there was the work assumption made as below：

WA: Introduce an explicit indication to 5GC in case that NG-U resources are not setup. Details are FFS.

In last RAN3 meeting, it is common understanding in RAN3 that it would be beneficial to let 5GC be aware that  NG-RAN node decides to not establish shared NG-U resources towards the 5GC.To support this, it is needed to introduce a  new IE i.e.Shared NG-U Not Established IE into the MBS Session Setup or Modification Response Transfer IE, which is in the BROADCAST SESSION SETUP RESPONSEBROADCAST SESSION SETUP RESPONSE message and BROADCAST SESSION MODIFICATION RESPONSE message.

Proposal 3-1: It is proposed to introduce new IE i.e. Shared NG-U Not Established IE in the MBS Session Setup or Modification Response Transfer IE.

The last RAN3 meeting also has discussed whether to introduce a new gNB triggered procedure ( seen in SA2 work) to request establishment of NG-U resources during an ongoing BC session. In SA2 work, if NG-RAN node fails to receive DL data via the selected PLMN for which the user plane tunnel is established with no other user plane tunnel available, NG-RAN node should be able to request the tunnel establishment towards 5GC. Therefore, a new gNB triggered shared N3 setup procedure is needed for broadcast.

Proposal 3-2: A new gNB triggered shared N3 setup procedure is needed for broadcast.

Currently, it is already supported for NG-RAN node to request the NG-U tunnel establishment for multicast via Distribution Setup procedure. So, there could be two options to support NG-RAN initiated NG-U tunnel setup for broadcast service as below：

Option 1: Reuse the current Distribution Setup procedure.

Option 2: Introduce another Broadcast Distribution Setup procedure.

As discussed in previous RAN3 meeting, in Rel-17, for multicast, only one tunnel for the area in which the UE is located is established during multicast distribution setup  procedure. However, for broadcast, when setup a new MBS service, multiple tunnels for all MBS areas in the concerned NG-RAN node should be established. Besides, different procedures are used for Broadcast service and multicast service since Rel-17, So, it is more suitable  to introduce a dedicated Distribution Setup procedure for broadcast.

Similarly as multicast service, Broadcast Distribution Release procedure should also be introduced which enables NG-RAN node to release the NG-U tunnel if not needed any more.

Proposal 3-3: It is proposed to introduce Broadcast Distribution Setup procedure and Broadcast Distribution Release procedure to enable NG-RAN node to request the establishment/Release of NG-U tunnel for Broadcast service.

Decision on the number of NG-U tunnels for CP/UP separation cases.

Although gNB-CU-UP provide the NG-U tunnel information i.e. BC Bearer Context NG-U TNL Info at NG-RAN to gNB-CU-CP, it need further discussion which node makes decision on whether the NG-U tunnel should be established or not,i.e. CU-CP or CU-UP.From our perspective,the control function should be located in CU-CP. Therefore, we prefer that gNB-CU-CP decide the establishment of NG-U tunnel if gNB-CU-UP is shared. With that,it is proposed to introduce the new Establish NG-U tunnel IE in BC Bearer Context To Setup IE which in the BC BEARER CONTEXT SETUP message to indicate gNB-CU-UP whether to establish NG-U tunnel.

Proposal 4: It is proposed to introduce the new Establish NG-U tunnel IE in BC Bearer Context To Setup IE which in the BC BEARER CONTEXT SETUP message to indicate gNB-CU-UP whether to establish NG-U tunnel.

F1 functions: Broadcast context setup for MOCN sharing scenario

For broadcast context setup for MOCN sharing, the progress is as below in the last RAN3 meeting：

Support, for MOCN, sharing of F1-U resources among multiple broadcast MBS sessions with the same associated session ID.

To be continued: Whether, for MOCN, F1 supports establishment of a single Broadcast Context for multiple MBS sessions at the DU.

For MOCN sharing scenario, different PLMNS have different TMGI, but different MBS session IDs correspond to the same content of broadcast context established in DU. Multiple TMGIs share the same MRB configuration given to all UEs via MCCH, and the CU only needs to send common broadcast context setup message one time, no need to send multiple setup messages.

From our point of view, it would be useless and also waste of resources in DU if multiple broadcast contexts which are completely the same are established in DU. For MOCN, it is sufficient for F1 to support the establishment of a single Broadcast Context for multiple MBS sessions at the DU. Therefore, it is also sufficient for gNB-CU to send setup/modification message in a single F1AP including a list of TMGIs and an MBS RAN Sharing efficiency Information

Proposal 5-1: For MOCN, it is sufficient for F1 to support establishment of a single Broadcast Context for multiple MBS sessions at the DU.

Proposal 5-2: It is also sufficient for gNB-CU to send setup/modification message in a single F1AP including a list of TMGIs and an MBS RAN Sharing efficiency Information

PDCP aspects for RAN sharing with multiple Cell ID broadcast

For MOCN scenario, since gNB-CU is shared, it is natural that the same MRB is allocated in the shared gNB-CU. However, for multiple cell-ID broadcast scenarios, it is difficult for different gNB-CUs to have the same flow to MRB mapping for the same MBS sessions since the gNB-CU from different PLMN are deployed separately. The last RAN3 meeting has agreed the work assumption that In case of RAN sharing with multiple Cell ID broadcast, the entity controlling the logical DUs decides which MRB-PDCP-ConfigBroadcast to provide on MCCH. Last meeting has discussed two solutions i.e.OAM configures the PDCP configuration and signaling based solution work. Both of the above solutions can work, but configuration based solution is too staticit, and it is not workable to configure the PDCP configuration by OAM since PDCP configuration is per DRB which is dynamic. Therefore, we have a slight preference on signaling based solution.

Proposal 6-1：It is proposed to adopt the signaling based solution rather than OAM based solution for PDCP aspects for RAN sharing with multiple Cell ID broadcast.

Since the Multiple Cell ID scenario is quite similar with shared CU-UP scenario discussed in Rel-17, we think the mechanism for Rel-17 shared CU-UP scenario could be reused,i.e. CU-CP informs DU whether DU could use the available configruaiont or requested configuration. As to how the DU select the configuration is used, there are two possible options on the table as below:

Solution1: gNB-DU uses the flow to MRB mapping decided by the first gNB-CU which requests to establish the broadcast session and then informs to other gNB-CUs of the information;

Solution2: gNB-DU uses the flow to MRB mapping decided by the primary gNB-CU

Figure 1 illustrates an examplified interaction of F1AP about the establishment of MRB in multiple cell-IDs broadcast scenario.

Figure 1: The establishment of MRB in multiple cell-IDs broadcast scenario

- Solution 1(gNB-DU uses the flow to MRB mapping decided by the first gNB-CU):

1.	The shared gNB-CU1 sends BROADCAST CONTEXT SETUP REQUEST message to the shared gNB-DU, providing MBS session ID, MRB configuration, and the new Requested Action for Multiple Cell-IDs Broadcast IE which includes three values: apply available configuration, apply requested configuration,

, apply available configuration if same as requested.

If the Requested Action for Multiple Cell-IDs Broadcast IE is set to：

-	"apply available configuration" The gNB-DU shall apply the existing PDCP configuration if the current PDCP configuration is available.

-	"apply requested configuration" If the MRB PDCP configuration is different, the gNB-DU will fallback to R17.i.e.the gNB-DU will allocate separate resources as requested by the gNB-CU and rebuild MRB with its own MRB PDCP configuration.

-	"apply available configuration if same as requested" The gNB-DU shall make use of the available MRB PDCP configuration only if the available MRB PDCP configuration is the same as the one requested by the gNB-CU.

Since gNB-CU1 is the first CU to setup the MBS context, gNB DU just follows the configuration from this CU.

2.	After the shared gNB-DU setup MRB successfully, the gNB-DU replies to the gNB-CU1 with BROADCAST CONTEXT SETUP RESPONSE, providing the current MRB configuration.

3.	Then, the gNB-CU2 sends BROADCAST CONTEXT SETUP REQUEST message to the shared gNB-DU. And if the Requested Action for Multiple Cell-IDs Broadcast IE is set to “apply available configuration”, the gNB-DU will follow the previous PDCP configuration provided by the first gNB-CU.i e. gNB-CU1. And if the Requested Action for Multiple Cell-IDs Broadcast IE is set to “apply requested configuration”,gNB-DU will setup the another MRB using the PDCP configuration decided by the gNB-CU2.

4.	The gNB-DU informs the gNB-CU2 the current MRB configuration.

5.	Then, the gNB-CU3 sends BROADCAST CONTEXT SETUP REQUEST message to the shared gNB-DU. Similar as in Step 3

6.	The gNB-DU informs the gNB-CU3 the current MRB configuration and so on until gNB-DU completed the establishment of MRB providing by all shared gNB-CUs.

- Solution 2(gNB-DU uses the flow to MRB mapping decided by the primary gNB-CU):

1/2 	Similar as the step 1/2 of Solution 1.

3.	The gNB-CU2 sends BROADCAST CONTEXT SETUP REQUEST message to the shared gNB-DU. If the gNB-CU2 is the primary gNB-CU, the shared DU will reverse the initially selected MRB PDCP configuration base on subsequently received MRB PDCP configuration decided by the primary gNB-CU2.

4.	After the shared gNB-DU setuped MRB successfully, the gNB-DU informs the gNB-CU2 the current MRB configuration and informs other gNB-CU i.e.the gNB-CU1 of the current MRB configuration.

4a.		The gNB-DU sends BROADCAST CONTEXT MODIFICATION REQUIRED message to the shared gNB-CU.

4b.		The gNB-CU sends BROADCAST CONTEXT MODIFICATION CONFIRM message to the shared gNB-DU.

5/6.	Similar as the step 5/6 of Solution 1.

Proposal 6-2: For Multiple cell-ID broadcast scenarios, it is proposed to reuse the mechanism for shared CU-UP.RAN3 could further discuss whether gNB-DU should follow the configruaiton of the first gNB-CU or the primiary gNB-CU.

Proposal 6-3: It is proposed to introduce the new Requested Action for Multiple Cell-IDs Broadcast IE in BROADCAST CONTEXT SETUP REQUEST.

Proposal 6-4: The gNB-DU provides the current MRB configuration to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE message.

# 3	Conclusion

Based on the discussion in section 2, we have the following proposals

Proposal 1-1: When there is overlapping cells/TAs in the MBS areas from different PLMNs, NG-RAN node deduces identical broadcast content based on whether there is overlapping cells/TAs among the areas identified by different area session ID, and the overlapping cells/TAs could share the radio resources.

Proposal 1-2: In case there is no overlapping cells/TAs in the MBS areas from different PLMNs, the benefit is only one NG-U tunnel saved, so it seems not necessary to consider this case. The optimization space is small.

Proposal 2-1:It is proposed to include Associcated Session ID in MBS Session ID IE.

Proposal 2-2: Similar as NGAP, it is also useful to transfer the Associated Session ID IE included in MBS Session ID IE from gNB-CU to gNB-DU.

Proposal 2-3: It is also useful to transfer the Associated Session ID IE included in Global MBS Session ID IE from gNB-CU-CP to gNB-CU-UP.

Proposal 3-1: It is proposed to introduce new IE i.e. Shared NG-U Not Established IE in the MBS Session Setup or Modification Response Transfer IE.

Proposal 3-2: A new gNB triggered shared N3 setup procedure is needed for broadcast.

Proposal 3-3: It is proposed to introduce Broadcast Distribution Setup procedure and Broadcast Distribution Release procedure to enable NG-RAN node to request the establishment/Release of NG-U tunnel for Broadcast service.

Proposal 4: It is proposed to introduce the new Establish NG-U tunnel IE in BC Bearer Context To Setup IE which in the BC BEARER CONTEXT SETUP message to indicate gNB-CU-UP whether to establish NG-U tunnel.

Proposal 5-1: For MOCN, it is sufficient for F1 to support establishment of a single Broadcast Context for multiple MBS sessions at the DU.

Proposal 5-2: It is also sufficient for gNB-CU to send setup/modification message in a single F1AP including a list of TMGIs and an MBS RAN Sharing efficiency Information

Proposal 6-1：It is proposed to adopt the signaling based solution rather than OAM based solution for PDCP aspects for RAN sharing with multiple Cell ID broadcast.

Proposal 6-2: For Multiple cell-ID broadcast scenarios, it is proposed to reuse the mechanism for shared CU-UP.RAN3 could further discuss whether gNB-DU should follow the configruaiton of the first gNB-CU or the primiary gNB-CU.

Proposal 6-3: It is proposed to introduce the new Requested Action for Multiple Cell-IDs Broadcast IE in BROADCAST CONTEXT SETUP REQUEST.

Proposal 6-4: The gNB-DU provides the current MRB configuration to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE message.

The TP for F1AP, E1AP, NGAP and 38.401 are provided in annex.

# 4	Reference

[1]	RAN3#119bis-e Chair Notes

[2] S2-2306249 Resolving the EN on the MOCN enhancement for the location dependent MBS session

# 5	TP for BLCR on 38.401

7.7.x	Support of resource efficiency for RAN Sharing

7.7.x1	General

Editor’s Note:	TBD

7.7.x2	Support of resource efficiency for MOCN

F1-AP supports resource efficiency in RAN sharing for MOCN.

For one MBS session, gNB-CU initiates one Broadcast Context Setup procedure for multiple PLMNs to establish

MBS context in the shared gNB-DU.

7.7.x3	Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast

F1AP supports resource efficiency in RAN Sharing with multiple cell-ID broadcast.

gNB-DUs sharing the same physical cell resources receive via F1-C information enabling identifying broadcast MBS sessions providing identical content.

Applying resource efficiency for RAN Sharing with multiple cell-ID broadcast

-	resolve different QoS requirements received from the participating 5GCs in an implementation specific way.

-    apply the MRB configuration from the first gNB-CU or the primary gNB-CU which requests the establishment of MBS context（To be updated based on the conclusion in this meeting）.

Editor’s Note:	Whether specific text for location dependent MBS sessions is necessary is FFS.

# 6	TP for BLCR on 38.413

-----------Start of the Change----------

## 8.17	Broadcast Session Management Procedures

//No change//

### 8.17.X	Broadcast Distribution Setup

#### 8.17.X.1	General

The purpose of the Broadcast Distribution Setup procedure is to assign NG-U resources for a broadcast MBS session. The procedure uses non-UE-associated signalling.

#### 8.17.X.2	Successful Operation

Figure 8.17.X.2-1: Broadcast Distribution Setup, successful operation.

The NG-RAN node initiates the procedure by sending a BROADCAST DISTRIBUTION SETUP REQUEST message to the AMF. The AMF responds with a BROADCAST DISTRIBUTION SETUP RESPONSE message.

#### 8.17.X.3	Unsuccessful Operation

Figure 8.17.X.3-1: Broadcast Distribution Setup, unsuccessful operation.

In case the shared NG-U transport cannot be setup successfully, the AMF shall respond with the BROADCAST DISTRIBUTION SETUP FAILURE message to the NG-RAN node with an appropriate cause value.

#### 8.17.X.4	Abnormal Conditions

Void.

### 8.17.Y	Broadcast Distribution Release

#### 8.17.Y.1	General

The purpose of the Broadcast Distribution Release procedure is to enable the release of already established NG-U resources for a given broadcast MBS session, or for a given area session of the broadcast MBS session. The procedure uses non-UE-associated signalling.

#### 8.17.Y.2	Successful Operation

Figure 8.17.Y.2-1: Broadcast Release, successful operation.

The NG-RAN node initiates the procedure by sending a BROADCAST DISTRIBUTION RELEASE REQUEST message.

Upon receipt of the BROADCAST DISTRIBUTION RELEASE REQUEST message, the AMF shall send the BROADCAST DISTRIBUTION RELEASE RESPONSE message after successfully removing the corresponding NG-U resource for the MBS session.

#### 8.17.Y.3		Unsuccessful Operation

Not applicable.

## 9.2	Message Functional Definition and Content

-----------Next Change----------

#### 9.2.16.a	BROADCAST DISTRIBUTION SETUP REQUEST

This message is sent by the NG-RAN node to request the setup of the NG-U transport for a broadcast MBS session.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.207 |  | YES | reject |
| MBS Distribution Setup Request Transfer | M |  | OCTET STRING | Containing the MBS Distribution Setup Request Transfer IE specified in subclause 9.3.5.7. | YES | reject |

#### 9.2.16.b	BROADCAST DISTRIBUTION SETUP RESPONSE

This message is sent by the AMF to confirm the setup of the NG-U transport.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.207 |  | YES | reject |
| Broadcast Distribution Setup Response Transfer | M |  | OCTET STRING | Containing the Broadcast Distribution Setup Response Transfer IE specified in subclause 9.3.5.X | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.16.c	BROADCAST DISTRIBUTION SETUP FAILURE

This message is sent by the AMF to indicate that the setup of the NG-U transport was unsuccessful.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.207 |  | YES | reject |
| MBS Distribution Setup Unsuccessful Transfer | M |  | OCTET STRING | Containing the MBS Distribution Setup Unsuccessful Transfer IE specified in subclause 9.3.5.9. | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.16.d	BROADCAST DISTRIBUTION RELEASE REQUEST

This message is sent by the NG-RAN node to request the release of the NG-U transport.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.207 |  | YES | reject |
| MBS Distribution Release Request Transfer | M |  | OCTET STRING | Containing the MBS Distribution Release Request Transfer IE specified in subclause 9.3.5.10. | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.16.e	BROADCAST DISTRIBUTION RELEASE RESPONSE

This message is sent by the AMF to confirm the release of the NG-U transport.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.207 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

-----------Next Change----------

#### 9.3.1.206	MBS Session ID

This IE uniquely identifies an MBS session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TMGI | M |  | OCTET STRING (SIZE(6)) | Encoded as defined in TS 23.003 [23]. | - | - |
| NID | O |  | 9.3.3.42 |  | - | - |
| Associated Session ID | O |  | 9.3.1.X |  | YES | ignore |

//No change//

#### 9.3.1.X	Associated Session ID

This IE is used by NG-RAN to identify MBS sessions transmitting the same content in MOCN network sharing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Session ID | M |  | OCTET STRING (SIZE(6)) |  |

-----------Next Change----------

#### 9.3.2.X	Shared NG-U Broadcast TNL Information

This IE provides the shared NG user plane transport layer information per Area Session ID associated with an MBS session at the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IP Broadcast Address | M |  | Transport Layer Address 9.3.2.4 |  |
| IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |
| GTP-TEID at 5GC | M |  | GTP-TEID 9.3.2.5 |  |

-----------Next Change----------

#### 9.3.5.5	MBS Session Setup or Modification Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session TNL Information NG-RAN | O |  | 9.3.2.17 |  | - | - |
| Shared NG-U Not Established | O |  | ENUMERAT ED (not established, ...) | This IE indicates that NG-RAN node decides to not establish shared NG-U resources towards the 5GC | Yes | ignore |

-----------Next Change----------

#### 9.3.5.X	Broadcast Distribution Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.206 |  |
| Shared NG-U Broadcast Information List |  | 1 |  |  |
| >Shared NG-U Broadcast Information Item |  | 0..<maxnoofMBSAreaSessionIDs> |  |  |
| >>MBS Area Session ID | O |  | 9.3.1.207 |  |
| >>Shared NG-U Broadcast TNL Information | O |  | 9.3.2.X |  |
| >>MBS QoS Flows To Be Setup List | M |  | 9.3.1.236 |  |
| >>MBS Service Area | O |  | 9.3.1.208 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs allowed within one MBS session. Value is 256. |

-----------Next Change----------

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

AMFConfigurationUpdate,

AMFConfigurationUpdateAcknowledge,

AMFConfigurationUpdateFailure,

AMFCPRelocationIndication,

AMFStatusIndication,

BroadcastSessionModificationFailure,

BroadcastSessionModificationRequest,

BroadcastSessionModificationResponse,

BroadcastSessionReleaseRequest,

BroadcastSessionReleaseRequired,

BroadcastSessionReleaseResponse,

BroadcastSessionSetupFailure,

BroadcastSessionSetupRequest,

BroadcastSessionSetupResponse,

BroadcastDistributionReleaseRequest,

BroadcastDistributionReleaseResponse,

BroadcastDistributionSetupFailure,

BroadcastDistributionSetupRequest,

BroadcastDistributionSetupResponse,

CellTrafficTrace,

ConnectionEstablishmentIndication,

DeactivateTrace,

DistributionReleaseRequest,

DistributionReleaseResponse,

DistributionSetupFailure,

DistributionSetupRequest,

DistributionSetupResponse,

////////////////////////////////No change/////////////////////////////////

FROM NGAP-PDU-Contents

id-AMFConfigurationUpdate,

id-AMFCPRelocationIndication,

id-AMFStatusIndication,

id-BroadcastSessionModification,

id-BroadcastSessionRelease,

id-BroadcastSessionReleaseRequired,

id-BroadcastSessionSetup,

id-BroadcastDistributionSetup,

id-BroadcastDistributionRelease,

id-CellTrafficTrace,

id-ConnectionEstablishmentIndication,

id-DeactivateTrace,

id-DistributionSetup,

id-DistributionRelease,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURES NGAP-ELEMENTARY-PROCEDURE ::= {

NGAP-ELEMENTARY-PROCEDURES-CLASS-1			|

NGAP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

broadcastSessionModification	|

broadcastSessionRelease			|

broadcastSessionSetup			|

broadcastDistributionSetup		|

broadcastDistributionRelease	|

distributionSetup				|

distributionRelease				|

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

aMFConfigurationUpdate NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AMFConfigurationUpdate

SUCCESSFUL OUTCOME		AMFConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	AMFConfigurationUpdateFailure

PROCEDURE CODE			id-AMFConfigurationUpdate

CRITICALITY				reject

}

aMFCPRelocationIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AMFCPRelocationIndication

PROCEDURE CODE			id-AMFCPRelocationIndication

CRITICALITY				reject

}

aMFStatusIndication NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		AMFStatusIndication

PROCEDURE CODE			id-AMFStatusIndication

CRITICALITY				ignore

}

broadcastSessionModification NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionModificationRequest

SUCCESSFUL OUTCOME		BroadcastSessionModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionModificationFailure

PROCEDURE CODE			id-BroadcastSessionModification

CRITICALITY				reject

}

broadcastSessionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionReleaseRequest

SUCCESSFUL OUTCOME		BroadcastSessionReleaseResponse

PROCEDURE CODE			id-BroadcastSessionRelease

CRITICALITY				reject

}

broadcastSessionReleaseRequired NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionReleaseRequired

PROCEDURE CODE			id-BroadcastSessionReleaseRequired

CRITICALITY				reject

}

broadcastSessionSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionSetupRequest

SUCCESSFUL OUTCOME		BroadcastSessionSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionSetupFailure

PROCEDURE CODE			id-BroadcastSessionSetup

CRITICALITY				reject

}

broadcastDistributionSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastDistributionSetupRequest

SUCCESSFUL OUTCOME		BroadcastDistributionSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastDistributionSetupFailure

PROCEDURE CODE			id-BroadcastDistributionSetup

CRITICALITY				reject

}

broadcastDistributionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastDistributionReleaseRequest

SUCCESSFUL OUTCOME		BroadcastDistributionReleaseResponse

PROCEDURE CODE			id-BroadcastDistributionRelease

CRITICALITY				reject

}

cellTrafficTrace NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-CellTrafficTrace

CRITICALITY				ignore

}

connectionEstablishmentIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ConnectionEstablishmentIndication

PROCEDURE CODE			id-ConnectionEstablishmentIndication

CRITICALITY				reject

}

deactivateTrace NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

distributionSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DistributionSetupRequest

SUCCESSFUL OUTCOME		DistributionSetupResponse

UNSUCCESSFUL OUTCOME	DistributionSetupFailure

PROCEDURE CODE			id-DistributionSetup

CRITICALITY				reject

}

distributionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DistributionReleaseRequest

SUCCESSFUL OUTCOME		DistributionReleaseResponse

PROCEDURE CODE			id-DistributionRelease

CRITICALITY				reject

}

-----------Next Change----------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

////////////////////////////////No change/////////////////////////////////

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

BroadcastDistributionSetupResponseTransfer,

CancelAllWarningMessages,

////////////////////////////////No change/////////////////////////////////

FROM NGAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

NGAP-PRIVATE-IES,

NGAP-PROTOCOL-EXTENSION,

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

////////////////////////////////No change/////////////////////////////////

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-BroadcastDistributionSetupResponseTransfer,

id-CancelAllWarningMessages,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

##### -- BROADCAST SESSION RELEASE RESPONSE

--

-- **************************************************************

BroadcastSessionReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionReleaseResponseIEs} },

...

}

BroadcastSessionReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID								PRESENCE mandatory	}|

{ ID id-MBSSessionReleaseResponseTransfer		CRITICALITY ignore	TYPE OCTET STRING (CONTAINING MBSSessionReleaseResponseTransfer)		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Broadcast Distribution Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST DISTRIBUTION SETUP REQUEST

--

-- **************************************************************

BroadcastDistributionSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastDistributionSetupRequestIEs} },

...

}

BroadcastDistributionSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionSetupRequestTransfer			CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBS-DistributionSetupRequestTransfer)			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST DISTRIBUTION SETUP RESPONSE

--

-- **************************************************************

BroadcastDistributionSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastDistributionSetupResponseIEs} },

...

}

BroadcastDistributionSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-BroadcastDistributionSetupResponseTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING BroadcastDistributionSetupResponseTransfer)		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- BROADCAST DISTRIBUTION SETUP FAILURE

--

-- **************************************************************

BroadcastDistributionSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastDistributionSetupFailureIEs} },

...

}

BroadcastDistributionSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionSetupUnsuccessfulTransfer	CRITICALITY ignore	TYPE OCTET STRING (CONTAINING MBS-DistributionSetupUnsuccessfulTransfer)	PRESENCE mandatory	}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Broadcast Distribution Release Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST DISTRIBUTION RELEASE REQUEST

--

-- **************************************************************

BroadcastDistributionReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { BroadcastDistributionReleaseRequestIEs} },

...

}

BroadcastDistributionReleaseRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionReleaseRequestTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBS-DistributionReleaseRequestTransfer)		PRESENCE mandatory	}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST DISTRIBUTION RELEASE RESPONSE

--

-- **************************************************************

BroadcastDistributionReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastDistributionReleaseResponseIEs} },

...

}

BroadcastDistributionReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Distribution Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- DISTRIBUTION SETUP REQUEST

--

-- **************************************************************

DistributionSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DistributionSetupRequestIEs} },

...

}

DistributionSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionSetupRequestTransfer			CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBS-DistributionSetupRequestTransfer)			PRESENCE mandatory	},

...

}

-----------Next Change----------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AlternativeQoSParaSetList,

id-AssociatedSessionID

id-BurstArrivalTimeDownlink,

////////////////////////////////No change/////////////////////////////////

#### -- A

////////////////////////////////No change/////////////////////////////////

AssociatedQosFlowList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF AssociatedQosFlowItem

AssociatedQosFlowItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

qosFlowMappingIndication		ENUMERATED {ul, dl, ...}							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AssociatedQosFlowItem-ExtIEs} }	OPTIONAL,

...

}

AssociatedQosFlowItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetIndex	PRESENCE optional	},

...

}

AssociatedSessionID ::= OCTET STRING (SIZE(6))

AuthenticatedIndication ::= ENUMERATED {true, ...}

////////////////////////////////No change/////////////////////////////////

BroadcastPLMNItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-NPN-Support					CRITICALITY reject	EXTENSION NPN-Support				PRESENCE optional}|

{ID id-ExtendedTAISliceSupportList	CRITICALITY reject	EXTENSION ExtendedSliceSupportList 	PRESENCE optional}|

{ID id-TAINSAGSupportList			CRITICALITY ignore	EXTENSION TAINSAGSupportList		PRESENCE optional},

...

}

BroadcastDistributionSetupResponseTransfer ::= SEQUENCE {

mBS-SessionID								MBS-SessionID,

sharedNGU-BroadcastInformationList		SharedNGU-BroadcastInformationList,

iE-Extensions		ProtocolExtensionContainer { {BroadcastDistributionSetupResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

BroadcastDistributionSetupResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasurementConfiguration ::= SEQUENCE {

bluetoothMeasConfig				BluetoothMeasConfig,

bluetoothMeasConfigNameList		BluetoothMeasConfigNameList											OPTIONAL,

bt-rssi							ENUMERATED {true, ...}												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { BluetoothMeasurementConfiguration-ExtIEs } } 	OPTIONAL,

...

}

////////////////////////////////No change/////////////////////////////////

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF TAI

MBS-SessionID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-SessionID-ExtIEs} } 		OPTIONAL,

...

}

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AssociatedSessionID		CRITICALITY ignore	EXTENSION AssociatedSessionID		PRESENCE optional	},

...

}

MBSSessionFailedtoSetupList ::= SEQUENCE (SIZE(1.. maxnoofMBSSessions)) OF MBSSessionFailedtoSetupItem

MBSSessionFailedtoSetupItem ::= SEQUENCE {

mBS-SessionID			MBS-SessionID,

mBS-AreaSessionID		MBS-AreaSessionID															OPTIONAL,

cause					Cause,

iE-Extensions			ProtocolExtensionContainer { { MBSSessionFailedtoSetupItem-ExtIEs} }		OPTIONAL,

...

}

MBSSessionFailedtoSetupItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

////////////////////////////////No change/////////////////////////////////

SgNB-UE-X2AP-ID ::= INTEGER (0..4294967295)

SharedNGU-BroadcastInformationList ::= SEQUENCE (SIZE(0.. maxnoofMBSAreaSessionIDs)) OF SharedNGU-BroadcastInformationItem

SharedNGU-BroadcastInformationItem ::= SEQUENCE {

mBS-AreaSessionID							MBS-AreaSessionID					OPTIONAL,

sharedNGU-MulticastTNLInformation			SharedNGU-MulticastTNLInformation,

mBS-QoSFlowsToBeSetupList					MBS-QoSFlowsToBeSetupList,

mBS-ServiceArea								MBS-ServiceArea						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SharedNGU-BroadcastTNLInformationItem-ExtIEs} }	OPTIONAL,

...

}

SharedNGU-BroadcastTNLInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SharedNGU-BroadcastTNLInformation ::= SEQUENCE {

iP-BroadcastAddress 		TransportLayerAddress,

iP-SourceAddress			TransportLayerAddress,

gTP-TEID						GTP-TEID,

iE-Extensions				ProtocolExtensionContainer { {SharedNGU-BroadcastTNLInformation-ExtIEs} } 	OPTIONAL,

...

}

SharedNGU-BroadcastTNLInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SharedNGU-MulticastTNLInformation ::= SEQUENCE {

iP-MulticastAddress 		TransportLayerAddress,

iP-SourceAddress			TransportLayerAddress,

gTP-TEID						GTP-TEID,

iE-Extensions				ProtocolExtensionContainer { {SharedNGU-MulticastTNLInformation-ExtIEs} } 	OPTIONAL,

...

}

SharedNGU-MulticastTNLInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-----------Next Change----------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-DistributionRelease 						ProcedureCode ::= 70

id-MulticastSessionActivation 				ProcedureCode ::= 71

id-MulticastSessionDeactivation 			ProcedureCode ::= 72

id-MulticastSessionUpdate 					ProcedureCode ::= 73

id-MulticastGroupPaging						ProcedureCode ::= 74

id-BroadcastSessionReleaseRequired			ProcedureCode ::= 75

id-BroadcastDistributionSetup 				ProcedureCode ::= XX

id-BroadcastDistributionRelease 			ProcedureCode ::= XX

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-AssociatedSessionID									ProtocolIE-ID ::= XXX

id-BroadcastDistributionSetupResponseTransfer			ProtocolIE-ID ::= XXX

END

-- ASN1STOP

-----------End of the Change----------

# 7	TP for BLCR on 38.473

-----------Start of the Change----------

#### 9.2.13.1	BROADCAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a broadcast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Requested Action for Multiple Cell-IDs Broadcast | O |  | 9.3.1.X |  | YES | reject |
| Additional TMGI List | O |  | 9.3.1.Y |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

-----------Next Change----------

#### 9.2.13.2	BROADCAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| >>MRB PDCP Config Broadcast | O |  | OCTET STRING | Includes the MRB-PDCP-ConfigBroadcast IE, as defined in TS 38.331[8]. | YES | reject |
| CHOICE BroadcastAreaScope | M |  |  |  | YES | ignore |
| >CompleteSuccess |  |  | NULL |  |  |  |
| >PartialSuccess |  |  |  |  |  |  |
| >>Broadcast Cell List |  | 1 |  |  |  |  |
| >>>Broadcast Cell Item |  | 1 .. <maxcellingNBDU> |  |  |  |  |
| >>>>Cell ID | M |  | NR CGI 9.3.1.12 | Identifier of the cells that establish the broadcast service successfully |  |  |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxcellingNBDU | Maximum no. of cells which establish the MRBs successfully in one DU, the maximum value is 512 |

-----------Next Change----------

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

//No change//

#### 9.3.1.218	MBS Session ID

This IE indicates the MBS Session ID uniquely identifies an MBS session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TMGI | M |  | OCTET STRING (SIZE(6)) | Coded as Temporary Mobile Group Identity (TMGI) defined in TS 23.003 [23]. | - | - |
| NID | O |  | 9.3.1.155 |  | - | - |
| Associated Session ID | O |  | 9.3.1.Z |  | YES | ignore |

//No change//

#### 9.3.1.X	Requested Action for Multiple Cell-IDs Broadcast

This IE provides information about the requested gNB-DU’s action with regards to a potentially available MRB PDCP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested Action for Multiple Cell-IDs Broadcast | M |  | ENUMERATED (apply available configuration, apply requested configuration, ,apply available configuration if same as requested…) |  |

#### 9.3.1.Y	Additional TMGI List

The purpose of the Additional TMGI List IE is to provide the additional TMGI list which identifies the same MBS service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Additional TMGI List |  | 0..< maxnoofBPLMNs -1> |  |  |
| >TMGI | M |  | OCTET STRING (SIZE(6)) | Coded as Temporary Mobile Group Identity (TMGI) defined in TS 23.003 [23]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

#### 9.3.1.Z	Associated Session ID

This IE is used by NG-RAN to identify MBS sessions transmitting the same content in MOCN network sharing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Session ID | M |  | OCTET STRING (SIZE(6)) |  |

-----------Next Change----------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

////////////////////////////////No change/////////////////////////////////

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue

RequestedActionforMultipleCellIDsBroadcast

AdditionalTMGIList

////////////////////////////////No change/////////////////////////////////

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-RequestedActionforMultipleCellIDsBroadcast,

id-AdditionalTMGIList,

maxCellingNBDU,

maxnoofCandidateSpCells,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- BROADCAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP REQUEST

--

-- **************************************************************

BroadcastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupRequestIEs} },

...

}

BroadcastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE	GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-MBS-Session-ID					CRITICALITY reject 	TYPE	MBS-Session-ID					PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea					CRITICALITY reject 	TYPE	MBS-ServiceArea					PRESENCE optional   }|

{ ID id-MBS-CUtoDURRCInformation		CRITICALITY reject	TYPE	MBS-CUtoDURRCInformation		PRESENCE mandatory	}|

{ ID id-SNSSAI							CRITICALITY reject	TYPE	SNSSAI							PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-ToBeSetup-List	CRITICALITY reject	TYPE	BroadcastMRBs-ToBeSetup-List	PRESENCE mandatory	}|

{ ID id-RequestedActionforMultipleCellIDsBroadcast	CRITICALITY reject	TYPE	RequestedActionforMultipleCell-IDsBroadcast			PRESENCE optional   }|

{ ID id-AdditionalTMGIList				CRITICALITY reject	TYPE	AdditionalTMGIList				PRESENCE optional   }

,

...

}

BroadcastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeSetup-ItemIEs} }

BroadcastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeSetup-Item	CRITICALITY reject	TYPE 	BroadcastMRBs-ToBeSetup-Item	PRESENCE mandatory	},

...

}

////////////////////////////////No change/////////////////////////////////

-----------Next Change----------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

////////////////////////////////No change/////////////////////////////////

id-Source-MRB-ID,

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-AssociatedSessionID,

maxNRARFCN,

maxnoofErrors,

////////////////////////////////No change/////////////////////////////////

#### -- A

////////////////////////////////No change/////////////////////////////////

AdditionalPath-List::= SEQUENCE (SIZE(1..maxnoofPath)) OF AdditionalPath-Item

AdditionalPath-Item ::=SEQUENCE {

relativePathDelay	RelativePathDelay,

pathQuality			TRPMeasurementQuality 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalPath-Item-ExtIEs } }	OPTIONAL

}

AdditionalPath-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MultipleULAoA	CRITICALITY ignore EXTENSION MultipleULAoA		PRESENCE optional}|

{ ID id-pathPower		CRITICALITY ignore EXTENSION UL-SRS-RSRPP		PRESENCE optional},

...

}

AdditionalTMGIList::= SEQUENCE (SIZE(0.. maxnoofBPLMN-1)) OF TMGI

ExtendedAdditionalPathList ::= SEQUENCE (SIZE (1.. maxNoPathExtended)) OF ExtendedAdditionalPathList-Item

////////////////////////////////No change/////////////////////////////////

Associated-SCell-Item ::= SEQUENCE {

sCell-ID		NRCGI,

iE-Extensions	ProtocolExtensionContainer { { Associated-SCell-ItemExtIEs } }	OPTIONAL

}

Associated-SCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedSessionID ::= OCTET STRING (SIZE(6))

AvailablePLMNList ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF AvailablePLMNList-Item

AvailablePLMNList-Item ::= SEQUENCE {

pLMNIdentity			PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { { AvailablePLMNList-Item-ExtIEs} } OPTIONAL

}

AvailablePLMNList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

////////////////////////////////No change/////////////////////////////////

BroadcastMRBs-Setup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-Setup-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID mRB-PDCP-Config-Broadcast			CRITICALITY reject	EXTENSION OCTET STRING				PRESENCE optional },

...

}

////////////////////////////////No change/////////////////////////////////

#### -- R

////////////////////////////////No change/////////////////////////////////

MBSF1UInformation ::= SEQUENCE {

mbs-f1u-info				UPTransportLayerInformation,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformation-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformation-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSInterestIndication ::= OCTET STRING

MBS-Session-ID ::= SEQUENCE {

tMGI						TMGI,

nID							NID					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Session-ID-ExtIEs} } OPTIONAL,

...

}

MBS-Session-ID-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AssociatedSessionID		CRITICALITY ignore	EXTENSION AssociatedSessionID		PRESENCE optional	},

...

}

MBS-Area-Session-ID  ::= INTEGER (0..65535, ...)

////////////////////////////////No change/////////////////////////////////

ReportingPeriodicity ::= ENUMERATED{ms500, ms1000, ms2000, ms5000, ms10000, ...}

RequestedActionforMultipleCellIDsBroadcast ::= ENUMERATED {

apply-available-configuration,

apply-requested-configuration,

apply-available-configuration-if-same-as-requested,

…

}

RequestedBandCombinationIndex ::= OCTET STRING

-----------Next Change----------

### 9.4.7	Constant Definitions

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-RequestedActionforMultipleCellIDsBroadcast		ProtocolIE-ID ::= XXX

id-AdditionalTMGIList                               ProtocolIE-ID ::= XXX

id-AssociatedSessionID								ProtocolIE-ID ::= XXX

END

-- ASN1STOP

-----------End of the Change----------

# 8	TP for BLCR on 37.483

-----------Start of the Change----------

### 9.3.1	Radio Network Layer Related IEs

-----------Next Change----------

#### 9.3.1.108	Global MBS Session ID

This IE indicates the TMGI uniquely identifies an MBS session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TMGI | M |  | OCTET STRING (SIZE(6)) | Encoded as defined in TS 23.003. | - | - |
| NID | O |  | 9.3.1.82 | Defined in TS 23.003 [23]. | - | - |
| Associated Session ID | O |  | 9.3.1.X |  | YES | ignore |

-----------Next Change----------

#### 9.3.1.X	Associated Session ID

This IE is used by NG-RAN to identify MBS sessions transmitting the same content in MOCN network sharing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Session ID | M |  | OCTET STRING (SIZE(6)) |  |

### 9.3.3	Container and List IE definitions

-----------Next Change----------

#### 9.3.3.26	BC Bearer Context To Setup

This IE contains MBS session resource related information used to request BC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| BC Bearer Context NG-U TNL Info at 5GC | M |  | 9.3.1.112 |  | - | - |
| BC MRB To Setup List | M |  | BC MRB Setup Configuration 9.3.1.114 |  | - | - |
| Requested Action for Available Shared NG-U Termination | O |  | 9.3.1.115 |  | - | - |
| Establishment of NG-U tunnel | O |  | ENUMERATED (true, ...) | Indicates whether to establish NG-U tunnel | YES | ignore |

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

////////////////////////////////No change/////////////////////////////////

id-MCForwardingResourceRelease,

id-MCForwardingResourceReleaseIndication,

id-PDCP-COUNT-Reset,

id-MBSSessionAssociatedInfoNonSupportToSupport,

id-AssociatedSessionID,

id-EstablishmentofNGUtunnel,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

////////////////////////////////No change/////////////////////////////////

#### -- A

ActivityInformation	::=	CHOICE	{

dRB-Activity-List						DRB-Activity-List,

pDU-Session-Resource-Activity-List		PDU-Session-Resource-Activity-List,

uE-Activity								UE-Activity,

choice-extension						ProtocolIE-SingleContainer	{{ActivityInformation-ExtIEs}}

}

ActivityInformation-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ActivityNotificationLevel	::=	ENUMERATED	{

drb,

pdu-session,

ue,

...

}

AdditionalHandoverInfo ::= ENUMERATED	{

discard-pdpc-SN,

...

}

AdditionalPDCPduplicationInformation	::=	ENUMERATED	{

three,

four,

...

}

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AveragingWindow  ::= INTEGER (0..4095, ...)

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParameterIndex		INTEGER(1..8,...),

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedSessionID ::= OCTET STRING (SIZE(6))

////////////////////////////////No change/////////////////////////////////

#### -- B

##### -- BCBearerContextToSetup

BCBearerContextToSetup ::= SEQUENCE {

snssai								SNSSAI,

bcBearerContextNGU-TNLInfoat5GC		BCBearerContextNGU-TNLInfoat5GC,

bcMRBToSetupList						BCMRBSetupConfiguration,

requestedAction									RequestedAction4AvailNGUTermination		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToSetup-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToSetup-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-EstablishmentofNGUtunnel		CRITICALITY ignore	EXTENSION EstablishmentofNGUtunnel		PRESENCE optional	},

...

}

////////////////////////////////No change/////////////////////////////////

#### -- G

GlobalMBSSessionID ::= SEQUENCE {

tmgi	OCTET STRING (SIZE(6)),

nid		NID						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { GlobalMBSSessionID-ExtIEs } } OPTIONAL,

...

}

GlobalMBSSessionID-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AssociatedSessionID		CRITICALITY ignore	EXTENSION AssociatedSessionID		PRESENCE optional	},

...

}

////////////////////////////////No change/////////////////////////////////

ECGI-Support-Item ::= SEQUENCE {

eCGI	ECGI,

iE-Extensions				ProtocolExtensionContainer { { ECGI-Support-Item-ExtIEs } }	OPTIONAL

}

ECGI-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

EstablishmentofNGUtunnel	::=	ENUMERATED	{

true,

...

}

EUTRAN-QoS-Support-List ::= SEQUENCE (SIZE(1.. maxnoofEUTRANQOSParameters)) OF EUTRAN-QoS-Support-Item

EUTRAN-QoS-Support-Item ::= SEQUENCE {

eUTRAN-QoS	EUTRAN-QoS,

iE-Extensions			ProtocolExtensionContainer { { EUTRAN-QoS-Support-Item-ExtIEs } }	OPTIONAL

}

////////////////////////////////No change/////////////////////////////////

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-MCForwardingResourceResponse									ProtocolIE-ID ::= 181

id-MCForwardingResourceRelease									ProtocolIE-ID ::= 182

id-MCForwardingResourceReleaseIndication						ProtocolIE-ID ::= 183

id-PDCP-COUNT-Reset												ProtocolIE-ID ::= 184

id-MBSSessionAssociatedInfoNonSupportToSupport					ProtocolIE-ID ::= 185

id-AssocoatedSessionID											ProtocolIE-ID ::= XXX

id-EstablishmentofNGUtunnel											ProtocolIE-ID ::= XXX

END

-- ASN1STOP

-----------End of the Change----------