3GPP TSG-RAN WG3 Meeting #120	R3-233175

Incheon, KR, 22 May – 26 May, 2023

Title: 	(TP to TRS_URLLC BLCR for TS 38.413, TS 38.423 and TS 38.473) Support of 5G Timing Resiliency enhancements

Source: 	Huawei, China Unicom

Agenda item:	26.2

Document Type:	Other

# 1. Introduction

The previous RAN3#119bis-e meeting discussed the new WID on NR Timing Resiliency and URLLC enhancements [1], and agreed the BLCRs containing the editor notes/FFSs and with the following open issues:

When the Clock Quality Detail Level is set to “metrics”, is there any additional information included in the Clock Quality Reporting Control Information (e.g. clock quality metrics subscribed by the UE)? To be continued… Impact of mobility on RAN feedback for low latency communication is FFS. Whether to use new or existing procedures to support RAN TSS reporting over NG and F1 is FFS.

Meanwhile, the previous RAN2#121bis-e meeting made the following agreements relevant to RAN3 [2]:

Agreements - RAN2 to confirm that there is no need to support “group of cells across gNBs” for scope of the report ID. The scenario, as per RAN3 will be supported - different cells within gNB that are served by different DUs and different clock quality is possible. - Confirm the following SA2’s conclusions regarding NG-RAN procedures on 5G clock quality information: - RRC_INACTIVE/IDLE UE to be informed of timing synchronization status via an event ID in SIB9 by comparison with the one maintained locally. - RRC_CONNECTED UE to be informed of 5G clock quality information via dedicated RRC message

This paper continues to discuss the potential RAN3 impacts according to the latest agreements at SA2, and provides corresponding TPs in the annex parts.

# 2. Discussion

## 2.1 Messages for RAN TSS reporting

As discussed in the SOD in R3-231971 at the previous meeting, generally there are two major options as follows.

Solution 1: AMF requests reporting of RAN TSS information using a new Class 1 procedure. - New AMF-initiated Class 1 procedure to enable the AMF to request RAN TSS information from the gNB. - New gNB-initiated Class 2 procedure to enable the gNB to report RAN TSS information to the AMF. Solution 2: gNB is configured via OAM to provide RAN TSS, and existing procedures are reused. - Enhance the NG SETUP REQUEST and RAN CONFIGURATION UPDATE messages to optionally include RAN TSS information. Solution 2A: gNB is configured via OAM to provide RAN TSS, and gNB reports RAN TSS using a new Class 2 procedure. - New gNB-initiated Class 2 procedure to enable the gNB to report RAN TSS information to the AMF.

Meanwhile, the SA2 agreed CR in [4] and [5] has the following changes.

In S2-2306224: <Skip the irrelevance> - TSCTSF may receive node-level information about timing synchronization status from gNB and/or UPF/NW-TT directly from OAM or alternatively, if supported by a node, using control plane signalling at node level. Node level signalling uses UMIC for UPF/NW-TT case and an AMF service to report N2 node level information for gNB case. In the latter case, the AMF controls the gNB node level reporting and subscription using NGAP messages (see TS 38.413 [34]). <Skip the irrelevance> The network timing synchronization status information from gNB or UPF/NW-TT to the TSCTSF may contain the following information as described in the Table 5.27.1.12-1. However, it is up to gNB to determine whether to provide its timing synchronization status reporting and which of the information elements to include in the TSS report to the TSCTSF, i.e., based on the implementation gNB may report all, some, or none of the information elements from Table 5.27.1.12-1. In S2-2306225 4.15.9.X.Y TSCTSF subscription to RAN and/or UPF/NW-TT timing synchronization status <Skip the figure> 0. The AF requests creation or modification of ASTI or PTP based time synchronization service as described in clauses 4.15.9.4 and 4.15.9.3 including clock quality acceptance criteria in the request. 1-3. (When the procedure is triggered by the AF request to influence the 5G access stratum time distribution or by PTP instance activation, modification): Upon the reception of the clock quality acceptance criteria in the AF request in step 0, the TSCTSF needs to be subscribed to NG-RAN timing synchronization status updates at the NG-RAN nodes that may provision access stratum time distribution information to the target UE. NG-RAN timing synchronization status updates provisioning may be configured via OAM or via AMF (with node level signalling as illustrated in steps 1-3).

Though SA2 has specified that the AMF can “control” the gNB node level reporting and subscription, in our view, there is no real motivation for the AMF request the TSS, while it is finally up to the RAN to report all, some, or none of attributes based on the gNB implementation. When the OAM configures the TSS attributes, it should configure the same towards the CN and the RAN. Also note that TS 23.501 states that “RAN nodes may be pre-configured with the thresholds for each timing synchronization status attribute”.

By contrast, solution 2/2A is simple and straightforward. If there are some concerns to reuse the existing NG interface management messages to convey the TSS information, then the solution 2A can be taken as starting point, and further discuss if the new class 1 procedure is needed.

- For NGAP, for gNB reporting TSS to the AMF, take solution 2A as starting point, then discuss if the AMF triggered procedure is needed.

In CU-DU split deployment, it should be the DU that maintains the RAN timing synchronization status, and reports the CU, finally to the AMF. Thus, the node-level RAN timing synchronization status reporting from the DU to the CU is needed over the F1AP interface.

Similar to discussions above for NGAP, a new class 2 procedure to report the (DU) node-level RAN timing synchronization status reporting from the gNB-DU to the gNB-CU can be introduced.

- For F1AP, introduce a new class 2 procedure to report the node-level RAN timing synchronization status reporting from the gNB-DU to the gNB-CU, as a starting point.

## 2.2 TSS area report to the AMF

The previous RAN3-119bis-e meeting has the following common understanding.

- It’s the common understanding that RAN Timing Synchronization Status Information can be the same or different between “groups of cells within a single gNB” (e.g., cells served by different gNB-DUs).

As discussed in the reply LS to SA2, the clock quality information could be the same for some or all cells of a gNB as follows.

RAN3 answer: - Depending on gNB topology, clock quality information can be the same for some or all cells of a gNB. Also, topologies can exist where clock quality information is different between “groups of cells within a single gNB” (e.g. cells served by different gNB-DUs).

Meanwhile the SA2 agreed CR in In S2-2306224 has clearly indicated that the scope of the TSS (either node level, or a list of cells) are reported to the AMF, as follows.

gNBs may be pre-configured with thresholds for each timing synchronization status attribute, if supported, that is described in Table 5.27.1.12-1. When the network timing synchronization status exceeds the thresholds (i.e. status degradation), or the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB notifies the TSCTSF (either using N2 node level signalling via AMF, or via OAM) with the gNB ID, the scope of the timing synchronization status (i.e., all cells or a list of Cell IDs within a single gNB) and the corresponding network timing synchronization status (TSS) attributes as described in this clause. The gNB indicates the status change to the UEs via SIB9:

It can be observed that the gNB may report a node level or a group of cells level TSS to the AMF. In non-CU-DU split architecture, a node-level synchronization status is sufficient. While for the CU-DU split case, different gNB-DUs connected to the same gNB-CU may hold different clock quality information. Hence, RAN node should notify the TSCTSF about the timing synchronization status per TSS area over the NGAP interface. About the detailed TSS area, it can be a set as a list of cells. Hence, a CHOICE structure is proposed for the TSS report, either in a RAN node level, or a RAN area level TSS (in terms of cells).

Below provides an exemplary tabular structure.

#### 9.3.1.XX1 RAN Timing Synchronisation Status Information Scope This IE indicates the time synchronization status information of the NG-RAN node as defined in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE TSS Information; M. >RAN node level. >> RAN Timing Synchronisation Status Information; M; 9.3.1.X3. >RAN node area level. >>RAN Timing Synchronization Status Area List; M; 9.3.1.xx2. #### 9.3.1.XX2 RAN Timing Synchronization Status Area List This IE indicates the time synchronization status list of the NG-RAN node as defined in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Timing Synchronization Status Area Item; 1..<maxnoofTSSItems>. >Cell List Areas; 1..<maxnoofCelllistareas>. >>NG-RAN CGI; M; 9.3.1.73. > RAN Timing Synchronisation Status Information; M; 9.3.1.X3.

- For NGAP, introduce a CHOICE structure for RAN timing synchronization status report (either node level or cell list level) from the gNB (gNB-CU) to the AMF. While there is no such need for F1AP.

## 2.3 TSS parameters encoding

In the SA2 agreed CR [4], the TSS information elements included in the RAN timing synchronization status is updated as follows:

Table 5.27.1.12-1: Information elements that gNB or UPF/NW-TT timing synchronization status information may contain (all optional) Information Name; Description. Synchronization state; Indicates the state of the node synchronization, represented by the values "Locked", "Holdover", or "Freerun" (NOTE 1).. Clock quality. >> Traceable to GNSS; Indicates whether the current time source is traceable to the GNSS and represented by values “Yes” or “No”. >> Traceable to UTC; Indicates whether the current time source is traceable to the UTC and represented by values “Yes” or “No”. >> Frequency stability; Describes the estimate of the variation of the local clock when it is not synchronized to another source (NOTE 2).. >> PTP clockClass; The attribute is defined in Table 5.28.3.1-2 and available under the assumption that the nodes is synchronized using (g)PTP.. >> Clock Accuracy; Describes the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock (NOTE 3). Parent time source; Describes the primary source the node is currently using, represented by the values "PTP", "GNSS", "atomic clock", "terrestrial radio", "serial time code", "NTP", "hand_set", "other".. NOTE 1: Clock is in the "Locked", "Holdover", or "Freerun" mode, as defined in ITU-T G.810 [164]. NOTE 2: Frequency stability is estimated in a similar manner as for offsetScaledLogVariance attribute defined in clause 7.6.3.5 of IEEE Std 1588 [126]. NOTE 3: Clock accuracy measurement considers accuracy up to gNB antenna and RAN internal process..

Below provides the encoding format for each TSS attribute.

- Frequency Stability

This IE refers to the offsetScaledLogVariance attribute defined in clause 7.6.3.5 of IEEE Std 1588. As clarified in the following note excerpted from the IEEE Std 1588 specification, it can be observed that the smallest value and the largest value of offsetScaledLogVariance are 0 (corresponds to 000016) and 65534 (corresponds to FFFE16). Then the Frequency Stability can take 2byte integer value (i.e. from 0 to 65534), and refer to the IEEE Std 1588 specification.

NOTE 2—The smallest variance that can be represented is 2–128 or ~3 × 10–39 s2, which results in an offsetScaledLogVariance of 000016. The maximum variance that can be represented is ~2+127.99219, which results in an offsetScaledLogVariance of FFFE16.

- PTP clockClass

As shown in Table 4 in clause 7.6.2.5 of IEEE Std 1588, the PTP clockClass can take a value from 0 to 255. Then the PTP clockClass can take an integer value (i.e. from 0 to 255), and refer to the IEEE Std 1588 specification.

- Clock Accuracy

In clause 7.6.2.6 of IEEE Std 1588, the value of clockAccuracy can take the enumeration value in the following table. Hence, the Clock Accuracy can refer to Table 5 of clause 7.6.2.6 of IEEE Std 1588 specification, and take an integer value from 0 to 255.

Table 5—clockAccuracy enumeration Value (hex); Specification. 00 to 16; Reserved. 17; The time is accurate to within 1 ps. 18; The time is accurate to within 2.5 ps. 19; The time is accurate to within 10 ps. 1A; The time is accurate to within 25 ps. 1B; The time is accurate to within 100 ps. 1C; The time is accurate to within 250 ps. 1D; The time is accurate to within 1 ns. 1E; The time is accurate to within 2.5 ns. 1F; The time is accurate to within 10 ns. 20; The time is accurate to within 25 ns. 21; The time is accurate to within 100 ns. 22; The time is accurate to within 250 ns. 23; The time is accurate to within 1 µs. 24; The time is accurate to within 2.5 µs. 25; The time is accurate to within 10 µs. 26; The time is accurate to within 25 µs. 27; The time is accurate to within 100 µs. 28; The time is accurate to within 250 µs. 29; The time is accurate to within 1 ms. 2A; The time is accurate to within 2.5 ms. 2B; The time is accurate to within 10 ms. 2C; The time is accurate to within 25 ms. 2D; The time is accurate to within 100 ms. 2E; The time is accurate to within 250 ms. 2F; The time is accurate to within 1 s. 30; The time is accurate to within 10 s. 31; The time is accurate to >10 s. 32 to 7F; Reserved. 80 to FD; Designated for assignment by alternate PTP Profiles. FE; Unknown. FF; Reserved.

Besides, in RRC specification, the uncertainty parameter contained in the ReferenceTimeInfo IE also indicates the accuracy of the 5G clock. Alternatively, the Clock Accuracy can refer to the uncertainty parameter contained in the ReferenceTimeInfo IE, i.e. take an integer value from 0 to 32767 and in the unit of 25ns. Hence, the Clock Accuracy can take following values  for the Choice structure:

- Choice 1: Refer to Table 5 of clause 7.6.2.6 of IEEE Std 1588 specification, and take an integer value from 0 to 255.

- Choice 2: Take an integer value from 0 to 32767 and in the unit of 25ns.

- Parent Time Source

As observed in the SA2 CR [3], ‘syncE’ has been removed from the values of Parent Time Source. Accordingly, it can be removed from the BL CRs.

Based on the above discussion, the detailed TSS parameters are proposed as follows.

- The Frequency Stability can take an Integer value from 0 to 65534, and can refer to the IEEE Std 1588 specification.

- The PTP clockClass can take an integer value (i.e. from 0 to 255), and refer to Table 4 in clause 7.6.2.5 of the IEEE Std 1588 specification.

- The Clock Accuracy includes the following two choices:

- Choice 1: Refer to Table 5 of clause 7.6.2.6 of IEEE Std 1588 specification, and take an integer value from 0 to 255.

- Choice 2: Take an integer value from 0 to 32767 and in the unit of 25ns.

- Remove the ‘syncE’ value for Parent Time Source.

## 2.4 AMF providing clock quality reporting control information per-UE to the gNB

The SA2 CR [3] captures the following descriptions about the clock quality reporting control information.

For 5G access stratum time synchronization service, clock quality reporting control information manages the gNBs timing synchronization status reporting to the UE. When AMF provides the 5G access stratum time distribution indication and the Uu time synchronization error budget to gNB, the AMF also includes the clock quality reporting control information (CQRCI) provided by the TSCTSF or retrieved from UDM. CQRCI may be a part of Access and Mobility Subscription data at the UDM, and AF may include CQRCI in its request. CQRCI contains the following fields: - Clock quality detail level. It indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication. - If the clock quality detail level equals "clock quality metrics", the NG-RAN provides clock quality metrics to the UE that reflect its current timing synchronization status. Clock quality metrics refers to one or more of the following information elements: clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state as defined in Table 5.27.1.12-1. - If the clock quality detail level equals "acceptable/not acceptable indication", clock quality acceptance criteria for the UE. The gNB provides an acceptable indication to the UE if the gNB's timing synchronization status matches the acceptance criteria received from the AMF; otherwise, the gNB indicates "not acceptable" to the UE. Clock quality acceptance criteria can be defined based on one or more information elements listed in Table 5.27.1.12-1 with the exception of PTP clockClass. If AF includes clock quality acceptance criteria in its request towards TSCTSF, the AF shall be notified about the result once TSCTSF determines whether the clock quality acceptance criteria can be met or not. Based on the notification, the AF may decide to modify the service if preferred (e.g., disable the service upon degradation or enable it again upon recovery).

It can be observed the clock quality reporting control information includes Clock quality detail level, and can take one value from clock quality metrics or acceptable/not acceptable indication.

- If Clock quality detail level takes the value of clock quality metrics, the RAN node can provide the detailed 5G Clock quality information to the UEs. A bitmap IE can be indicated as the quality metrics, in which each bit corresponds to one clock quality metric.

- If Clock quality detail level takes the value of acceptable/not acceptable indication, the AMF shall also provide the clock quality acceptance criteria. Since the Clock Quality Acceptance Criteria will be designed with the CN, e.g., between the TSCTSF and AF, its IE type can be set to octet string, and the exact encodings can be left to CT4.

- About the per-UE clock quality reporting control information,

- A bitmap is introduced to indicate the exact clock quality metrics, in which each bit corresponds to one clock quality metric;

- The Clock Quality Acceptance Criteria is set as Octet string and the details up to other groups.

## 2.4 Potential LS to RAN2

As agreed by RAN2-120bis-e meeting,

- RRC_CONNECTED UE to be informed of 5G clock quality information via dedicated RRC message

In order to assist RAN2 progress, a LS to RAN2 is needed to indicate the clock quality reporting control information. Typically,

- When the clock quality detail level equals “clock quality metrics”, RAN3 can indicate to RAN2 about the detailed clock quality formats for all possible metrics including clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state etc.

- When the clock quality detail level equals "acceptable/not acceptable indication", the gNB just notifies the UE acceptable/not acceptable indication.

Based on the above, RAN2 can design the dedicated RRC message for connected UE about the 5G clock information at the first stage.

- In addition, RAN2 agreed the event ID is used for the RRC inactive/idle UE without any scope ID since the gNB ID can be used to represent the scope of the report ID. In the case where different DUs have different 5G Clock quality information, the “Event ID” can be different for different DUs configured by the OAM. Thus it seems no need to contact with the RAN2.

- Send a LS to RAN2 about the details of the 5G Clock quality information delivered to the connected UE to assist RAN RRC signalling design.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

- For NGAP, for gNB reporting TSS to the AMF, take solution 2A as starting point, then discuss if the AMF triggered procedure is needed.

- For F1AP, introduce a new class 2 procedure to report the node-level RAN timing synchronization status reporting from the gNB-DU to the gNB-CU, as a starting point.

- For NGAP, introduce a CHOICE structure for RAN timing synchronization status report (either node level or cell list level) from the gNB (gNB-CU) to the AMF. While there is no such need for F1AP.

- The Frequency Stability can take an Integer value from 0 to 65534, and can refer to the IEEE Std 1588 specification.

- The PTP clockClass can take an integer value (i.e. from 0 to 255), and refer to Table 4 in clause 7.6.2.5 of the IEEE Std 1588 specification.

- The Clock Accuracy includes the following two choices:

- Choice 1: Refer to Table 5 of clause 7.6.2.6 of IEEE Std 1588 specification, and take an integer value from 0 to 255.

- Choice 2: Take an integer value from 0 to 32767 and in the unit of 25ns.

- Remove the ‘syncE’ value for Parent Time Source.

- About the per-UE clock quality reporting control information,

- A bitmap is introduced to indicate the exact clock quality metrics, in which each bit corresponds to one clock quality metric;

- The Clock Quality Acceptance Criteria is set as Octet string and the details up to other groups.

- Send a LS to RAN2 about the details of the 5G Clock quality information delivered to the connected UE to assist RAN RRC signalling design.

The TPs to TS 38.413, TS 38.473 and TS 38.423 are captured in Annexes respectively.  Furthermore, an LS to RAN2 is provided in the Annex D.

# 4. Reference

- RP-230754, New WID on NR Timing Resiliency and URLLC enhancements, 3GPP TSG RAN Meeting #99.

- R2-230xxxx, Report of 3GPP TSG RAN WG2 meeting #121bis-e, Online, 3GPP TSG-RAN WG2 meeting #122.

- R3-231971 Summary of Offline Discussion for CB # URLLC_RANenh, Nokia (moderator)

- S2-2306224, Addressing ENs on timing synchronization status reporting, Ericsson et al.

- S2-2306225, Procedures to support network timing synchronization status and reporting, Nokia et al

- R3-230811, Reply LS on proposed method for time synchronization status reporting to UE(s), 3GPP TSG-RAN WG3 Meeting #119.

# Annex A – TP to TS 38.413

Change Begins

<Unchanged Text Omitted>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

<Unchanged Text Omitted>

[50]	3GPP TS 23.203: "Policy and charging control architecture".

[51]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[52]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[AA]	IEEE Std 1588: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems", Edition 2019.

<Unchanged Text Omitted>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

ACL	Access Control List

AMF	Access and Mobility Management Function

CAG	Closed Access Group

CGI	Cell Global Identifier

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DC	Dual Connectivity

DL	Downlink

EPC	Evolved Packet Core

FN-RG	Fixed Network Residential Gateway

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coax

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

N3IWF	Non 3GPP InterWorking Function

NB-IoT	Narrow Band Internet of Things

NID	Network Identifier

NGAP	NG Application Protocol

NPN	Non-Public Network

NRPPa	NR Positioning Protocol Annex

NSAG	Network Slice AS Group

NSCI	New Security Context Indicator

NSSAI	Network Slice Selection Assistance Information

OTDOA	Observed Time Difference of Arrival

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity Services

PSCell	Primary SCG Cell

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RSN	Redundancy Sequence Number

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SgNB	Secondary gNB

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

TAC	Tracking Area Code

TAI	Tracking Area Identity

TSS	Timing Synchronization Status

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNLA	Transport Network Layer Association

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

UL	Uplink

UP	User Plane

UPF	User Plane Function

V2X	Vehicle-to-Everything

W-AGF	Wireline Access Gateway Function

WUS	Wake Up Signal

<Unchanged Text Omitted>

# 8	NGAP Procedures

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

<Unchanged Text Omitted>

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| TSS Information Transfer | TSS INFORMATION TRANSFER |

<Unchanged Text Omitted>

### 8.XX		TSS Procedures

#### 8.xx.1	TSS Information Transfer

#### 8.xx.1.1	General

The purpose of the TSS Information Transfer procedure is to transfer the RAN timing synchronization status information to the AMF. The procedure uses non-UE associated signalling.

#### 8.xx.1.2	Successful Operation

Figure 8.17.4.2-1: Broadcast Session Release Required, successful operation.

The NG-RAN node initiates the procedure by sending the TSS INFORMATION TRANSFER message to the AMF.

#### 8.xx.1.3	Abnormal Conditions

Void.

<Unchanged Text Omitted>

### 9.2.y		TSS Messages

#### 9.2.y.1	TSS INFORMATION TRANSFER

This message is sent by the NG-RAN node to provide the timing synchronization status.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Timing Synchronisation Status Information | M |  | 9.3.1.y1 |  | YES | reject |

<Unchanged Text Omitted>

#### 9.3.1.y1	Timing Synchronization Status Information

This IE indicates the time synchronization status information of the NG-RAN node as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE TSS Information | M |  |  |  |
| >RAN node level |  |  |  |  |
| >>RAN Timing Synchronisation Status Information | M |  | 9.3.1.x3 |  |
| >RAN area level |  |  |  |  |
| >>Timing Synchronization Status Area List | M |  | 9.3.1.y2 |  |

#### 9.3.1.y2	Timing Synchronization Status Area List

This IE indicates the time synchronization status list of the NG-RAN node as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Timing Synchronization Status Area Item |  | 1..<maxnoofTSSItems> |  |  |
| >Cell List Areas |  | 1..<maxnoofCelllistareas> |  |  |
| >>NG-RAN CGI | M |  | 9.3.1.73 |  |
| > RAN Timing Synchronisation Status Information | M |  | 9.3.1.X3 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTSSItems | Maximum no. of TSS area items. Value is 32 [FFS]. |
| maxnoofCelllistareas | Maximum no. of cell list items. Value is 256. |

<Unchanged Text Omitted>

#### 9.3.1.x1	Clock Quality Reporting Control Information

This IE indicates the clock quality reporting control information as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Quality Detail Level | M |  |  |  |
| >clock quality metrics |  |  |  |  |
| >>Clock Quality Metrics indication |  |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents one clock quality metric: first bit = Synchronization state, second bit = Traceable to GNSS, third bit = Traceable to UTC, fourth bit = Frequency stability, fifth bit = PTP clockClass, sixth bit = Clock Accuracy, seventh bit = Parent time source: Other bits reserved for future use. Value '1' indicates 'should be provided to the UE'. Value '0' indicates 'should not be provided to the UE'. |
| >acceptance indication |  |  |  |  |
| >>Clock Quality Acceptance Criteria | M |  | 9.3.1.x2 |  |

#### 9.3.1.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Clock Quality Acceptance Criteria | M |  | OCTET STRING | Indicates the clock quality acceptance criteria as specified in TS xx.xxx [FFS]. |

#### 9.3.1.x3	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided towards the AMF as defined in TS 23.501 [9].

Editor’s Note: The non-UE associated NGAP procedure(s) used to convey this IE towards the AMF is FFS.

Editor’s Note: This IE may be further refined based on SA2 and RAN3 progress.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | 9.3.1.x4 |  |
| Clock Accuracy | O |  | 9.3.1.x5 |  |
| PTP clockClass | O |  | 9.3.1.x6 |  |
| Parent Time Source | O |  | ENUMERATED (pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

#### 9.3.1.x4	Clock Frequency Stability

This IE indicates the clock frequency stability as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Clock Frequency Stability | M |  | INTEGER(0..65534,..) | Indicates the offsetScaledLogVariance as specified in IEEE Std 1588 [AA]. |

#### 9.3.1.x5	Clock Accuracy

This IE indicates the clock accuracy as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >Index |  |  |  |  |
| >>Accuracy index |  |  | INTEGER(0..255,..) | The integer value indicates the clockAccuracy enumeration value specified in Table 5 of clause 7.6.2.6 of IEEE Std 1588 [AA]. |
| >Value |  |  |  |  |
| >>Accuracy value | M |  | INTEGER(0..32767,..) | Indicates the absolute clock accuracy value. Unit in 25ns. |

#### 9.3.1.x6	PTP clockClass

This IE indicates the PTP clockClass as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PTP clockClass | M |  | INTEGER(0..255,..) | The integer value indicates the clockClass value specified in Table 4 in clause 7.6.2.5 of IEEE Std 1588 [AA]. |

<Unchanged Text Omitted>

Change Ends

# Annex B – TP to TS 38.473

Change Begins

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

<Unchanged Text Omitted>

[46]	3GPP TS 37.213: "NR; Physical layer procedures for shared spectrum channel access".

[47]	3GPP TS 37.483: "E1 Application Protocol (E1AP)".

[AA]	IEEE Std 1588: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems", Edition 2019.

<Unchanged Text Omitted>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

ARP	Antenna Reference Point

ARPI	Additional RRM Policy Index

BH	Backhaul

CAG	Closed Access Group

CN	Core Network

CG	Cell Group

CG-SDT	Configured Grant-Small Data Transmission

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FSA ID	MBS Frequency Selection Area (FSA) ID

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

NID	Network Identifier

NPN	Non-Public Network

NSAG	Network Slice AS Group

NSSAI	Network Slice Selection Assistance Information

PDC	Propagation Delay Compensation

PEIPS	Paging Early Indication with Paging Subgrouping

posSIB	Positioning SIB

PNI-NPN	Public Network Integrated NPN

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RANAC	RAN Area Code

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RRC	Radio Resource Control

RSRP	Reference Signal Received Power

SDT	Small Data Transmission

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

TEG	Timing Error Group

TRP	Transmission-Reception Point

TSS	Timing Synchronization Status

U2N	UE-to-Network

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

Z-AoA	Zenith Angles of Arrival

<Unchanged Text Omitted>

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

<Unchanged Text Omitted>

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| Multicast Group Paging |  | MULTICAST GROUP PAGING |  |
| Broadcast Context Release Request |  | BROADCAST CONTEXT RELEASE REQUEST |  |
| Multicast Context Release Request |  | MULTICAST CONTEXT RELEASE REQUEST |  |
| PDC Measurement Report |  | PDC MEASUREMENT REPORT |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| PDC Measurement Failure Indication |  | PDC MEASUREMENT FAILURE INDICATION |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| Measurement Activation |  | MEASUREMENT ACTIVATION |  |
| QoE Information Transfer |  | QOE INFORMATION TRANSFER |  |
| Positioning System Information Delivery |  | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |  |
| TSS Information Transfer |  | TSS INFORMATION TRANSFER |  |

<Unchanged Text Omitted>

## 8.xx	TSS Procedures

### 8.xx.1	TSS Information Transfer

#### 8.xx.1.1	General

The purpose of the TSS Information Transfer procedure is to transfer the RAN timing synchronization status information from the gNB-DU to the gNB-CU. The procedure uses non-UE associated signalling.

#### 8.xx.1.2	Successful operation

Figure 8.16.1.2-1: TSS Information Transfer procedure

The gNB-DU initiates the procedure by sending the TSS INFORMATION TRANSFER message to the gNB-CU.

#### 8.xx.1.3	Abnormal Conditions

Not applicable.

<Unchanged Text Omitted>

### 9.2.y	TSS messages

#### 9.2.y.1	TSS INFORMATION TRANSFER

This message is sent by a gNB-DU to a gNB-CU, to indicate the RAN timing synchronization status.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Timing Synchronisation Status Information | M |  | 9.3.1.x1 |  | YES | reject |

<Unchanged Text Omitted>

#### 9.3.1.x1	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided from the gNB-DU to the gNB-CU.

Editor’s Note: The non-UE associated F1AP procedure(s) used to convey this IE towards the gNB-CU is FFS.

Editor’s Note: This IE may be further refined based on SA2 and RAN3 progress.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | 9.3.1.x2 |  |
| Clock Accuracy | O |  | 9.3.1.x3 |  |
| PTP clockClass | O |  | 9.3.1.x4 |  |
| Parent Time Source | O |  | ENUMERATED (pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

#### 9.3.1.x2	Clock Frequency Stability

This IE indicates the clock frequency stability as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Clock Frequency Stability | M |  | INTEGER(0..65534,..) | Indicates the offsetScaledLogVariance as specified in IEEE Std 1588 [AA]. |

#### 9.3.1.x3	Clock Accuracy

This IE indicates the clock accuracy as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >Index |  |  |  |  |
| >> Accuracy index |  |  | INTEGER(0..255,..) | The integer value indicates the clockAccuracy enumeration value specified in Table 5 of clause 7.6.2.6 of IEEE Std 1588 [AA]. |
| >Value |  |  |  |  |
| >>Accuracy value | M |  | INTEGER(0..32767,..) | Indicates the absolute clock accuracy value. Unit in 25ns. |

#### 9.3.1.x4	PTP clockClass

This IE indicates the PTP clockClass as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PTP clockClass | M |  | INTEGER(0..255,..) | The integer value indicates the clockClass value specified in Table 4 in clause 7.6.2.5 of IEEE Std 1588 [AA]. |

Change Ends

# Annex C – TP to TS 38.423

Change Begins

<Unchanged Text Omitted>

#### 9.2.3.x1	Clock Quality Reporting Control Information

This IE indicates the clock quality reporting control information as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Quality Detail Level | M |  |  |  |
| >clock quality metrics |  |  |  |  |
| >>Clock Quality Metrics indication |  |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents one clock quality metric: first bit = Synchronization state, second bit = Traceable to GNSS, third bit = Traceable to UTC, fourth bit = Frequency stability, fifth bit = PTP clockClass, sixth bit = Clock Accuracy, seventh bit = Parent time source: Other bit reserved for future use. Value '1' indicates 'should be provided to the UE'. Value '0' indicates 'should not be provided to the UE'. |
| >acceptance indication |  |  |  |  |
| >>Clock Quality Acceptance Criteria | M |  | 9.3.1.x2 |  |

#### 9.2.3.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Clock Quality Acceptance Criteria | M |  | OCTET STRING | Indicates the clock quality acceptance criteria as specified in TS xx.xxx [FFS]. |

Change Ends

# Annex D: LS to RAN2

3GPP TSG-RAN WG3 Meeting #120-e R3-23xxxx ncheon, KR, 22 May – 26 May, 2023 Title: [Draft] LS on 5GS network timing synchronization status and reporting Response to: Release: Release 18 Work Item: TRS_URLLC-NR-Core Source: Huawei [will be RAN3] To: RAN2 Cc: Contact Person: #### Name: Feng Han E-mail Address: hanfeng3@huawei.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: # 1 Overall description RAN3 discussed the 5GS network timing synchronization status and reporting, and made the following agreements. Regarding the 5G clock quality information delivered to the UE, there are two kinds of indications as follows. • Clock quality metrics: Indicates the detailed 5G clock attributes including: - Synchronization state: Indicates the state of the node synchronization, represented by the values “Locked”, “Holdover”, or “Freerun”. - Synchronization performance: Boolean true/false for Traceable to UTC or Traceable to GNSS - Clock accuracy: - Choice 1:an integer value from 0 to 255 and referring to the IEEE Std 1588 specification. - Choice 2: an integer value from 0 to 32767 and in the unit of 25ns. - Frequency Stability: an Integer value from 0 to 65534, and referring to the IEEE Std 1588 specification - PTP clockClass: an integer value (i.e. from 0 to 255), and referring to Table 4 in clause 7.6.2.5 of the IEEE Std 1588 specification - Parent time source: Describes the primary source the node is currently using, represented by the values, “PTP”, “GNSS”, “atomic clock”, “terrestrial radio”, “serial time code”, “NTP”, “hand set”, “other”. - Frequency stability: the meaning of this information is FFS dependent SA2 progress. • Acceptable indication: Indicates whether the NG-RAN's timing synchronization status matches the acceptance criteria received from 5GC, represented by the values “acceptable”, “not acceptable”. # 2 Actions To RAN2: ACTION: RAN3 respectfully asks RAN2 to take the above into account for RRC signalling design. # 3 Dates of next RAN3 meetings Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/ RAN3#121 2023-08-21 - 2023-08-25 Toulouse , FR RAN3#121bis 2023-10-09 - 2023-10-13 Xiamen , CN