3GPP TSG-RAN WG3 #120								R3-233295

Incheon, Korea, May 22-26, 2023

Agenda Item:	20.2

Source:	China Telecom

Title: 	Discussion on Remaining Issues for RAN Paging

Document for:	Discussion

Introduction

In RAN3#119bis meeting, the following agreements and open issues on MT-SDT procedure were listed below:

RAN3 acknowledges the case that DL non-SDT data arrives at the last serving gNB following the MT-SDT paging procedure before receiving UE Context Retrieval Request message. It is FFS whether it is left to gNB implementation, or reusing existing IE(s), or introducing a new IE.

In XnAP: RAN Paging message includes the MT-SDT Data Size IE. FFS on the MT-SDT indicator IE and the presence of MT-SDT Data Size IE.

In this paper, we will further discuss the remaining issues for RAN Paging.

Discussion

- Detail of the MT-SDT information

Once DL data packets for MT-SDT DRBs or NAS signalling arrived at anchor gNB, the MT-SDT information shall be contained in RAN paging message was agreed. With MT-SDT information, the receiving gNB can determine whether to send RRC paging message to UE depends on its own DL data volume threshold. However, the MT-SDT the encoding and name of MT-SDT information IE is FFS. In last RAN3 meeting, both DL SDT user data and DL SDT signalling can trigger MT-SDT procedure. The MT-SDT information need to contain the following information:

- MT-SDT indication: only applicable for DL SDT signalling

- Buffered data size for MT-SDT: only applicable for DL SDT user data

If the MT-SDT is triggered by DL SDT signalling, the MT-SDT indication is enough. Therefore, the MT-SDT information contains one MT-SDT indication, which is only applicable for DL SDT signalling, and buffered data size for MT-SDT, which is only applicable for DL SDT user data.

Proposal 1: the MT-SDT information contains one MT-SDT indication, which is only applicable for DL SDT signalling, and buffered data size for MT-SDT, which is only applicable for DL SDT user data.

In addition, in E1AP, the range of SDT data size was agreed in (1…96000, …). Therefore, the max data size for SDT data size should be align with that in E1AP.

Proposal 2: The range for MT-SDT data size is (1…96000, …).

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: the MT-SDT information contains one MT-SDT indication, which is only applicable for DL SDT signalling, and buffered data size for MT-SDT, which is only applicable for DL SDT user data.

Proposal 2: The range for MT-SDT data size is (1…96000, …).

Reference

- RP-213583，New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR，ZTE Corporation