3GPP TSG-RAN WG3 Meeting #120
          R3-233315
Incheon, KR,  22th May- 26th May 2023
Agenda Item:
9.1

Source:
ZTE(Moderator)

Title:
Summary of Offline Discussion on CB: # 4_QoEAreaScope
Document for:
Approval

Introduction

CB: # 4_QoEAreaScope

- Align LTE and NR with the same mechanism?

- Check the details and coversheet

(moderator - ZTE)

Summary of offline disc R3-233315
Structure of this CB discussion:

In the round 1 discussion, moderator prefers to firstly locate the issue in NR and/or LTE.Then detail correction can be discussed in round 2 if needed.

For the round 1 discussion, please provide your comments before 23:59pm, May 23th(Tuesday), Incheon local time.
 For the Chairman’s Notes

TBD
Discussion
QoE area scope IE in NR

2 companies provided the CRs on the correction of the QoE area scope with various reasons shown below:

In [9], to consider the usability of the LocationFilter, the Area Scope IE in QoE configuration IE shall be corrected to optional in NGAP.

In [7] and [8], to fulfill RAN3 previous agreement shown below and to align the area scope definition in both SA5 spec and RAN3 NR spec, the area scope IE for QoE configuration shall be correct to optional in NGAP. 

In the NGAP and XnAP QoE IE, the presence of Area Scope IE should be O, and maxnoofUEAppLayerMeas should be = 16
In addition, similar to what RAN3 has agreed in previous meeting on MDT area scope IE correction[10], the procedural text for the receiver node behaviour shall also be provided when the IE is not present in both NGAP and XnAP. 
Q1.1:Please provide your views on whether the area scope IE in NR shall be correct as optional in NGAP.

	Company
	Yes or No 
	Comment

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


Q1.2: Please provide your views on whether to add procedural text for the QoE area scope IE in both NGAP and XnAP to specify the receiver node behaviour when the IE is not present. 

	Company
	Yes or No 
	Comment

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


QoE area scope IE in LTE

The area scope issue in QoE field also occurs in LTE specs. Currently, the presence of the area scope IE is mandatory in both TS 36.413 and TS 36.423. Meanwhile, in SA5 specifications like TS 28.405, it clearly explains that:

If the parameter is not present the QMC shall be done throughout the PLMN specified in PLMN target.

Hence, misalignment exists between RAN3 LTE spec and SA5 spec. Serial contributions also provided for TS 36.413 and TS 36.423 to fix this misalignment. Similar to what RAN3 has done in MDT aspects, a PLMN wide option is added into the mandatory QoE Area Scope IE(CHOICE structure).
Q2.1 Please provide your views on whether the misalignment exists between RAN3 LTE spec(e.g. TS 36.413, TS 36.423) and SA5 spec(e.g. TS 28.405).

	Company
	Yes or No 
	Comment

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


Q2.2: If the answer in Q2.1 is yes, please provide your views on how to fix this misalignment.

	Company
	Comment

	
	

	
	

	
	

	
	

	
	

	
	

	
	


Q2.3: If the answer in Q2.1 is no, please provide your views on how does the receiver node handle the QoE configuration without area scope IE in S1AP and/or X2AP messages based on the current spec.

	Company
	Comment

	
	

	
	

	
	

	
	

	
	

	
	

	
	


Any other issue

Companies may provide any other issues in this section.

Q3. Please provide any other views if any.

	Company
	Comment

	
	

	
	

	
	

	
	

	
	

	
	

	
	


References

R3-233267
Discussion on QoE area scope IE in NR and LTE (ZTE, China Telecom, CMCC)
Discussion

R3-233262
Correction on the QoE Area Scope IE in UE Application layer measurement configuration (ZTE, China Telecom, CMCC)
CR1913r, TS 36.413 v15.11.0, Rel-15, Cat. F

R3-233265
Correction on the QoE Area Scope IE in UE Application layer measurement configuration (ZTE, China Telecom, CMCC)
CR1752r, TS 36.423 v16.10.1, Rel-16, Cat. A

R3-233263
Correction on the QoE Area Scope IE in UE Application layer measurement configuration (ZTE, China Telecom, CMCC)
CR1751r, TS 36.423 v15.13.0, Rel-15, Cat. F

R3-233264
Correction on the QoE Area Scope IE in UE Application layer measurement configuration (ZTE, China Telecom, CMCC)
CR1914r, TS 36.413 v16.12.0, Rel-16, Cat. A

R3-233266
Correction on the QoE Area Scope IE in UE Application layer measurement configuration (ZTE, China Telecom, CMCC)
CR1915r, TS 36.413 v17.4.0, Rel-17, Cat. A

R3-233268
Correction on the QoE Area Scope IE in QMC Configuration Information (ZTE, China Telecom, CMCC)
CR1001r, TS 38.413 v17.4.0, Rel-17, Cat. F

R3-233293
Correction on the QoE Area Scope IE in QMC Configuration Information (ZTE, China Telecom, CMCC)
CR1067r, TS 38.423 v17.4.0, Rel-17, Cat. F

R3-233129
Correction on the presence of Area Scope of QMC over NGAP (Qualcomm Technologies Int)
CR0997r, TS 38.413 v17.4.0, Rel-17, Cat. F

R3-232097
Correction on the Area Scope IE in MDT Configuration (Huawei, CMCC, China Unicom) CR1034r, TS 38.423 v16.13.0, Rel-16, Cat. F
