3GPP TSG-RAN WG3 #120																			R3-23xxxx

Incheon, 22nd – 26th May 2023

Agenda Item:	14.4

Source:	ZTE

Title:	(TP to TS 38.423): Support of SCG selective activation

Document for:	Discussion and Approval

- Introduction

To reflect agreements made at RAN3#120 meeting.

- Text Proposal

  - TP to TS 38.423

--------------------------------------------------Start of the change---------------------------------------------------

9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name |  | Presence | Range |  | IE type and reference |  | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  | 9.2.3.1 |  |  | YES | reject |
| M-NG-RAN node UE XnAP ID |  | M |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID |  | M |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | Allocated at the S-NG-RAN node | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |  |  |  |
| Conditional PSCell Change Information Required |  | O |  |  |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  |  | 1 |  |  |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  |  | 1 .. <maxnoofTargetSNs> |  |  |  |  | – |  |
| >>>Target S-NG-RAN node ID |  | M |  |  | Global NG-RAN Node ID 9.2.2.3 |  |  | – |  |
| >>>CPC Indicator |  | M |  |  | ENUMERATED (CPC-initiation, CPC-modification, CPC-cancellation, ...) |  |  | – |  |
| >>>Maximum Number of PSCells To Prepare |  | M |  |  | INTEGER (1..8, …) |  | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>Estimated Arrival Probability |  | O |  |  | INTEGER (1..100) |  | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container |  | M |  |  | OCTET STRING |  | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. |  |  |
| >Selective Activation Information Required | O |  |  | 9.2.3.xxx |  | This information is used for SCG selective activation. FFS on the details. Editor’s Note: This may be revisited based on RAN2 progress. |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

-----------------------------------------------------End of the change-------------------------------------------------------