3GPP TSG-RAN Meeting #121bis	R3-235135

Xiamen, China, 9th – 13th October 2023

Agenda Item:	14.2

Source:	CATT

Title:	Discussion about candidate cell RS configuration

Document for:	Discussion

Introduction

Discussion

How the CU acquire candidate configuration

For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU.

Agree to add the RS configuration in UE Context Setup/Modification in stage2 and stage3.

The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations.

Last meeting, we get a conclusion that after CU acquire cell RS-configuration of all the candidate cells, it will send them to all the related DUs for requesting the RS report configuration. Only after that, the CU is able to get the completely LTM candidate cell configuration.

One thing still not clearly is how the candidate cells transfer the candidate cell RS-configuration to CU, by a UE associate message or by a non-UE associate message. For getting the completely LTM candidate cell configuration, there are three possible method.

Fig 1 Current procedure in BLCR

Method 1: Using UE associated message for requesting RS configuration(step 3/4), and UE associated message for acquiring the RS report configuration(step 7/8).

This method is merged in current BLCR. CU uses UE-associate message to request RS-configuration and other information (e.g., incompletely CellGroupConfig, TCI information, RACH information) from candidate DU in step 3, and candidate DU replies it in step4. And then, after the CU collected the RS-configuration from all the candidate cells, CU transfers it to each candidate DU by step 7, and candidate DU provide the updated CellGroupConfig including RS report configuration to the CU.

There is no doubt the method 1 is the most straight forward way. However, this method looks a little complex in RAN3. For acquiring complete candidate cell configuration, CU needs to trigger two round trips of parallel procedures, per candidate per UE.

Observation 1: for method 1

Advantage: the most straight forward way, support further enhancement, e.g., CSI-RS.

Disadvantage: the most complex from RAN3 perspective, for each cell each UE, CU need trigger two round trip of parallel procedure.

Action: nothing

Method 2: using UE associated message to collection RS configuration (step 3/4), and UE associated message for acquiring the RS report configuration (step 7/8)..

The method 2 needs request RAN2 placing measurement report configurations for candidate cells outside of LTM candidate cell configurations. Thus, it will be placed out of CellGroupConfig in CU to DU RRC Information.

So, CU uses UE-associate message to request RS-configuration and other information (e.g., CellGroupConfig, TCI information, RACH information) from candidate DU in step 3, and candidate DU replies it in step 4. And then, after the CU collected the RS-configuration from all the candidate cells, CU transfers it to each candidate DU in step 7 by a non-UE associated message, requesting the RS report configuration for all the candidate cells in the same DU.

This method can make RAN3 signaling designation simply, no need to trigger two round trips of parallel procedures. Also, this method can support further enhancement as it is UE specific when requesting RS configuration. However, this method is against RAN2 agreement. The intention for RAN2 to design the RRC structure is used for UE configuration, and it not very reasonable to ask RAN2 to modify their specific due to it not easy to use in RAN3. And what’s more, the RS report configuration will be used only if the candidate cell turns to serving cell, so, it is reasonable for RAN2 to place measurement report configurations for candidate cells inside of serving cell configurations.

Observation 2: for method 2

Advantage: this method can reduce signaling overload in RAN3, and can support further enhancement, e.g., CSI-RS.

Disadvantage: this method is not aligning with RAN2 previous agreement, and it not very reasonable to let RAN2 change their RRC structure due to it not easy to use in RAN3.

Action: send LS to RAN2, and request them to place measurement report configurations for candidate cells outside of LTM candidate cell configurations

Method 3: using non-UE associated message to collect RS configuration (before step3/4), and UE associate message to transfer RS report configuration (step3/4).

Fig 2 Method 3

Use non-UE associate message to request the RS information for all the cells served in this DU, e.g., at the F1 setup phase, thus no need step 3-4 anymore. And after CU get all the RS configurations, using UE-associate message(UE CONTEXT SETUP REQUEST) to carry the RS information to candidate DU and request the RS report configuration.

This method is aligning with RAN2 agreement, and also can reduce signaling overload. However, it has two drawback: this method is only can work when the RS configuration is cell specific; As there is no admitted control before provide RS report configuration, a candidate cell may generate RS report configuration for a non-admitted cells.

Observation 3: for method 3

Advantage: this method is aligning with RAN2 agreement, and can reduce signaling overload in RAN3.

Disadvantage: this method can not further enhance for CSI-RS, and a candidate cell may generate RS report configuration for a non-admitted cells

Action: add the new LTM candidate cell RS configuration defined in RAN2 to F1 setup message; introduce an indicator to denote whether need request RS configuration in F1 setup procedure.

All in all, method 2 will have huge impact on RAN2, and it not suitable for RAN3 to request RAN2 modify their RRC structure. And if we really send LS to RAN2, it will re-open their discussion and can not guarantee this WI can finish on time. For the method 3, it is hard to enhancement in further study and may cause resource waste without admission control before prepare RS report configuration.

When back to method 1, seems it can be seen as an acceptable method, the largest drawback is the complexity signaling interactions, and will introduce many many parallel procedures. But is RAN3 really care about it? First, it only some same parallel existing message, no need to introduce new message; Secondly, the signaling overload is not a very serious problem in interface; and last, this procedure is in candidate cells preparation phase, parallel signlling not have impact on the execute delay of LTM. So, based on the above, we suggest to use the method 1 for acquire completely candidate cell configuration. Using UE associated message at step 3-4 for request RS configuration, and UE associated message at step 7-8 to request the RS report configuration.

Proposal 1: Using UE associated message at step 3-4 for request RS configuration, and UE associated message at step 7-8 to request the RS report configuration.

Reply LS about execution

In last meeting, RAN2 send the LS[1] about two approaches during execution back. And in the reply LS, is suggest to set approach 1 to be the baseline, while the support of Approach 2 could be up to the network implementation without any specification impact.

During RAN2 discussion, seems no explicit parameter has been identified need to transfer immediately before triggering the LTM command (approach 2). So, we also suggest setting approach 1 as baseline; whether support approach 2 is pend on network implementation. What’s more, it is better to discuss approach 2 until specific use case is identified, e.g., a parameter needs dynamic interaction before the LTM cell switch.

Proposal 2: Set approach 1 to be the baseline, and discuss approach 2 until specific use case is identified.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Observation 1: for method 1, two UE associated message

Advantage: the most straight forward way, support further enhancement, e.g., CSI-RS.

Disadvantage: the most complex from RAN3 perspective, for each cell each UE, CU need trigger two round trip of parallel procedure.

Action: nothing

Observation 2: for method 2, UE associate message and one non-UE associate message

Advantage: this method can reduce signaling overload in RAN3, and can support further enhancement, e.g., CSI-RS.

Disadvantage: this method is not aligning with RAN2 previous agreement, and it not very reasonable to let RAN2 change their RRC structure due to it not easy to use in RAN3.

Action: send LS to RAN2, and request them to place measurement report configurations for candidate cells outside of LTM candidate cell configurations.

Observation 3: for method 3, non-UE associate message and one UE associate message

Advantage: this method is aligning with RAN2 agreement, and can reduce signaling overload in RAN3.

Disadvantage: this method can not further enhance for CSI-RS, and a candidate cell may generate RS report configuration for a non-admitted cells

Action: add the new LTM candidate cell RS configuration defined in RAN2 to F1 setup message; introduce an indicator to denote whether need request RS configuration in F1 setup procedure.

Proposal 1: Using UE associated message at step 3-4 for request RS configuration, and UE associated message at step 7-8 to request the RS report configuration.

Proposal 2: Set approach 1 to be the baseline, and discuss approach 2 until specific use case is identified.

# 4. Reference

[1] R3-235014, Reply LS on Approaches during execution for inter-DU LTM