3GPP TSG-RAN WG3 Meeting #121bis	R3-235243

Xiamen, China, 9th – 12th October, 2023

Agenda Item:	13.2

Source: 	Lenovo

Title: 	(TP to TS 38.420 BL CR) Discussion on mobile IAB-MT handover

Document for:	Discussion and decision

- Introduction

A new WID for mobile IAB-node [1] has been approved to be discussed in Rel-18, and following detail objectives are identified mainly focus on RAN3 and RAN2.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. - Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. - Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, we continue to discuss the general principles of mobile IAB-node inter-donor topology adaptation, including consecutive IAB-MT handover and NG-based IAB-MT handover.

- Discussion

Consecutive mIAB-MT handover

RAN3 118: The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the: • gNB ID of the target donor CU for the mIAB-MT HO. • ID(s) of the mIAB-MT. How the mIAB-MT ID is maintained across migrations needs to be further discussed. The mIAB-MT ID sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU is the XnAP UE ID. FFS which donor generates this ID. RAN3 119bis-e: For consecutive partial migration, after the mIAB-MT HO is completed, the mIAB-DU’s CU sends the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the mIAB-MT’s target CU, including the UE XnAP ID assigned to the mIAB-MT by the mIAB-MT’s target CU as the “Non-F1-Terminating IAB-donor UE XnAP ID”.

As we had agreed in RAN3 118 for consecutive IAB-MT handover, it’s the source RRC-terminating IAB-donor-CU to inform the gNB ID and XnAP UE ID to the F1-terminating IAB-donor-CU. In last meeting, some companies think a common solution may be used for IAB-MT handover and IAB-DU migration to transfer the gNB ID and XnAP UE ID. However, as agreed for IAB-DU migration, the information is included in the F1 setup request message, but the consecutive IAB-MT handover doesn’t involve the procedure for F1 setup. Therefore, the agreement shall be confirmed for consecutive IAB-MT handover.

Proposal 1: XnAP message is used to transfer gNB ID of the target RRC-terminating IAB-donor-CU and ID of the mIAB-MT from source RRC-terminating IAB-donor-CU to the F1-terminating IAB-donor-CU.

As in Figure 1, to realize consecutive partial migration, source IAB-donor A of IAB-MT HO needs to inform F1-terminating IAB-donor C that gNB ID of target IAB-donor B for the IAB-MT HO. Then the target IAB-donor B and F1-ternating IAB-donor C can perform XnAP IAB TMM procedure for the mobile IAB-node4 to transport the F1 traffic via the topology of target IAB-donor B. In addition, a UE XnAP ID also needs to be informed from the source IAB-donor A to the F1-terminating IAB-donor C to identify the mobile IAB-node4. As the F1-terminating IAB-donor C maintain the UE XnAP ID allocated by both the source IAB-donor A and F1-terminating IAB-donor C, therefore, the UE XnAP ID allocated by either source IAB-donor A or F1-terminating IAB-donor C is feasible to identify the mobile IAB-MT.

Proposal 2: UE XnAP ID allocated by source RRC-terminating IAB-donor or F1-terminating IAB-donor for mobile IAB-node is informed from the source RRC-terminating IAB-donor to the F1-terminating IAB-donor to identify the mobile IAB-MT’s Handover.

Figure 1. Example of consecutive IAB-MT handover

Editor’s NOTE: FFS if this information includes the mIAB-MT’s XnAP UE ID generated by the target RRC-terminating IAB-donor-CU during the procedure of MT handover from the source RRC-terminating IAB-donor-CU or a different ID generated by the target RRC-terminating IAB-donor-CU.

As in the IAB TMM REQUEST message, both F1-Terminating IAB-donor UE XnAP ID and Non-F1-Terminating IAB-donor UE XnAP ID are mandatory. If F1-terminating IAB-donor C initiates the IAB TMM procedure to target IAB-donor B, the F1-terminating IAB-donor C should have the UE XnAP ID assigned by the target IAB-donor B in advance. Then, we have two following options for the UE XnAP ID assigned by the target IAB-donor B.

- Option 1: Reuse the UE XnAP ID allocated by target IAB-donor B over the Xn interface between the source IAB-donor A and the target IAB-donor B.

- Option 2: A new UE XnAP ID allocated by target IAB-donor B over the Xn interface between the F1-terminating IAB-donor C and the target IAB-donor B.

- Option 3: Use BAP address allocated by target IAB-donor B to identify the mobile IAB-node and ignore the Non-F1-Terminating IAB-donor UE XnAP ID in the IAB TMM request message.

For Option 1, the same UE XnAP ID is used for two Xn interfaces. After IAB-MT handover from source RRC-terminating IAB-donor-CU to target RRC-terminating IAB-donor-CU, the UE XnAP ID may need to be released as legacy upon receiving the UE context release message. While if Option 2 (per interface UE XnAP ID) is used, there is no such problems.

And for Option 3, BAP address of the IAB-node is allocated by the target IAB-donor B after IAB-MT handover, and the BAP address needs to be informed to source IAB-donor A firstly and then delivered to F1-terminating IAB-donor C. In addition, in the IAB TMM procedure, some further clarification should be involved to an invalid value of Non-F1-Terminating IAB-donor UE XnAP ID.

Proposal 3: F1-terminating IAB-donor needs to be indicated from source IAB-donor with a UE XnAP ID allocated by target IAB-donor over the Xn interface between the F1-terminating IAB-donor and the target IAB-donor.

As in R17, a boundary IAB-node was specified for an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. While in R18, the definition for boundary IAB-node does not continue to be used. And in R18, the RRC-terminating IAB-donor-CU may be an F1-terminating IAB-donor-CU or a Non-F1-terminating IAB-donor-CU. Therefore, some descriptions for IAB related procedures in TS 38.420 need to be updated, i.e., IAB Transport Migration function and IAB Resource Coordination function.

Proposal 4: Agree the TP to TS 38.420 in Annex.

NG-based IAB-MT handover

For mobile IAB-node, IAB-MT and IAB-DU can migrate/handover to different IAB-donors, and F1-terminating IAB-donor may serve wider range than RRC-terminating IAB-donor in order to avoid frequent IAB-DU migration, which accompanies with handover for lots of UEs. From deployment point of view, an F1-terminating IAB-donor may associate with some RRC-terminating IAB-donors in a specific area, and the Xn interface can be setup in advance between the F1-terminating IAB-donor and the associated RRC-terminating IAB-donors. In addition, due to the time limitation to this release, we can only consider the scenario where there is Xn connection between F1-terminating IAB-donor and RRC-terminating IAB-donor.

Proposal 5: RAN3 assumes there is Xn connection between F1-terminating IAB-donor and RRC-terminating IAB-donor in R18.

IAB-DU migration can be divided into following 3 cases:

- Case 1: IAB-MT handover from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor A.

- Case 2: IAB-MT handover from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor B.

- Case 3: IAB-MT handover from IAB-donor A to IAB-donor B and IAB-DU stays connected to IAB-donor C.

With this assumption, both Case 1 and Case 2 are Xn-based handover. While for Case 3, the Xn connection between IAB-donor A and IAB-donor B cannot be guaranteed, but the NG-based handover procedure for UE can be reused for IAB-MT handover, and the following IAB TMM procedure and IAB resource coordination procedure can also reuse the XnAP procedures defined in R17. Therefore, with this assumption, the IAB TMM procedure and IAB resource coordination procedure don’t have to be extended to NGAP.

Proposal 6: Only considering NG-based handover for mIAB-MT in case mIAB-MT handover from IAB-donor A to IAB-donor B while mIAB-DU still connect to another IAB-donor C.

Proposal 7: IAB TMM procedure and IAB resource coordination procedure don’t have to be extended to NGAP.

# Conclusion

This contribution aims to discuss the general principles of mobile IAB-node inter-donor topology adaptation. And following observations and proposals are concluded.

Consecutive mIAB-MT handover

Proposal 1: XnAP message is used to transfer gNB ID of the target RRC-terminating IAB-donor-CU and ID of the mIAB-MT from source RRC-terminating IAB-donor-CU to the F1-terminating IAB-donor-CU.

Proposal 2: UE XnAP ID allocated by source RRC-terminating IAB-donor or F1-terminating IAB-donor for mobile IAB-node is informed from the source RRC-terminating IAB-donor to the F1-terminating IAB-donor to identify the mobile IAB-MT’s Handover.

Proposal 3: F1-terminating IAB-donor needs to be indicated from source IAB-donor with a UE XnAP ID allocated by target IAB-donor over the Xn interface between the F1-terminating IAB-donor and the target IAB-donor.

Proposal 4: Agree the TP to TS 38.420 in Annex.

NG-based IAB-MT handover

Proposal 5: RAN3 assumes there is Xn connection between F1-terminating IAB-donor and RRC-terminating IAB-donor in R18.

Proposal 6: Only considering NG-based handover for mIAB-MT in case mIAB-MT handover from IAB-donor A to IAB-donor B while mIAB-DU still connect to another IAB-donor C.

Proposal 7: IAB TMM procedure and IAB resource coordination procedure don’t have to be extended to NGAP.

# Reference

- RP-222671. Revised WID for NR_mobile_IAB.

- Chairman notes of RAN3 118 – 121 meetings.

# Annex – TP to TS 38.420

-------------------------------------------Start of Change------------------------------------------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Boundary IAB-node: as defined in TS 38.401 [2].

corresponding node: as defined in TS 38.425 [7].

F1-terminating IAB-donor-CU: as defined in TS 38.401 [2].

IAB-DU: as defined in TS 38.300 [8].

IAB-MT: as defined in TS 38.300 [8].

Mobile IAB-node: as defined in TS 38.300 [8].

Non-F1-terminating IAB-donor-CU: as defined in TS 38.401 [2].

NG-RAN node: as defined in TS 38.300 [8].

RRC-terminating IAB-donor-CU: as defined in TS 38.401 [2].

secondary node: as defined in TS 37.340 [9].

-------------------------------------------Next Change------------------------------------------

### 5.2.10	 IAB support function

#### 5.2.10.1	F1-C Traffic Transfer function

This function is used to deliver F1-C traffic between the M-NG-RAN node and the S-NG-RAN node serving a dual-connected IAB-node, where the F1-C traffic is either received from the IAB-node or sent to the IAB-node.

#### 5.2.10.2	IAB Transport Migration function

This function allows the exchange of information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node, for the purpose of managing the migration of the boundary and descendant IAB-node traffic between the topologies managed by the two IAB-donor-CUs. This function also allows the exchange of information between the F1-terminating IAB-donor-CU and the RRC-terminating IAB-donor-CU of a mobile IAB-node, for the purpose of managing the migration of the mobile IAB-node traffic between the topologies managed by the two IAB-donor-CUs.

#### 5.2.10.3	IAB Resource Coordination function

This function is used to exchange information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node in order to support resource multiplexing between the IAB-MT and the IAB-DU of the boundary IAB-node. This function is also used to exchange information between the F1-terminating IAB-donor-CU and the RRC-terminating IAB-donor-CU of a mobile IAB-node in order to support resource multiplexing between the IAB-MT and the IAB-DU of the mobile IAB-node.

-------------------------------------------Next Change------------------------------------------

### 6.2.10	IAB procedures

The IAB procedures are used to enable the transfer of F1/non-F1 traffic for IAB, to exchange information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node, to enable the delivery of F1-C traffic between the M-NG-RAN node and the S-NG-RAN node serving a dual-connected non-boundary IAB-node, to exchange resource multiplexing related information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node, to exchange information between the F1-terminating IAB-donor-CU and the RRC-terminating IAB-donor-CU of a mobile IAB-node, and to exchange resource multiplexing related information between the F1-terminating IAB-donor-CU and the RRC-terminating IAB-donor-CU of a mobile IAB-node:

-	F1-C Traffic Transfer

-	IAB Transport Migration Management

-	IAB Transport Migration Modification

-	IAB Resource Coordination

-------------------------------------------End of Change-------------------------------------------