3GPP TSG-RAN WG3 Meeting #121bis	R3-235256

Xiamen, China, 9th – 13th October 2023

Agenda item:	  25.2.2

Source:		Lenovo

Title:		Discussion on PDU Set handling in non-homogenous cases, PDU Set based discard and data forwarding

Document for:		Discussion and Decision

- Introduction

The contribution discusses the PDU Set handling in non-homogenous cases, PDU Set based discard and data forwarding according to RAN2 and SA2 agreements.

- Discussion

2.1 PDU Set handling in non-homogenous cases

In last meeting for non-homogenous use cases, three cases are discussed:

-	Case1: NG-RAN node supports PDU set QoS parameters, and the corresponding QoS flow is setup successfully

-	Case2: NG-RAN node supports PDU set QoS parameters, but the corresponding QoS flow is setup as legacy QoS flow

-	Case3: NG-RAN node does not support PDU set QoS parameters, the corresponding QoS flow is setup as legacy QoS flow

According to SA2’s latest agreement, case 3 is the main case of which issues needs to be solved [1].

RAN3 should focus on case 3: NG-RAN node does not support PDU set QoS parameters, the corresponding QoS flow is setup as legacy QoS flow.

For case 3, three scenarios are discussed by SA2 [2]:

-	PDU session setup/modify

-	Xn handover

-	NG handover

For PDU session setup/modify, if the gNB supports PDU set based handling, the gNB responds to SMF ‘PDU Set Based Handling Support Indication’ in the in the PDU Session Resource Setup Response Transfer IE, PDU Session Resource Modify Response Transfer IE. If the ‘PDU Set Based Handling Support Indication’ is not included, the SMF shall treats the gNB does not support PDU set handling.

For Xn handover, if the target gNB supports PDU set concept, the gNB responds to SMF ‘PDU Set Based Handling Support Indication’ in the Path Switch Request Transfer IE. Followed by Path Switch Request procedure, the SMF shall provide PDU Set QoS parameters to the gNB in the PDU session setup/modify request procedure. If the ‘PDU Set Based Handling Support Indication’ is not included, the SMF shall treats the gNB does not support PDU set based handling.

For NG handover, similar as PDU session setup, if the gNB supports PDU set concept, the gNB responds to SMF ‘PDU Set Based Handling Support Indication’ in the in the Handover Request Acknowledge Transfer IE. If the ‘PDU Set Based Handling Support Indication’ is not included, the SMF shall treats the gNB does not support PDU set based handling.

NG-RAN node provides SMF with ‘PDU Set Based Handling Support Indication’ in the PDU Session Resource Setup Response Transfer IE, PDU Session Resource Modify Response Transfer IE, Path Switch Request Transfer IE and Handover Request Acknowledge Transfer IE.

2.2 PDU Set based discard

- PSI based discard timer configuration

In RAN2#121, it was agreed:

RAN2 thinks PSI can be useful for PDU set-based discard. RAN2 aims to introduce a mechanism to allow UE to handle discarding of packets with different PSI in case of congestion. FFS for other cases.

As per RAN2’s agreement, it means that gNB can configure per PSI based PDCP discard timer to the UE.  PDU Set Importance (PSI) identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

Currently, the PSI is only carried in the GTP-U header of the QoS flow, which means that only gNB-CU-UP can get PSI information after receiving the user data of the QoS flow. But the PSI based discard timer configuration is usually configured by the gNB-CU-CP. In order to support PSI based discard timer configuration, the gNB-CU-CP needs to be aware of PSI information.

One of solution is the gNB-CU-UP sends PSI information to the gNB-CU-CP. But the gNB-CU-UP can only get the PSI information only after receiving the user data of the QoS flow since the PSI information is carried in the GTP-U header of the user data as per SA2 agreement. It means that the gNB-CU-CP may not be able to configure the PSI based discard timer at DRB setup.

The other more reasonable solution is the SMF sends the PSI information by NG-C signalling as a part of PDU Set QoS parameters to the gNB-CU-CP.

In case of aggregated gNB, the gNB should also be aware of PSI information before data arrival e.g., for DRB establishment.

PSI information should be provided to gNB or gNB-CU-CP for setting PSI based discard timer. Send an LS to SA2 to ask providing PSI information by NG-C signalling from SMF.

For UL data transmission, the gNB-CU-CP configures PSI based discard timer to the UE by RRC message. For DL data transmission, the gNB-CU-CP configures PSI based discard timer to the gNB-CU-UP in the PDCP Configuration IE via E1AP Bearer Context Setup Request or Bearer Context Modification request messages.

The PSI based discard timer is included in the PDCP Configuration IE in E1 AP.

- DL PDU Set Discard in F1-U

The gNB-CU may decide to discard the whole PDUs of the PDU set for example, when the PDSB or PDCP discard timer expires and The PDU Set Integrated Handling Information is received. In this case, the gNB-CU needs to indicate the gNB-DU to discard the PDCP PDUs of the PDU set that have not been delivered to lower layer. Currently, the information of downlink NR PDCP PDUs to be discarded for user data associated with a specific data radio bearer has been supported in the DL USER DATA frame as specified in the TS 38.425. if the gNB-CU decides to discard the PDU set, the gNB-CU can provide related PDCP SNs to be discarded to the gNB-DU following the TS 38.425. But to support more efficient discard information, it is worth to discuss whether the gNB-CU can indicate a PDU Set SN to be discard or PSI of PDU sets to be discard. For example, when the discard indication is to indicate to discard user data for a certain PSI level, the gNB-DU discard all PDCP PDUs as indicated with the certain PSI level, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers; when the discard indication is to indicate to discard user data for a certain PDU Set, the gNB-DU discard all PDCP PDUs as indicated with PDU set SN, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers;

RAN3 discusses to introduce PDU set or PSI based discard indication in F1-U protocol.

When the gNB-DU discard some PDCP PDUs that would be cause PDCP SN gap in the receiving side which causes PDCP window stalling if the PDCP SN is not used anymore.  How to avoid PDCP SN gap when PDCP discard performing. In order to reduce PDCP SN gap in the receiving side caused by PDCP discard, the gNB-DU provides discard feedback to gNB-CU. The discard feedback includes the PDCP SN information for the discarded PDCP PDUs. The discarded PDCP SN information may be in the F1-U user plane protocol, e.g., in the DL Data Delivery Status frame. The discarded PDCP SN information includes the PDCP SNs for all discarded PDCP PDUs. When receives the discarded PDCP SN information, the gNB-CU assigns the PDCP SNs that has been discarded by gNB-DU to new PDCP SDU, so that PDCP SN gap issue will not happen in UE side.

gNB-DU provides discard feedback that includes PDCP SNs of discarded PDCP PDUs to gNB-CU in F1-U protocol.

2.3 Data Forwarding

- There are two types of data forwarding, DRB based data forwarding and PDU session based data forwarding. As in the legacy, we believe that both DRB based data forwarding and PDU session based data forwarding should be supported. For PDU session based data forwarding, the PDU Set Information is added in the in the GTP-U extension header, i.e., PDU Session container as specified in TS 38.415. However, for DRB based data forwarding, how to provide the PDU Set Information needs to be discussed, which GTP-U extension header is used, i.e., RAN container as specified in 38.425 v.s. PDU session container as specified in TS 38.415.

For DRB based data forwarding, the PDU Set Information should be added in the GTP-U extension header of PDCP SDU.

- PDU Set PSDB defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. the duration between the reception time of the first PDU (at the N6 termination point for DL or the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE for DL or N6 termination point for UL). The AN PSDB is derived at NG-RAN by subtracting CN PDB from the PSDB. Then AN-PSDB represents the difference between the time when the first PDU of the PDU Set is arrived at the gNB and the time when the last PDU or all PDUs of the PDU Set has been delivered to the UE successfully. However, handover may happen during transmission of a PDU set. There is a case that partial PDUs of the PDU set has been successfully transmitted in the source gNB, while the other PDUs of the PDU set will be transmitted by the target gNB. In this case, the target gNB should be aware of the remaining AN-PSDB for transmitting the remaining PDUs of the PDU set for more accurate QoS management and scheduling. One of possible solution is the source gNB provides the remaining AN-PSDB of a PDU set to the target gNB by Xn-U protocol.

- Figure 1. Handover happens during data transmission of a PDU Set

For data forwarding, the source gNB provides the remaining AN-PSDB of a PDU set in the GTP-U extension header to the target gNB.

- To support PDU Set based QoS handling, an indication of End PDU of the PDU set is provided by UPF in the GTP-U header. In order to support lossless handover, data forwarding from source gNB to target gNB is performed. In case of PDU set integration transmission is needed, the target gNB needs to know the transmission status of the PDU set in source gNB.  How the target gNB knows the transmission status of a PDU set needs to be solved at least in the following case:

-	The source gNB receives the End PDU of a PDU set from UPF.

-	Before handover, the End PDU (PDU#3) has been transmitted to UE successfully, but a PDU (PDU#2) before the End PDU have not been acknowledged from UE.

-	Currently, the source gNB shall forward PDU#2 to the target gNB during data forwarding.

-	Since the End PDU is not forwarded to the target gNB, the target gNB does not know whether the transmission of the PDU set is complete and thus is not able to perform PDU set integrated handling for the PDU set

- Figure 2. Issue of End PDU Indication during handover

For data forwarding, if the End PDU of a PDU set has been transmitted to UE successfully in source gNB, the source gNB needs to put End PDU indication in the GTP-U header of the last forwarded PDCP SDU of the PDU set.

3	Conclusion

The contribution discusses the PDU Set handling in non-homogenous cases, PDU Set based discard and data forwarding according to RAN2 and SA2 agreements.

- RAN3 should focus on case 3: NG-RAN node does not support PDU set QoS parameters, the corresponding QoS flow is setup as legacy QoS flow.

- NG-RAN node provides SMF with ‘PDU Set Based Handling Support Indication’ in the PDU Session Resource Setup Response Transfer IE, PDU Session Resource Modify Response Transfer IE, Path Switch Request Transfer IE and Handover Request Acknowledge Transfer IE.

- PSI information should be provided to gNB or gNB-CU-CP for setting PSI based discard timer. Send an LS to SA2 to ask providing PSI information by NG-C signalling from SMF.

- The PSI based discard timer is included in the PDCP Configuration IE in E1 AP.

- RAN3 discusses to introduce PDU set or PSI based discard indication in F1-U protocol.

- gNB-DU provides discard feedback that includes PDCP SNs of discarded PDCP PDUs to gNB-CU in F1-U protocol.

- For DRB based data forwarding, the PDU Set Information should be added in the GTP-U extension header of PDCP SDU.

- For data forwarding, the source gNB provides the remaining AN-PSDB of a PDU set in the GTP-U extension header to the target gNB.

- For data forwarding, if the End PDU of a PDU set has been transmitted to UE successfully in source gNB, the source gNB needs to put End PDU indication in the GTP-U header of the last forwarded PDCP SDU of the PDU set.

Reference

[1] S2-2309860 PDU Set QoS handling: Race conditions at mobility from supporting to non-supporting gNB, Qualcomm Incorporated, CATT, Ericsson, Nokia, Nokia Shanghai Bell, OPPO, ZTE, Samsung, vivo, China Mobile, Huawei, Lenovo, NTT DoCoMo, Google, Xiaomi

[2] S2-2309861 Non-homogeneous support of PDU set handling in NG-RAN. Lenovo, CATT, Samsung, vivo, xiaomi, Google, Nokia, Nokia Shanghai Bell, Ericsson