3GPP TSG-RAN WG3 #121bis								R3-235649

Xiamen, China, 9th – 13th Oct 2023

Agenda Item:	25.2.1

Source:	China Telecom

Title: 	Discussion on XR Enhancement in Split Architecture

Document for:	Discussion

Introduction

The updated WID on XR Enhancements for NR was agreed in RAN#99 meeting [1]. In RAN3#121 meeting, the following agreements on enhancement for E1 and F1 interface were achieved:

Add the PDU Set QoS Parameters in the QoS Flow Level QoS Parameters IE over NGAP, XnAP, F1AP and E1AP.

The PDU Set QoS parameters includes:

- PDU Set Delay Budget (PSDB).

- PDU Set Error Rate (PSER).

- PDU Set Integrated Handling Information (PSIHI).

- Enhance XnAP/F1AP/E1AP TSCAI to include the N6 Jitter information associated with DL Periodicity.

The intention of this paper is to discuss on E1 and F1 impact of XR enhancement, and provide some proposals.

Discussion

To configure UE power saving management scheme for connected mode DRX, SA2 agreed that UL/DL Periodicity, the N6 Jitter Information associated with the DL Periodicity also need to be provided to NG-RAN. In RAN3#121 meeting, N6 jitter information associated with DL periodicity was agreed to be included in E1 and F1 interface. For UL direction, RAN2 has agreed the following agreements in RAN2#123 meeting:

-	UE reports Burst Arrival time and Jitter associated with the UL data burst periodicity in uplink using UAI.

-	UE reports UL data burst periodicity in uplink using UAI.

With such information, the NG-RAN can configure properly radio resources for UE to mitigate the extra access delay caused by jitter.  Therefore, CU-CP need provide UL Burst Arrival time, UL Jitter, and UL data burst periodicity to DU.

Proposal 1: CU-CP need provide UL Burst Arrival time, UL Jitter, and UL data burst periodicity to DU

In XR, multiple flows are supported and the scheduling may be frequent in some duration. In some cases, XR traffic may have low latency requirement, and short DRX cycle is more suitable to reduce latency. Additionally, the short DRX cycle is optional in current spec and the operator can choose whether to configure short DRX cycle according to the traffic pattern. Therefore, it is better to support the short DRX cycle in rational numbers to better support the XR traffic.

In addition, how to resolve the mismatch between DRX cycle and XR traffic periodicity was widely discussed, and the following agreement was achieved in RAN2 #122 meeting.

Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications.

Therefore, a new DRX cycle based on rational numbers need to be contained in F1 interface. Details of new DRX cycle depend on RAN2 progress.

Proposal 2: a new DRX cycle based on rational numbers need to be contained in F1 interface.

To support signalling the congestion information from RAN to the CN in alignment with SA2 is another objective of XR Enhancement. According to TS23.501, the NG-RAN may be required to provide the UL and/or DL QoS Flow congestion information. In addition, NG-RAN also supports ECN marking for L4S to expose the congestion information. This function exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaptation [2]. For split architecture, the gNB-CU-CP requires the gNB-DU to perform congestion monitoring, the gNB-DU report the congestion indication to gNB-CU-UP, and gNB-CU-CP need to inform gNB-CU-UP to perform ECN marking for a specific QoS flow.

Proposal 3: the gNB-CU-CP requires the gNB-DU to perform congestion monitoring, the gNB-DU report the congestion indication to gNB-CU-UP

Proposal 4: gNB-CU-CP need to inform gNB-CU-UP to perform ECN marking for a specific QoS flow

We believe the Stage-2 TS38.401 also need to be updated to support XR, thus we provide a TP for TS38.401 in [3].

Proposal 5: the Stage-2 TS38.401 is need to be updated to support XR

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: CU-CP need provide UL Burst Arrival time, UL Jitter, and UL data burst periodicity to DU

Proposal 2: a new DRX cycle based on rational numbers need to be contained in F1 interface.

Proposal 3: the gNB-CU-CP requires the gNB-DU to perform congestion monitoring, the gNB-DU report the congestion indication to gNB-CU-UP

Proposal 4: gNB-CU-CP need to inform gNB-CU-UP to perform ECN marking for a specific QoS flow

Proposal 5: the Stage-2 TS38.401 is need to be updated to support XR

Reference

- RP-230786，Updated WID on XR Enhancements for NR，Nokia, Qualcomm，3GPP TSG RAN Meeting #99 Rotterdam, Netherlands, March 20-23, 2023

- 3GPP TS 23.501 V18.2.2，System architecture for the 5G System (5GS);Stage 2

- R3-235650 (TP for BLCR 38.401) Introduction of XR enhancements