3GPP TSG-RAN WG3 Meeting #121bis	R3-235686

Xiamen, China, October 09 – 13, 2023

Agenda Item:	25.2

Source:	Ericsson, Orange, Deutsche Telekom, AT&T, Verizon Wireless

Title:	(TP for XR CR for TS 38.425) ECN Marking for L4S

Document for:	Discussion, Decision

# Introduction

This paper contains a TP for the procedure used to exchange information supporting ECN Marking for L4S information.

# TP for TS 38.425

Below, a draft TP is presented where the proposals made in R3-235683 are reflected,

Start of Changes

#### 5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 |  |  | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=2) |  |  |  |  | PDCP Dupl. Ind. |  | Assistance Info. Ind. |  | UL Delay Ind. | DL Delay Ind. | 1 |
| Spare |  |  |  | ECN marking percentage ind. UL |  |  |  |  | ECN marking percentage ind. DL | PDCP Duplication Activation Suggestion | 1 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |
| UL Delay DU Result |  |  |  |  |  |  |  | 0 or 4 |
| DL Delay DU Result |  |  |  |  |  |  |  | 0 or 4 |
| ECN Marking Percentage UL | 0 or 1 |
| ECN Marking Percentage DL | 0 or 1 |

Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format

Next Change

#### 5.5.3.x1	ECN marking percentage Indication UL

Description: This field indicates the presence of the ECN Marking Percentage UL.

Value range: {0 = ECN Marking Percentage not present, 1= ECN Marking Percentage UL present}.

Field length: 1 bit.

#### 5.5.3.x2	ECN marking percentage Indication DL

Description: This field indicates the presence of the ECN Marking Percentage DL.

Value range: {0 = ECN Marking Percentage DL not present, 1= ECN Marking Percentage DL present}.

Field length: 1 bit.

#### 5.5.3.y	ECN marking percentage UL

Description: This field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked. As an example, value 9574 corresponds to a percentage of 95.74%

Value range: {0..10000}.

Field length: 1 octet.

#### 5.5.3.z	ECN marking percentage DL

Description: This field indicates the percentage of DL IP packets up to two decimal points that should be ECN marked. As an example, value 9574 corresponds to a percentage of 95.74%

Value range: {0..10000}.

Field length: 1 octet.

End of Changes