3GPP TSG-RAN3 Meeting #121bis	R3-235695

Xiamen, CN - October 9th – October 13th 2023

Agenda Item:	8.1

Source:	Ericsson

Title:	Response to R3-235337

Document for:	Discussion and Decision

# Introduction

This contribution provides response to R3-235337 in the form of an alternative version of a CR to 38.473

# Appendix with 38.423CR

3GPP TSG-RAN3 Meeting #121bis	R3-235695

Xiamen, CN - October 9th – October 13th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

x

| Title: | Corrections on multiple traces for Xn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2023-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedural text clarifying precedence rule on choosing the Trace Activation IE to be forwarded when multiple traces are available |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedural text clarifying that if multiple trace activation IEs are available only the most recent trace activation IE is to be forwarded within the relevant message. Impact assessment towards the previous version of the specification (same release): This CR has no impact on the protocol. This CR has limited functionality impact. The fucntioanlity impact can be considered isolated because the change affects only clarifying procedural text changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.2.4.2, 8.3.1.1, 8.3.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of the 1st Set of Changes >>>>>>>>>>>>>>>>>>>>

### 8.2.1	Handover Preparation

-- TEXT OMITTED –

#### 8.2.1.2	Successful Operation

-- TEXT OMITTED –

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23]. If multiple Trace Activation IEs are available only the most recent Trace Activation IE shall be forwarded within the HANDOVER REQUEST message.

<<<<<<<<<<<<<<<<<<<< End of the 1st Set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of the 2nd Set of Changes >>>>>>>>>>>>>>>>>>>>

### 8.2.4	Retrieve UE Context

-- TEXT OMITTED –

#### 8.2.4.2	Successful Operation

-- TEXT OMITTED –

If the Trace Activation IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23]. If multiple Trace Activation IEs are available only the most recent Trace Activation IE shall be forwarded within the RETRIEVE UE CONTEXT RESPONSE message.

<<<<<<<<<<<<<<<<<<<< End of the 2nd Set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of the 3rd Set of Changes >>>>>>>>>>>>>>>>>>>>

### 8.3.1	S-NG-RAN node Addition Preparation

-- TEXT OMITTED –

#### 8.3.1.1	General

-- TEXT OMITTED –

If Trace Activation IE has previously been received for this UE, it shall be included in the S-NODE ADDITION REQUEST message. If the Trace Activation IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [23]. If multiple Trace Activation IEs are available only the most recent Trace Activation IE shall be forwarded within the S-NODE ADDITION REQUEST message.

<<<<<<<<<<<<<<<<<<<< End of the 3rd Set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of the 4th Set of Changes >>>>>>>>>>>>>>>>>>>>

### 8.3.14	Trace Start

-- TEXT OMITTED –

#### 8.3.14.2	Successful Operation

-- TEXT OMITTED –

The Trace Start procedure is initiated by the M-NG-RAN sending the TRACE START message to the S-NG-RAN for that specific UE. Upon reception of the TRACE START message, the S-NG-RAN node shall initiate the requested trace session as described in TS 32.422 [23].  If multiple Trace Activation IEs are available only the most recent Trace Activation IE shall be forwarded within the TRACE START message.

<<<<<<<<<<<<<<<<<<<< End of the 4th Set of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>