3GPP TSG-RAN WG3 #120							R3-233357

22th – 26th May 2023

Incheon, Korea

Agenda Item:	14.2

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion: CB: # 3_MDTUserConsent

Document for:	Discussion

# Introduction

This is the offline summary for the following comeback:

CB: # 3_MDTUserConsent

- Check the motivation

- The issue on the management based MDT allowed IE?

(moderator - HW)

Summary of offline disc R3-234498

# For the Chairman’s Notes

# Discussion

## Issue 1: Check the motivation

In TS 32.422, section 4.6.2, the user consent handling for m-based MDT for E-UTRAN are cited here for your ference.

4.6	User consent handling in MDT for UTRAN and E-UTRAN

4.6.2	Management based MDT

In case of management based MDT getting user consent is required before activating the MDT functionality because of privacy and legal obligations. The same user consent information can be used for management based MDT and for signalling based MDT (i.e. there is no need to differentiate the user consent per MDT type).

Collecting the user consent shall be done via customer care process. The user consent information availability shall be considered as part of the subscription data and as such this shall be provisioned to the HSS database.

The following figure shows an example scenario summarizing the functionality.

Figure 4.6.2.1: Example for delivering user consent information in management  based MDT

When UE attaches to the network, the HSS shall forward the user consent information, stored in the HSS database, to the corresponding MME/SGSN/MSC-S. When the MME/SGSN/MSC-S receives the user consent information it shall store it in its subscriber database.

The MME/SGSN/MSC-S shall also check the roaming status of the user. If the user is within his home operator’s PLMNs and the user has given his consent, the MME/ SGSN/MSC-S shall send the Management Based MDT Allowed IE to the eNB/RNC during the UE context setup procedure. Otherwise the MME/ SGSN/MSC-S shall not send the Management Based MDT Allowed IE to the eNB/RNC.

If the result of the roaming status check indicates a home subscriber, MME/SGSN/MSC-S shall forward the already stored user consent information to the corresponding eNodeB/RNC as part of Management Based MDT Allowed IE.

When the management based MDT activation is sent to eNodeB/RNC, eNodeB/RNC shall check the availability of the Management Based MDT Allowed IE before making the UE selection. In case the Management Based MDT Allowed IE is not available, the eNodeB/RNC shall not select the UE. In case the Management Based MDT Allowed IE is available, the eNodeB/RNC shall verify if the UE’s RPLMN matches the PLMN where TCE resides – Trace Reference PLMN (PLMN portion of the Trace Reference). In case of a mismatch, the eNodeB/RNC shall not select the UE.The eNB/RNC shall forward the received Management Based MDT Allowed IE during X2/Iur based handovers to the target node. The Management Based MDT Allowed IE is stored in the eNB/RNC as part of the UE context. If the user consent information is updated while a UE context is already set up in the eNB, the changed user consent should be taken into account in the next call/session setup.

The yellow sentence proves that the motivation of user consent update for MDT in E-UTRAN is confirmed valid.

Therefore, the moderator would like to propose :

Proposal 1: The motivation of user consent update for Management based MDT in E-UTRAN is confirmed valid.

## Issue 2: Issue on Management based MDT allowed IE

An issue was raised during online discussion,, which is that the Management based MDT allowed IE is also the user consent for management based MDT.

Do we need to consider to revoke this kind of user consent as well? We have two options.

Option 1: Yes. Then a new IE named as like management based MDT not allowed IE is needed in addition.

Option 2: No. The reason is that such IE is introduced in the first release of MDT which only support MDT measurement collection in the current serving PLMN without support of collecting in equivalent PLMNs. Therefore, a new IE named as Management based MDT PLMN list was introduced later on to replace the management based MDT allowed IE. That’s why we don’t use the management based MDT allowed IE in NG-RAN anymore.

The moderator thinks that the MDT function in LTE should keep alignment to NG-RAN, therefore, slightly prefer option 2.

But open to hear other views.

Please provide your views here.

| Company | Option 1 or option 2? | Comments |
| --- | --- | --- |

## Issue 3: Checking the CRs if issue 2 is resolved

TBD.