3GPP TSG-RAN WG3 #121							R3-234548

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	12.2.2.1

Source:	Ericsson (moderator)

Title:	Summary of Offline Discussion: CB: # AIRAN2_LB

Document for:	Discussion

# Introduction

This documents captures the results of offline discussions for the following comeback:

CB: # AIRAN2_LB

- Continue the discussion on the above open issues

- Check the details of stage3 CRs

- Capture agreements and open issues

(moderator – E///)

Summary of offline disc R3-234548

# For the Chairman’s Notes

# Discussion

## Partial Success Indicator Discussion

During the online discussion the following ways forward were identified with respect to the addition of the Partial Success Indicator in the AI/ML Information Request message:

- Opt1: Add a flag in the request message

- Opt2: Introduce new cause values in the failure measurement objectives in the response message, e.g., “partial measurement not available”

- Op3: Add priority indication for one or more requested measurement objectives in the request message

Try to draw conclusion on which option can be accepted in R18.

With respect to the partial success indicator, the moderator understanding is that the options applicable are Option 1 and Option 3. Option 2 relates to the introduction of cause values, which is not strictly related to the partial success indicator and that will be handled later.

### Partial Success Indicator Options

The moderator´s view is that Partial Success Indication is an optimization that can be achieved with different levels of enhancement.

Option 1 provides a solution in line with LTE that caters for the case of AI/ML model implementations where a model must receive specific inputs in order to be able to infer.

Option 3 provides a solution with higher flexibility, catering for implementations where an AI/ML model is designed with different importance weights for each input.

The moderator proposes the following:

- Aim for an agreement on Option 1, as it is the simplest option to achieve in Rel18

- If no agreement can be reached for Option 1, converge on the agreement that Partial Success Indicator is not pursued in Rel18

Summary of Discussion:

### Addition of new cause values

A discussion on the addition of new cause values was taken online. Several cause values have been proposed, for example:

R3-233912 proposes the following:

Proposal 03: The requested NG-RAN node includes the following failure cause values and the desired values in the AI/ML Information Response message:

•	measurement not supported with requested reporting periodicity and requested NG-RAN node’s supported value,

•	measurement temporarily not available with requested reporting periodicity and requested NG-RAN node’s supported value,

•	measurement not supported with current combination of requested information and requested NG-RAN node’s supported combination,

•	measurement temporarily not available with current combination of requested information and requested NG-RAN node’s available

R3-234329 proposes the following:

Proposal 1:  The reporting node can use the AI/ML INFORMATION RESPONSE messages to explicitly indicate the measurements that can be provided and the list of failed measurements together with the corresponding cause indicating the reason for failure per measurement. The following failure causes are supported:

- measurement not supported, reusing “Measurement not Supported For The Object”

- measurement temporarily not available, reusing “Measurement Temporarily not Available”

- measurement not supported with requested reporting periodicity,

- measurement temporarily not available with requested reporting periodicity,

- measurement not supported with current combination of requested information,

- measurement temporarily not available with current combination of requested information,

Proposal 2:  An alternative to reduce the number of new cause values to introduce is to support the following list of cause values:

- measurement not supported, reusing “Measurement not Supported For The Object”

- measurement temporarily not available, reusing “Measurement Temporarily not Available”

- measurement not supported with current combination of requested information and periodicity,

- measurement temporarily not available with current combination of requested information and periodicity,

The moderator believes that at a minimum level two new cause values should be introduced:

- A cause value indicating failures due to timing issues (e.g. not supported periodicity, not supported prediction time)

- A cause value indicating failures due to combination of requested information (e.g. a certain combination of prediction is not available)

The moderator would like to ask the group whether there is consensus in introducing new cause values, in particular to cover the failure cases above

Summary of Discussion:

### Failure indication in AI/ML Information Update

As part of the discussion on how to signal failure to report requested information, proposals were presented to indicate that, after a measurement has been configured via the AI/ML Information Reporting Configuration procedure, the measurement is no more available for reporting.

R3-234329 proposes the following:

Proposal 3:  The reporting node can use the AI/ML INFORMATION UPDATE messages to indicate potential issues preventing the reporting.

R3-233912 proposes the following:

Proposal 05: If a UE performance feedback measurement becomes no longer available/supported after a certain time (e.g., when UE moves out of this node), the requested NG-RAN node includes the measurement results since the latest reporting and the corresponding failure cause value in AI/ML information update message.

R3-234188 proposes the following:

Proposal 02: If target gNB can no longer provide UE performance feedback for a particular UE (e.g., due to HO, RRC state, or RLF), the target gNB indicates to the source gNB via a cause value.

All the proposals above point at potential additions into the AI/ML Information Update message to indicate that a measurement that has been requested and admitted can no longer be provided by the requesting node.

The moderator would like to ask the group whether there is consensus in enhancing the AI/ML Information Update message with information indicating that a measurement that has been requested and admitted can no longer be provided by the requesting node

Summary of Discussion:

## UE Performance Feedback

The following was captured during the offline discussion:

Do not support the finer granularity of UE performance feedback in R18.

UE performance feedback can be reported both periodically and one-time, how to define the reporting window?

As a reminder, the moderator would like to recall the agreements from RAN3-120 and provide his interpretation of it:

The problem of how to signal to the reporting node time configurations for the UE Performance Feedback measurement reporting is acknowledged. C

Namely, the above agreement implies that RAN3 shall address the issue of lack of time configuration for the UE Performance Feedback.

Time configuration for UE Performance Feedback measurement reporting consists at least of:

- Time duration of the UE Performance Feedback measurement collection (the time duration starting at handover execution and including the last UE Performance Feedback report)

- Whether, within such time duration, measurements are reported periodically or one time

- In case of periodic reporting, the period of UE Performance Feedback measurements (whether existing of new IE)

Namely, the above agreement implies that:

- An indication of a time window starting at handover execution and during which UE Performance Feedback is collected needs to be introduced in the AI/ML Information Request message

- RAN3 needs to agree how to indicate that UE Performance Feedback is signalled periodically or one-time

- RAN3 needs to agree whether to introduce a dedicated reporting period for the UE Performance Feedback or if to reuse the existing Reporting Periodicity for the UE Performance Feedback reporting

For the sake of achieving the above requirements in the simplest way allowing timely completion of work, the moderator proposes the following:

Proposal 1:

A new IE is introduced in the AI/ML Information Request, indicating the Repodring Duration for the UE Performance Feedback, starting at Handover Execution. As an example, the IE may be represented as follows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Reporting duration | O |  | ENUMERATED (0, 1s, 2s, 5s, 10s, 20s, 60s, ...) | Time duration for which measurements should be reported upon fulfilment of the event in seconds. If the value is zero, reporting occurs only once. |  |  |

Proposal 2:

It is agreed that the Reporting Periodicity applies also to the UE Performance Feedback.

Namely, for the duration of the reporting period, UE Performance Feedback is reported at the periodicity indicated by the Reporting Periodicity IE.

If the Reporting Periodicity IE is not present, UE Performance Feedback is reported only once.

The moderator would like to ask the group whether there is consensus on the above proposals

Summary of Discussion: