3GPP TSG-RAN WG3 Meeting #121	R3-234613

- Toulouse, France, 21st – 25th August 2023

Agenda Item:	25.2

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # R18XR

Document for:	Approval

# Introduction

CB: # R18XR

- Capture the agreements to TP

- Discuss the open issues as above

- Capture agreements and open issues

(moderator - Nok)

Summary of offline disc R3-234613

It is proposed to complete the discussion by 21:59 Aug 24 (Thursday)

# For the Chairman’s Notes

Propose the following:

Endorse Following TPs:

- R3-234652	(TP for TS38.413) Support for NR XR

- R3-234656	(TP for TS38.423) Support for NR XR

- R3-234653	(TP for TS38.473) Support for NR XR

- R3-234654	[TP for BLCR TS37.483] Introducing enhancement for NR XR

- R3-234651	(TP for Stage-2 TS 38.300) for XR Enhancements

- R3-234655	(TP for TS38.415) Support for NR XR

- R3-234657	(TP for TS38.425) Support for NR XR

Continue discussion on following issues:

- Issue 2: whether use RACS-like method to detect the peer node support PDU Set QoS handling.

- Issue 3: how to support the mobility from non-PDU Set capable source NG-RAN to a PDU Set capable target NG-RAN, and how to transfer the PDU Set QoS to target NG-RAN.

- Issue 4: how to support ECN marking in the NG-RAN

For example, whether adding an ECN marking Request during QoS flow establishment, whether providing a feedback when a QoS flow is setup with ECN marking accepted, how DU providing congestion information to CU, etc.

- Issue 5: How to support ECN marking in UPF and network information exposure

For example, whether adding a congestion monitoring request during QoS flow establishment, whether providing a feedback when a QoS flow is setup with congestion monitoring accepted, how DU provide congestion information CU, CU provide congestion information to UPF, etc.

- Issue 6: How to support PDU Set discard

- Issue 7: whether to include the QFI in the F1-U and Xn-U?

- Issue 8: TSCAI support in non-homogenous NG-RAN deployment (non-homogenous with regard to TSCAI support). After mobility, TSCAI shall work in the supporting gNB.

- Issue 9: how to handle End PDU Indication during data forwarding if the End PDU has been transmitted to UE in the source side but there are still other PDUs to be transmitted of the PDU Set.

- Issue 10: how the target gNB is aware of the remaining AN-PSDB of a PDU set if partial PDUs of the PDU set has been successfully transmitted in the source gNB, while the other PDUs of the PDU set will be transmitted by the target gNB.

# Discussion

## PDU Set QoS parameters in NGAP/XnAP/F1AP/E1AP

RAN3 agreed to enhance NGAP/XnAP/F1AP/E1AP to support following agreements

Add the PDU Set QoS Parameters in the QoS Flow Level QoS Parameters IE over NGAP, XnAP, F1AP and E1AP.

The PDU Set QoS parameters includes:

- PDU Set Delay Budget (PSDB).

- PDU Set Error Rate (PSER).

- PDU Set Integrated Handling Information (PSIHI).

Moderator suggest the new PDU Set QoS parameters will be added as a new IE in the QoS Flow Level QoS Parameters IE. The IE name can be FFS.

Q1: Please share your view on: Add a new XR IE (IE name FFS) in NGAP/XnAP/F1AP/E1AP QoS Flow Level QoS Parameters IE. The IE contains PSDB, PSER and PSIHI.

| Company | Comment |
| --- | --- |
| Nokia | Agree |
| China Telecom | Agree |
| Qualcomm | Yes |
| Huawei | Agree |
| Xiaomi | Agree |
| LGE | Agree |
| Ericsson | Agree – name can be “PDU Set QoS parameters”? as indicated by SA2. |
| Samsung | Agree |
| ZTE | Agree with Ericsson |
| Lenovo | Agree |

Summary

Potential proposals:

## TSCAI in NGAP/XnAP/F1AP/E1AP

RAN3 agreed

Enhance NGAP TSCAI to include the N6 Jitter information associated with DL Periodicity, and clarify TSCAI can also be used for non-GBR.

Enhance XnAP/F1AP/E1AP TSCAI to include the N6 Jitter information associated with DL Periodicity.

Same TSC Traffic Characteristics IE is defined in NGAP/XnAP/F1AP/E1AP. Only NGAP has the limitation that it is only for GBR.

| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

XnAP:

| >>>>TSC Traffic Characteristics | O |  | 9.2.3.114 |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

F1AP:

| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

E1AP:

| >>TSC Traffic Characteristics | O |  | 9.3.1.75 | Traffic pattern information associated with the QFI. Details in TS 23.501 [20]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

To support the new requirement that TSCAI can also be used for non-GBR, Moderator suggest to delete the limitation in NGAP Semantics Description, and use same semantics description to align with F1AP/E1AP. Other option is to delete the semantics description as XnAP. Moderator think it may be better to align with the majority 😊 F1AP/E1AP.

Q2: Please share your view on following

- 1: In NGAP, change semantics description as F1AP/E1AP, i.e.

| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | Traffic pattern information associated with the QFI. Details in TS 23.501 [9]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

- 2: In NGAP/XnAP/F1AP/E1AP, Add N6 Jitter information associated with DL Periodicity, e.g. use NGAP as an example:

#### 9.3.1.131	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.132 |  | - |  |
| Burst Arrival Time | O |  | 9.3.1.133 |  | - |  |
| Survival Time | O |  | 9.3.1.221 |  | YES | ignore |
| N6 Jitter | O |  | FFS | Only applicable to downlink | YES | ignore |

| Company | Comment |
| --- | --- |
| Nokia | Agree with both |
| China Telecom | Agree with both |
| Qualcomm | Agree |
| Huawei | Agree with both |
| Xiaomi | Agree with both |
| LGE | Agree with both |
| Ericsson | Agree with all |
| Samsung | Agree to align with majority |
| ZTE | Agree |
| Lenovo | Agree |

Summary

Potential proposals:

## Container for PDU Set information

RAN3 agreed

PDU Set Information as below needs to be transferred over NG-U, Xn-U and F1-U:

  - PDU Set Sequence Number.

  - Indication of End PDU of the PDU Set.

  - PDU Sequence Number within a PDU Set.

  - PDU Set Size in bytes.

  - PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

End of Data Burst information needs to be transferred over NG-U, Xn-U and F1-U.

Whether to create a new “PDU Set Container” or reusing “PDU Session Container"?

There are two options to be discussed:

- Option 1: extend existing container

- Option 2: introduce a new container

Q3: Please share your view on above two options, and anything can be captured in 38.415/425

| Company | Comment |
| --- | --- |
| Nokia | We prefer to have a further check with developers, and continue the discussion in next meeting. |
| China Telecom | Agree with Nokia, we need more time to further check and this discussion could be postponed to next meeting |
| Qualcomm | Need of new container can be discussed later. |
| Huawei | Agree with company above, to discuss next meeting after further check. |
| Xiaomi | Agree to discuss next meeting after the further check in network vendors. |
| LGE | Similar view to Nokia. We need to check the above options and discuss in the next meeting. |
| Ericsson | Fine to postpone to next meeting |
| Samsung | Fine to discuss in the next meeting. |
| ZTE | Fine to discuss in the next meeting. |
| Lenovo | Discuss it in next meeting. There are further progress in SA2 which we need take into account. |

Summary

Potential proposals:

## Open issues to be discussed in next meeting

Due to the limited time for the online/offline discussion, Moderator suggest to first agree the list of open issues and potential options, so companies can prepare contribution for next meeting:

- Issue 1: whether NG-RAN provide an indication to SMF on whether disa.

- Issue 2: whether use RACS-like method to detect the peer node support PDU Set QoS handling.

- Issue 3: how to support the mobility from non-PDU Set capable source NG-RAN to a PDU Set capable target NG-RAN, and how to transfer the PDU Set QoS to target NG-RAN.

- Issue 4: how to support ECN marking in the NG-RAN

For example, whether adding an ECN marking Request during QoS flow establishment, whether providing a feedback when a QoS flow is setup with ECN marking accepted, how DU providing congestion information to CU, etc.

- Issue 5: How to support ECN marking in UPF and network information exposure

For example, whether adding a congestion monitoring request during QoS flow establishment, whether providing a feedback when a QoS flow is setup with congestion monitoring accepted, how DU provide congestion information CU, CU provide congestion information to UPF, etc.

- Issue 6: How to support PDU Set discard

- Issue 7: whether to include the QFI in the F1-U and Xn-U?

- TSCAI support in non-homogenous NG-RAN deployment (non-homogenous with regard to TSCAI support). After mobility, TSCAI shall work in the supporting gNB.

Q4: Please share your view on above issue, and add any missing issues

| Company | Comment |
| --- | --- |
| China Telecom | We think the enhancement on TS38.401 is needed as there are impacts on F1/E1 interface. |
| Huawei | Suggest rewording to Issue 2 as Issue 2: How to support the indication of supporting PDU set handling from NG-RAN to SMF? And add Issue 7: whether to include the QFI in the F1-U and Xn-U? |
| Xiaomi | For issue 2, I understand this relates to SA2’s LS non-homogeneous deployment, does this mean we will not reply to SA2 this meeting? Regarding to non-homogeneous deployment, one more thing may need to be discussed, which is related to the EN in SA2 attached CR, that is whether and how the target gNB handles different types of PDU from source gNB and UPF, i.e. PDU with PDU set information and PDU without PDU set information. In our view, gNB can handle them by gNB implementation, we’d like to confirm this with all the companies in RAN3 and we think it would be helpful for SA2’s discussion if RAN3 can reply SA2 on this issue as early as possible. |
| Ericsson | We should align issue 1 with SA2’s question in the LS, which is simply about the support/non-support of PDU Set handling in non-homogeneous RAN deployment. We disagree on discussing something that is not decided by SA2 in stage 2. So we can only agree on the issue as long as it is within the frame of SA2 question in the LS. If so Issue 1 and 2 can be merged in a simple way: - Issue 1: How can NG-RAN provide an indication to SMF on whether it supports PDU Set based handling: Add an explicit indicator (MBS indicator method) or use RACS-like method to detect the peer node supported capabilities including PDU Set QoS handling. On issue 4-7, we suggest, please, to consider discussing these issues in a separate sub-AI dedicated to L4S 😊. PDU Set handling and signaling should be taken as a separate topic form L4S. We agree with Xiaomi that how gNB handles different type of PDUs is up to gNB implementation. |
| Vodafone | Add issue 8: TSCAI support in non-homogenous NG-RAN deployment (non-homogenous with regard to TSCAI support). After mobility, TSCAI shall work in the supporting gNB. |
| Samsung | Ok to these open issues. Fine to modify issue 1 and issue 2, as suggested by HW and Ericsson, either way is ok for us. It is not sure issue 4-7 belong to L4S only. At least for issue 6 and issue 7, it is related to the PDU set handling. Open to discuss. |
| ZTE | Fine with all the listed issues |
| Lenovo | We think there are more open issues should be addressed (as listed in our paper R3-234162) - (Proposal 13 – data forwarding related) how to handle End PDU Indication during data forwarding if the End PDU has been transmitted to UE in the source side but there are still other PDUs to be transmitted of the PDU Set. - (Proposal 12 – handover related) how the target gNB is aware of the remaining AN-PSDB of a PDU set if partial PDUs of the PDU set has been successfully transmitted in the source gNB, while the other PDUs of the PDU set will be transmitted by the target gNB. |

Summary

Potential proposals:

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-233731, LS reply on TSCAI for XR (SA2(VIVO))

- R3-233732, Reply LS on the N6 PDU Set Identification (SA2(OPPO))

- R3-233733, Reply LS on RAN information exposure for XRM (SA2(China mobile))

- R3-233734, LS Reply on Design of RTP Header Extension for PDU Set handling (SA2(Huawei))

- R3-233735, Non-homogeneous deployment of PDU Set based handling (SA2(Qualcomm))

- R3-233950, Discussion on the support of enhancement for NR XR (Samsung)

- R3-233951, [TP for BLCR TS37.483] Introducing enhancement for NR XR (Samsung)

- R3-233952, [TP for BLCR TS38.413, TS38.423] Introducing enhancement for NR XR mobility (Samsung)

- R3-233981, TP for Stage-2 TS 38.300 for XR Enhancements (Qualcomm Incorporated)

- R3-234019, Discussion on support for NR XR  (Nokia, Nokia Shanghai Bell)

- R3-234020, (TP for TS38.413) Support for NR XR	 (Nokia, Nokia Shanghai Bell)

- R3-234021, Discussion on multiple LSs  (Nokia, Nokia Shanghai Bell)

- R3-234119, Discussion on Support Enhancements on NR XR (CATT)

- R3-234120, Discussion on XR LSin from SA2 (CATT)

- R3-234121, (TP for XR for 38.413, 37.483 and 38.473) Support Enhancements on XR awareness (CATT)

- R3-234153, Discussion on discard operation for XR (Huawei, Qualcomm Inc.)

- R3-234154, (TP for NR_XR_enh BL CRs) Discussion on the enhancement for XR awareness (Huawei, Qualcomm Inc.)

- R3-234155, (TP for NR_XR_enh BL CR of TS 38.413) Discussion on incoming LS  from other WGs (Huawei, Qualcomm Inc.)

- R3-234162, Discussion on PDU Set QoS Parameters and PDU Set Information (Lenovo)

- R3-234163, Discussion on PDU Set based Discard (Lenovo)

- R3-234164, Discussion on ECN marking for L4S (Lenovo)

- R3-234257, Discussion on enhancement for XR (Xiaomi)

- R3-234258, (TPs for TS 38.300 and TS 38.413) Non-homogenous support of PDU set based handling (Xiaomi)

- R3-234275, Discussion on XRM support (Ericsson)

- R3-234276, Introduction of XRM enhancement (Ericsson)

- R3-234302, Support for L4S in NG-RAN (Ericsson, Deutsche Telekom)

- R3-234406, Disucssion on Support of XR Enhancement in Split Architecture (China Telecom)

- R3-234408, (BLCR to 38.401) Introduction of XR enhancements (China Telecom)

- R3-234452, Discussion on non-homogeneous deployment of PDU Set based handling (CMCC)

- R3-234482, (TP to TS 38.413) Discussion on support of XR enhancements (ZTE)

- R3-234483, (XR BL CR to TS 38.415) Introduction of XR enhancements (ZTE)

- R3-234484, (TP to TS 38.423) Support of XR enhancements (ZTE)