3GPP TSG-RAN WG3#122 	R3-237163

Chicago, USA, 13-17 November 2023

Agenda item:	14.2 Signalling Support for L1/L2 based Inter-Cell Mobility

Source:	Nokia, Nokia Shanghai Bell

Title:	TP (BL CR TS 38.401) LTM RS Configuration and Signalling

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we discuss the details on RS configuration and signalling procedure in the light of previous RAN1, RAN2, and RAN3 agreements. It is to be noted that the decisions of RAN2 in the previous meeting (October 2023) affect earlier RAN3 agreements in the endorsed BL CRs at RAN3#122-bis for stage-2 in TS 38.401, and stage-3 in TS 38.473. The proposals for correction are also provided.

# 2	Related Agreements

In the previous RAN3 meeting (RAN3#121), the following was agreed:

Agreements: Option 2, UE associated signaling (UE Context Setup/Modification) is adopted to retrieve the RS configuration from the gNB-DU. No need for the gNB-CU to explicitly request the candidate(source) gNB-DU to provide the RS configuration during context setup/modification. FFS on whether the RS configuration can be retrieved before step2.

In the RAN2#122, the following was agreed:

Agreements: For L1 measurements for LTM - The RS configuration is provided to the UE per LTM candidate cell. - RAN2 assumes that Each candidate DU needs to know the RS configuration of each candidate DUs in order to provide the LTM candidate configuration. - RAN2 assumes that the CU transmits to each C-DU the RS configuration of S-DU (if this is an LTM candidate cell) and/or other C-DUs, to generate the corresponding L1 configuration for LTM. - RAN2 assumes C-DU generates the RS configuration and send to the CU. The CU transmits to the Source DU the RS configuration per LTM candidate cell and the associated LTM candidate (when the CU receives LTM candidate configuration(s) from the C-DU). It is up to RAN3 whether the RS configuration is sent before (or at the same time of) the C-DU creates the LTM candidate configuration (and whether is semi-statis or UE associated). - The RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration. - CSI reports for LTM candidates (neighbour cell reports for the purpose of LTM cell switch) are configured by the serving cell in an IE that is like CSI-ReportConfig for LTM within the ServingCellConfig since this is the cell in which the report is to be transmitted. - RAN2 assumes the following about CSI measurement reporting for LTM (final decision up to RAN1): - UE reports all measured LTM candidate cells in a single report; or - UE reports one or a subset of measured LTM candidate cell(s) in a report.

In the RAN2#123bis, the following was agreed which was also included in the LS sent to RAN3 (R2-2311332):

Agreements: - For the model of CSI report configuration, RAN2 to implement Option 2 (as in current RRC running CR). - For the model of RS configuration, RAN2 to follow what indicated by RAN1 in the parameter list. - The LTM CSI resource configuration is generated by the CU. Send an LS to RAN3 (include in LS below) - The list of LTM CSI resource configuration is common for all the LTM candidate cells (as in current RRC running CR).

CSI resource configuration is captured in the current RRC running CR inside LTM-Config IE as follows:

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        OCTET STRING (CONTAINING RRCReconfiguration)                       OPTIONAL,   -- Need M

ltm-CandidateToReleaseList-r18        LTM-CandidateToReleaseList-r18                                     OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         LTM-CandidateToAddModList-r18                                      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1.. maxNrofCellsLTM-r18-plus-1)                           OPTIONAL,   -- Cond FirstLTM-Only

ltm-CSI-ResourceConfigToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLtmCSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL, -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLtmCSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL, -- Need N

attemptLTM-Switch-r18                          ENUMERATED {true}                                         OPTIONAL,    -- Need LTM

ltm-ServingCellUeMeasuredTA-ID-r18             INTEGER (1.. maxNrofCellsLTM-r18-plus-1)                  OPTIONAL,    -- Cond FirstLTM-Only

...

}

LTM-CandidateToReleaseList-r18 ::= SEQUENCE (SIZE (1..maxNrofCellsLTM-r18)) OF LTM-CandidateId-r18

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

CSI report configuration is captured in the current RRC running CR inside CSI-MeasConfig which is part of serving cell configuration.

-- ASN1START

-- TAG-CSI-MEASCONFIG-START

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource           OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId         OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet     OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId   OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet   OPTIONAL, -- Need N

csi-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig

OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId

OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig  OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId

OPTIONAL, -- Need N

reportTriggerSize                   INTEGER (0..6)                                                            OPTIONAL, -- Need M

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }                            OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList    SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }         OPTIONAL, -- Need M

...,

[[

reportTriggerSizeDCI-0-2-r16        INTEGER (0..6)                                                            OPTIONAL -- Need R

]],

[[

sCellActivationRS-ConfigToAddModList-r17  SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17   OPTIONAL, -- Need N

sCellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL  -- Need N

]],

[[

ltm-CSI-ReportConfigToAddModList-r18        SEQUENCE (SIZE (1..maxNrofLtmCSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18

OPTIONAL, -- Need N

ltm-CSI-ReportConfigToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofLtmCSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfigId-r18

OPTIONAL -- Need N

]]

}

-- TAG-CSI-MEASCONFIG-STOP

-- ASN1STOP

In the RAN1# 112 and RAN1#113, the following was agreed which was also included in RRC parameter list with parameter name ltm-SSB-Config-r18.

Agreement (RAN1 112) - For L1-RSRP measurement RS configuration - For SSB based L1-RSRP measurement: - As a starting point, at least the following information needs to be provided to a UE, e.g. - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst) - For inter-frequency: frequency domain location (e.g. center frequency), SCS - FFS: transmission power (for pathloss calculation) - Note: other parameters included in the configuration can be further discussed Agreement (RAN1# 113) For the configuration of SSB based L1-RSRP measurement, periodicity of SSB, SSB position in burst are provided as time domain information for intra- and inter- frequency.

LTM SSB configuration is captured in RAN2 running CR inside LTM-Candidate-r18 IE.

-- ASN1START

-- TAG-LTM-CANDIDATETOADDMODLIST-START

LTM-CandidateToAddModList-r18 ::= SEQUENCE (SIZE (1..maxNrofCellsLTM-r18)) OF LTM-Candidate-r18

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId,

ltm-SSB-Config-r18,                            LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration),         OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     SetupRelease { EarlyUL-SyncConfig-r18 }               OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofCellsLTM-r18-1)                    OPTIONAL,    -- Need M

ltm-dl-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-States-r18)) OF CandidateTCI-State-r18

OPTIONAL,    -- Need N

ltm-dl-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-States-r18)) OF CandidateTCI-StateId-r18

OPTIONAL,    -- Need N

ltm-ul-TCI-ToAddModList-r18                    SEQUENCE (SIZE (1..maxNrofCanaidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18

OPTIONAL,    -- Need N

ltm-ul-TCI-ToReleaseList-r18                   SEQUENCE (SIZE (1.. maxNrofCanaidateUL-TCI-r18)) OF CandidateTCI-UL-StateId-r18

OPTIONAL,    -- Need N

ltm-UeMeasuredTA-ID-r18                        INTEGER (1..maxNrofCellsLTM-r18-1)                    OPTIONAL,    -- Need M

...

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssbFrequency-r18                               ARFCN-ValueNR,

subCarrierSpacing-r18                          SubCarrierSpacing,

ssb-Periodicity-r18                             ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1}      OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                        CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                                OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                                     OPTIONAL,   -- Need R

...

}

-- TAG-LTM-CANDIDATETOADDMODLIST-STOP

-- ASN1STOP

Based on the RRC running CR, the understanding of LTM-Config IE structure is as follows:

Figure 1: LTM-Config IE structure assuming 3 candidate cells.

# 3	Discussion

Based on Section 2, our understanding of the complete procedure for RS configuration per candidate cell (i.e. SSB configuration for Rel. 18 LTM), common CSI resource configuration and serving cell CSI report configuration is as follows:

Step 1: The RS configuration per candidate cell (i.e. SSB configuration for Rel. 18 LTM) is retrieved from candidate gNB-DU(s) via a UE-associated procedure (i.e. UE Context Setup/Modification procedure).

Step 2: Based on all the received RS configuration for all candidate cells, gNB-CU generates a common CSI resource configuration for LTM.

Step 3: The generated common CSI resource configuration for LTM is provided to the source gNB-DU and candidate gNB-DU(s) via a UE-associated procedure (i.e. UE Context Modification procedure).

Step 4: Based on LTM CSI resource configuration, the source gNB-DU and candidate gNB-DU(s) provide the LTM CSI report configuration as part of the LTM cell configuration.

Therefore, it is to be noted that following agreements of RAN2 are ambiguous as to what signaling support expected at the different steps and conflict with later agreements at RAN2#123bis. Thus, propose that RAN3 sends an LS to RAN2 for clarification on this matter.

- RAN2 assumes that Each candidate DU needs to know the RS configuration of each candidate DUs in order to provide the LTM candidate configuration. - RAN2 assumes that the CU transmits to each C-DU the RS configuration of S-DU (if this is an LTM candidate cell) and/or other C-DUs, to generate the corresponding L1 configuration for LTM. - RAN2 assumes C-DU generates the RS configuration and send to the CU. The CU transmits to the Source DU the RS configuration per LTM candidate cell and the associated LTM candidate (when the CU receives LTM candidate configuration(s) from the C-DU). It is up to RAN3 whether the RS configuration is sent before (or at the same time of) the C-DU creates the LTM candidate configuration (and whether is semi-statis or UE associated). - The RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration.

Proposal 1: RAN3 to send an LS to RAN2 asking to clarify the above agreements.

Furthermore, based on RAN3 agreement to use the UE-associated signaling in order to retrieve the RS configuration (i.e. the SSB configuration) for each candidate cell from the gNB-DU, it is necessary that an additional UE Context Modification procedure is performed to provide the gNB-CU generated CSI resource configuration and to retrieve the CSI report configuration from the gNB-DU(s). Likewise, the current stage-2 BL CR for the intra-gNB-DU LTM procedure also needs to be updated, and in which a single class-1 UE Context Modification procedure is captured in the LTM preparation phase.

Observation 1: Current stage-2 BL CR for TS 38.401, does not capture the latest agreements about RS and CSI resource/report configuration correctly.

Proposal 2: Add an additional UE Context Modification procedure to the intra-gNB-DU LTM mobility scenario to correctly depict the LTM procedure and signaling of RS Configuration, CSI resource configuration, and CSI report configuration.

Similarly for inter-gNB-DU case, if a CSI resource configuration is provided to gNB-DU and the CSI report configuration is retrieved from the gNB-DU as part of step 5 and step 6 respectively. Thus, these steps cannot be used to provide new prepared cells, and which is currently marked as FFS in the stage-2 BL CR, and propose to remove them.

Proposal 3: Remove the FFS regarding new prepared cells from step 5/6 in the inter-gNB-DU LTM mobility scenario in BL CR of TS 38.401.

Furthermore, step 7 and step 8 are not optional anymore since these steps are required to provide the CSI resource configuration to candidate gNB-DU(s) and retrieve the corresponding CSI report configuration.

Proposal 4: Covert steps 7 and 8 from optional steps (dotted line) to mandatory steps (solid line) in the inter-gNB-DU LTM mobility scenario in BL CR of TS 38.401.

# 4	Other remaining Stage 2 issues

A remaining issue in the Stage-2 signaling flow diagrams is in regard to capturing signaling of the RRCReconfigurationComplete message signaled from the UE to the Candidate gNB-DU, and then signaled from the Candidate gNB-DU to the gNB-CU via an F1AP UL RRC MESSAGE TRANSFER message. This aspect is missing in and should be captured as additional steps after the “ACCESS SUCCESS” message.

Proposal 5: Update the signaling diagrams to capture signaling of the RRCReconfigurationComplete message when the UE has accessed the target cell in BL CR of TS 38.401.

Another remaining open item for the inter-gNB-DU LTM mobility scenario, is in regard to whether the gNB-CU utilizes a single or parallel messages in Steps 7 and 8 to provide the LTM CSI Resource Configuration for multiple prepared LTM cells at a given candidate gNB-DU. This is captured in the BL CR as an FFS.

In our view, it is natural to follow the approach and earlier decision of RAN3 to use parallel messages for LTM configuration. Thus, the following FFS can be removed: “FFS  on step 7 and 8 on whether should be parallel or single.”.

Proposal 6: Agree that the gNB-CU utilizes parallel messages to signal the LTM CSI Resource Configuration to the candidate NB-DU, and remove the FFS stating “FFS  on step 7 and 8 on whether should be parallel or single.”.

# 5	Conclusion

Proposal 1: RAN3 to send an LS to RAN2 asking to clarify the above agreements.

Observation 1: Current stage-2 BL CR for TS 38.401, does not capture the latest agreements about RS and CSI resource/report configuration correctly.

Proposal 2: Add an additional UE Context Modification procedure to the intra-gNB-DU LTM mobility scenario to correctly depict the LTM procedure and signaling of RS Configuration, CSI resource configuration, and CSI report configuration.

Proposal 3: Remove the FFS regarding new prepared cells from step 5/6 in the inter-gNB-DU LTM mobility scenario in BL CR of TS 38.401.

Proposal 4: Covert steps 7 and 8 from optional steps (dotted line) to mandatory steps (solid line) in the inter-gNB-DU LTM mobility scenario in BL CR of TS 38.401.

Proposal 5: Update the signaling diagrams to capture signaling of the RRCReconfigurationComplete message when the UE has accessed the target cell in BL CR of TS 38.401.

Proposal 6: Agree that the gNB-CU utilizes parallel messages to signal the LTM CSI Resource Configuration to the candidate NB-DU, and remove the FFS stating “FFS  on step 7 and 8 on whether should be parallel or single.”.

# References

- RP-221799 Further NR Mobility Enhancements, MediaTek

- R3-235968 (BL CR to 38.401) for L1L2Mob, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, ZTE

- RAN3#121bis Chair Notes

# Annex: TP for LTM BL CR TS 38.401

---------------------------------------------------------next change -------------------------------------------------------------

#### 8.2.1.X	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU LTM

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate LTM configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing one target candidate cell ID. The gNB-CU requests PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration.

- If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations (e.g., LTM TCI state configuration, RACH configuration, and the SSB configuration) for the accepted target candidate cell.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing the LTM CSI Resource configuration.

- The gNB-DU sends a UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU containing the LTM CSI Report configuration.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- Early synchronization is performed as specified in TS 38.300 [2].

- The UE sends the lower layer measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

- The gNB-DU sends the LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the TCI stage ID.

- The gNB-DU detects the UE access.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

- The UE sends a RRCReconfigurationComplete message to the gNB-DU.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

---------------------------------------------------------Next change -------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.Y-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.Y-1: Inter gNB-DU LTM

1. The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2. The gNB-CU determines to initiate LTM configuration.

3. The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing one target candidate cell ID. The gNB-CU indicates the source gNB-DU ID, and requests PRACH resources from the candidate gNB-DU. The gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration.

4. If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer configuration (e.g., LTM TCI state configuration and RACH configuration) and the SSB configuration for the accepted target candidate cell.

5. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the LTM CSI resource configuration, LTM TCI state configuration and RACH configuration for the accepted target candidate cell(s) in other gNB-DUs.

6. The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes the generated LTM CSI report configuration.

7. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the cell ID(s) of the prepared candidate cell(s) and LTM CSI resource configuration. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

Note: The candidate cell may be the same cell as source cell.

8. The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated low layer configuration in which contains the generated LTM CSI report configuration.

9. The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10. The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11. The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12. The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13. Early synchronization is performed as specified in TS 38.300 [2].

14. The candidate gNB-DU sends the TA. the associated CFRA resource information, the candidate cell ID and the source gNB-DU ID to the source gNB-DU via the gNB-CU.

FFS on the message name.

15. The UE sends the lower layer measurement result to the source gNB-DU.

16. The source gNB-DU decides to execute LTM to a candidate target cell.

17. The source gNB-DU sends LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

18. The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the TCI state ID.

19. The gNB-CU sends the target cell ID and the TCI state ID to the target gNB-DU.

FFS on the message to be used.

20. The candidate gNB-DU detects the UE access.

21. The candidate gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

22. The UE sends a RRCReconfigurationComplete message to the gNB-DU.

23. The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

24. The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

25. The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

---------------------------------------------------------next change -------------------------------------------------------------

#### 8.2.1.Z	LTM with gNB-CU-UP change

Figure 8.2.1.z-1 shows the procedure used for LTM with the change of gNB-CU-UP within a gNB.

Figure 8.2.1.z LTM with the change of gNB-CU-UP

0. The source gNB-DU forwards the Measurement Report to the gNB-CU-CP.

1. The gNB-CU-CP decides to initiate LTM configuration.

2. The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the target gNB-CU-UP.

3.	The target gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, DL TNL address information for NG-U, and the TNL address information for data forwarding to the target gNB-CU-UP.

4 . LTM configuration procedures are performed between gNB-CU and candidate gNB-DUs, and between gNB-CU and source gNB-DU as specified from step 3 to step 8 in section 8.2.1.Y.

5 - 6.The gNB-CU-CP sends the RRC Reconfiguration message to the UE.

7.  The UE sends the lower layer measurement result to the source gNB-DU, and the source gNB-DU decides to execute LTM to a candidate target cell.

8. The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU-CP with the selected target cell ID.

9-10. The gNB-CU-CP performs the Bearer Context Modification procedure to retrieve the PDCP UL/DL status and to exchange the TNL address information for data forwarding for the bearers.

11-12.	The gNB-CU-CP performs the Bearer Context Modification procedure to send the DL TNL address information for F1-U and the PDCP UP/DL status to the target gNB-CU-UP.

13.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

14.  The target gNB-DU detects the UE in the target cell.

15. The target gNB-DU sends an ACCESS SUCCESS message to the gNB-CU-CP.

16. The target gNB-DU sends an UL RRC MESSAGE TRANSFER containing the RRCReeconfigurationComplete message from the UE.

16-18. Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

19-20.	Bearer Context Release procedure may be performed to release the UE context in the source gNB-DU.

--------------------------------------------------------- the end of changes -------------------------------------------------------------