3GPP TSG-RAN WG3 #122							R3-237175

Chicago, USA, 13th – 17th Nov, 2023

Agenda Item:	12.2.2.2

Source:	CATT

Title:	TP to BLCR for TS 38.423 on AI/ML based mobility optimization

Document for:	Approval

- Introduction

According to the discussion in R3-237174, we provide TP to BLCR for TS 38.423.

- TP to 38.423

- 8.4.AA	Data Collection Reporting Initiation

8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of information to another NG-RAN node to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request IE is set to “stop”; or

-

If the Registration Request IE is set to “start” in the DATA COLLECTION REQUEST message and the Report Characteristics IE indicates cell-specific information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable  of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Per Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the Collection Time Duration IE contained in the Additional UE Trajectory Reporting Conditions IE in the DATA COLLECTION REQUEST message is present, the NG-RAN node2 shall use this value as a triggering condition for reporting the UE Trajectory. If the Number of handovers IE  contained in the Additional UE Trajectory Reporting Conditions IE in the DATA COLLECTION REQUEST message is present, the NG-RAN node2 shall use this value as a triggering condition for reporting the UE Trajectory.

If the UE Performance Configuration  IE in the DATA COLLECTION REQUEST message is present, it indicates the configuration for the collection of UE performance. The NG-RAN node2 shall take it into account when collecting the UE performance.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

9.2.3.P	Additional UE Trajectory Reporting Conditions

This IE contains additional conditions triggering the target node to report UE Trajectory after successful handover.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration | O |  | INTEGER (1..4096) ( | The maximum time duration, in seconds, starting from successful handover execution. |
| Number of handovers | O |  | INTEGER (1..16) | Maximum number of intra-node handovers after which the target node will report the UE Trajectory. |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

9.3.5	Information Element definitions

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- A

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated ::= SEQUENCE (SIZE(1..maxnoofTargetSNsMinusOne)) OF AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item ::= SEQUENCE {

pDUSessionResourceChangeConfirmInfo-SNterminated				PDUSessionResourceChangeConfirmInfo-SNterminated,

iE-Extensions		ProtocolExtensionContainer { { AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item-ExtIEs} } OPTIONAL,

...

}

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalUETrajectoryReportConditions ::= SEQUENCE {

collectTimeDuration								INTEGER (1..4096)							OPTIONAL,

-- The IE is FFS --

numberOfHandovers								INTEGER (1..16)							OPTIONAL,

-- The IE is FFS --

iE-Extensions					ProtocolExtensionContainer { { AdditionalUETrajectoryReportConditions-ExtIEs} }		OPTIONAL,

...

}

AdditionalUETrajectoryReportConditions-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>