3GPP TSG-RAN WG3 Meeting #122	R3-237183

Chicago, USA, Nov. 13th – Nov. 17th, 2023

Agenda Item:	13.2

Source: 	Canon Research Centre France

Title:	Discussion on IAB-node mobility

Document for:	Discussion

# Introduction

In this paper, we discuss various aspects related to the mobility of an IAB-node based on the following agreements from the previous RAN3 meeting reports [1], [2], [3], and [4]:

At RAN3#118 [1]: - The mIAB-MT ID sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU is the XnAP UE ID. FFS which donor generates this ID. At RAN3#119 [2]: - In case the target logical mIAB-DU’s CU is different from the mIAB-MT’s CU, the target logical mIAB-DU’s CU needs to be informed about the mIAB-MT’s CU ID and the mIAB-MT ID so that it can initiate the Xn TMM procedures towards mIAB-MT’s CU. At RAN3#120 [3]: - During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU. At RAN3#121bis [4]: - Option B: The gNB-ID of the MT’s CU and the MT-ID is passed over F1AP is selected as the option to adopt for MT migration. - WA: Use the BAP address as the identifier for the MT in the initial TMM message sent by the DU’s CU to the MT’s CU.

# Discussion

## Acceptance/Rejection of DU migration

When the DU of a mobile IAB-node is migrated from a source F1 donor-CU toward a target F1 donor-CU, the UEs served by the mobile IAB-node shall also be handed over toward this target F1 donor-CU. Upon reception of a handover request for a UE served by the mobile IAB-node, the target F1 donor-CU may reject the handover. If this happens, the cause of handover preparation failure should not be related to the radio network layer as the traffic associated to the UE is handled by the RRC terminating donor-CU serving the co-located mobile IAB-MT. However, the cause may be a control processing overload as a CU can handle a limited number of connected UEs. In case only a few donor-CUs support mobile IAB, those donor-CUs would handle many mobile IAB-nodes and thus many UEs.

A situation where most handovers of UEs served by a mobile IAB-node are rejected by the target F1 donor-CU should be avoided, as these UEs, and especially the onboard UEs, will have to be handed over to another cell and they will lose the benefit of the mobile cell. Thus, it may be useful to get the status at the target F1 donor-CU before initiating the first UE handover.

For this purpose, the F1 setup request in the scope of a DU migration may include an information related to the number of UEs served by the mobile IAB-node. This information would help the target donor-CU for the decision to accept or to reject the F1 setup request (due to control processing overload). In case of rejection, another target donor-CU may be selected, or a new attempt may be performed later.

Proposal 1: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU may include the number of connected UEs served by the mobile IAB-node.

Besides, the target F1 donor-CU may be allowed to partially accept the DU migration. For instance, the target F1 donor-CU may partially accept the DU migration in the F1 setup response by indicating the number of UEs it can accommodate.

Proposal 2: The target F1 donor-CU may accept the DU migration of a mobile IAB-node with some conditions, e.g., with a maximum number of UEs that can be served.

## F1 setup at DU migration

In the F1 setup request message, the mobile IAB-node should pass the following information to the target F1 donor-CU:

- The indication that the F1 setup request is related to a DU migration. Thanks to this information element, the target F1 donor-CU is aware of the context and can correctly handle the request.

- The PCI(s) of active cell(s) at the source logical DU. This information element helps the target F1 donor-CU to select appropriate PCI(s) for the cell(s) to be activated at the target logical DU.

Proposal 3: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU should include the explicit indication that the F1 setup is related to the DU migration of a mobile IAB-node, and it should include the list of PCIs at the source logical DU of this mobile IAB-node.

## Concurrent MT and DU migrations

We have observed that a mobile IAB-MT migration is driven by the radio conditions, and it should not be delayed to maintain backhaul connectivity and to avoid service interruption at the served UEs. It means that a mobile IAB-MT handover may be triggered and executed while a migration of the co-located mobile IAB-DU is on-going.

In the offline discussion [5] at RAN3#121bis meeting, it was proposed that when MT migration is conducted during DU migration, the source-DU’s CU and the target-DU’s CU need to be informed about the gNB-ID of the MT’s target CU and the MT-ID selected by the MT’s target CU.

Actually, the gNB-DU Configuration Update procedure can be used by a mobile IAB-node to inform the F1 terminating donor-CU about the gNB-ID of the RRC terminating donor-CU and the ID of the mobile IAB-MT at this CU, and the F1AP procedure gNB-DU Configuration Update is appropriate as it can be triggered at any time.

Thus, no new specifications effort is required to support concurrent DU and MT migrations.

Proposal 4: To consider concurrent DU and MT migrations as a working assumption.

# Conclusion

In this contribution, we have discussed the possible rejection of DU migration for a mobile IAB-node, the F1 setup procedure at DU migration, and the case of concurrent MT and DU migrations. Then, we have made the following proposals:

Proposal 1: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU may include the number of connected UEs served by the mobile IAB-node.

Proposal 2: The target F1 donor-CU may accept the DU migration of a mobile IAB-node with some conditions, e.g., with a maximum number of UEs that can be served.

Proposal 3: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU should include the explicit indication that the F1 setup is related to the DU migration of a mobile IAB-node, and it should include the list of PCIs at the source logical DU of this mobile IAB-node.

Proposal 4: To consider concurrent DU and MT migrations as a working assumption.

# References

[1] Chairman notes of RAN3#118 meeting

[2] Chairman notes of RAN3#119 meeting

[3] Chairman notes of RAN3#120 meeting

[4] Chairman notes of RAN3#121bis meeting

[5] R3-235724, Offline discussion on mIAB