3GPP TSG-RAN WG3 Meeting #122	R3-237191

Chicago, IL, U.S.A., November 13th – 17th 2023

Agenda Item:	13.5.

Source:	Ericsson

Title:	Remaining Issues Related to Mobile IAB-node-based Positioning

Document for:	Agreement

# Introduction

In this paper we discuss the remaining issues related to mIAB-node positioning. A TP for the mIAB BL CR for TS 38.470 is provided in the Annex.

# Stage-2 impact of mIAB-based positioning

RAN3 has previously agreed that, when available, the mobile IAB-MT’s UE ID (i.e., the GPSI) can be included in the F1AP TRP INFORMATION RESPONSE message, so that the donor can CU transfer this information to LMF. However, it is not described how the donor CU should interpret this information. Therefore, it needs to be clarified in stage 2 that the positioning function allows the gNB-DU to provide the Mobile IAB-MT UE ID (i.e., the GPSI) and location information to the donor CU of a mobile TRP. A TP for mIAB BL CR for TS 38.470 is provided in the Annex.

Proposal: Agree the TP for mIAB BL CR for TS 38.470 provided in the Annex.

# Conclusion

This paper discusses handling of mIAB-node authorization and positioning. The following is proposed:

Proposal: Agree the TP for mIAB BL CR for TS 38.470 provided in the Annex.

# Annex: TP for mIAB BL CR for TS 38.470

-------------------------------------------Start of changes-------------------------------------------

### 5.2.11	Positioning function

This function allows to transfer location management messages between gNB-CU and gNB-DU. With this function, gNB-CU request TRP information from gNB-DU, and gNB-DU response to gNB-CU with the TRP information. With this function, gNB-CU request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements. With this function, the gNB-DU can provide the Mobile IAB-MT UE ID (i.e., GPSI) and location information to the gNB-CU for a mobile TRP.

The function allows the gNB-CU to

-	transfer the positioning assistance data to gNB-DU. The gNB-DU is responsible for broadcasting the positioning assistance data according to the scheduling parameters available.

-	request the gNB-DU to configure SRS transmissions for UE.

-	request the gNB-DU to configure PRS transmissions.

-	request the gNB-DU to configure measurement gap or PRS processing window.

-	request the gNB-DU to activate or deactivate preconfigured measurement gap or PRS processing window.

-	request the gNB-DU to broadcast positioning system information.

-------------------------------------------End of changes-------------------------------------------